SA WG2 Meeting #149E e-meeting	S2-2200165r02

Elbonia, 14 – 25 Feb, 2022	(revision of S2-22xxxxx)

Source:	China Telecom

Title:	New solution for FS_AMP WT#1

Document for:	Approval

Agenda Item:	9.15

Work Item / Release:	FS_AMP / Rel-18

Abstract: This contribution proposes a solution to keep RFSP Index consistency when UE moves from 5GC to EPC using N26 interface.

# 1.	Introduction

In this document, a solution to FS_AMP WT#1, regarding how to provide RFSP Index consistency when UE moves from 5GC to EPC, is proposed for N26-based interworking.

The PCF can change the RFSP Index according to variable network situations when UE registers to 5GC. A UE may have conflict RFSP Index in use under 5G and 4G at the same time even the subscribed RFSP Index and local pre-configuration are the same in AMF and MME. Because the MME cannot receive updated AM policy from current PCC mechanism, while the AMF can receive that it from PCF.

It is assume that, because the PCF is capable of gathering more information, e.g., AF request, NWDAF output, etc., the PCF can make better decision on RFSP Index selection than MME which chooses RFSP index only based on static subscription and pre-configured information. The AMF is already support receiving AM control from PCF. If the MME is also able to receive RSPF Index from PCF under interworking architecture, then it can avoid the inconsistency in RFSP Index when UE moves between 5GC and EPC.

This contribution focus on the solution to allow MME to receive RFSP Index update from PCF when UE moves from 5GC to EPC using N26 interface.

# 2.	Proposal

It is proposed to adopt the following solution into 23.700-89.

# * * * * ALL NEW TEXT * * * *

## 6.X	Solution #X: Use N26 to provide PCF-updated RFSP Index from 5GC to MME

### 6.X.1	Description

This solution is to allow MME to receive RFSP Index update from PCF when UE moves from 5G to 4G in N26-based interworking architecture. When MME is able to receive updated RFSP index from PCF as AMF, the RFSP index in use will be consistent when UE moves between 5G and 4G.  This is a solution for N26-based interworking.

When N26-based interworking architecture applies, AMF and MME use N26 interface to transfer mobility management message. The parameters, subscribed RFSP index and RFSP index in use, are transferred in:

- Forward Relocation Request as in 5GS to EPS handover procedures using N26 interface, or

- Context Response as in 5GS to EPS Idle mode mobility using N26 interface.

Therefore, this solution proposes that, when UE moves from 5G to 4G, the old AMF should keep or defer to terminate the AM association of the UE within a reasonable period, i.e., AM Policy Association maintenance timer. When the PCF is triggered to update the RFSP Index, the updated RFSP index is delivered to the old AMF in the existing AM Policy association, and the AMF will transfer it to new MME use the N26 interface in a GTPv2 message.

For example, a UE may have subscribed RFSP Index of 5G prioritization in subscription data stored in HSS/UDM.

If a UE initially attaches to 5GC with AM Policy association established,

- If something triggers the PCF to change the RFSP Index as of 4G prioritization, the UE will moves from 5GC to EPC. In handover or idle mode mobility procedure, the old AMF sends the PCF-updated RFSP Index, i.e. the RFSP Index in use, to the new MME using N26 interface.

- With the received RFSP Index in use, the new MME may, as local logic, ignore the subscribed RFSP Index and keep the RFSP index in use as 4G prioritization. The UE will stay in 4G and will not be send right back to 5G as the subscribed RFSP Index as 5G prioritization. The old AMF may keep the established AM association or defer the AM policy association termination for the UE to receive further update on RFSP Index update from PCF, just as the old AMF will keep UE context for some time after the UE deregister.

- Later but before the AM Policy Association maintenance timer runs out, based on some trigger, the PCF may update UE’s RFSP index in order to direct the UE back from EPC to 5GC. The update RFSP index is deliver to the old AMF. Then the old AMF transfers it to the new MME the UE register to through N26 interface. Then the new MME updates the RFSP index in use and triggers the UE move back to 5G using current mechanism.

- When the AM Policy Association maintenance timer runs out, the old AMF initiates the AM Policy Association termination. This should be no later than the old AMF to release the UE context.

If a UE initially attaches to EPC, or the AM policy association is invalid, that is, no exist AM policy association in PCF for that UE, the MME may update UE’s RFSP Index according to current mechanism. The MME may decide to direct UE to 5GC by changing the RFSP Index. In some cases, when the EPS to 5GS procedure is complete and soon after a new AM association is established, the PCF may decides the UE should be in 4G. Then the EPS to 5GS procedure as described above may take place. With the proposed mechanism, the PCF selected RFSP Index will take over and the UE will not ping-pong between 4G and 5G.

Editor’s Note: After 5GS to EPS mobility, if the UE moves further to another MME, how the AMF sends updated RFSP index to the serving MME within the AM Policy Association maintenance timer is FFS

### 6.X.2	Procedures

Figure 6.x.2 depicts the procedure of propose solution of providing PCF-updated RFSP Index update from 5GC to MME with N26 interface.

Figure 6.X.2 PCF-updated RFSP Index update from 5GC to MME in N26-base architecture

Step 1 and Step 2 follow the procedure already defined in TS23.502. The change is: the AMF, where the UE originally register to, will not initiate the AM Policy Association termination procedure with the PCF when it received the Nudm_UECM_DeregistrationNotification from HSS/UDM as in clause 4.11.1.5.3 of TS23.502 (TAU of handover) nor clause 4.11.1.3.2 of TS23.502 (Idle mode mobility). But the AM Policy Association maintenance timer starts to run in AMF when the Nudm_UECM_DeregistrationNotification recieved.

Step 3 and Step 4, the PCF is trigger to update the RFSP Index and notify to the old AMF according to current description in TS23.503.

Step 5 and Step 6 are the proposed new mobility management messages using N26 interface, in which the AMF transfer the updated RFSP index to MME and MME reply with an acknowledgement.

When the AM Policy Association maintenance timer runs out, the AMF initiates the AM Policy Association termination as in TS23.502.

### 6.X.3	Impacts on services, entities and interfaces

The solution has the following impacts:

AMF:

•	It is responsible for the old AMF to transfer the updated RFSP Index from PCF to the MME where UE registers to in EPC when it is move from 5GS to EPS.

•	When a UE is moving from 5GS to EPS, and HSS/UDM invokes Nudm_UECM_DeregistrationNotification to notify the AMF associated with 3GPP access with reason as 5GS to EPS Mobility, the AMF should not initiate the AM Policy Association termination.

•	The AMP should maintain a timer for AM Policy Association maintenance in case of 5GS to EPS Mobility.

MME:

•	MME should be able to receive and apply the updated RFSP Index from AMF after the 5GS to EPS handover procedure or idle mode mobility complete.

N26:

•	New mobility management message are defined to support the RFSP Index transfer from AMF to MME.

# * * * * End of change * * * *