TSG SA WG2 Meeting #149E	S2-2200234

14 - 25 February, 2022, Electronic meeting

Source:	Lenovo, Motorola Mobility, Huawei? HiSilicon? Vivo? Ericsson? CATT?

Title:	Key issue on traffic characteristics of XR and media service

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: Proposes a Key Issue on traffic characteristics for XR and media service

1	Background

According to the SP-211610[1], the study on 5G System support for XR and media services targeted at R18 supports 5GS architectural and functional extensions so that XR and media service can be better supported. One of the objectives that this study attempts to address is:

WT#3.1:  Study the traffic characteristics of media service enabling improved network resources usage and QoE.

In this paper, traffic characteristics for XR and media service and the open issues are discussed.

2	Discussion

The following contents in this paper focus on real-time XR/media services. In general, XR and media service has the following traffic characteristics.

2.1 Frame and GOP

A video consists of a sequence of consecutive video frames. Each video frame is a picture encoded/compressed using different codec mechanisms (e.g. H.264/H.265/H.266, AV1 or AVS) for efficient storage and transmission. During encoding, the video spatial and temporal redundancy can be removed using, e.g. inter-frame/intra-frame prediction, motion estimation/compensation. Typically, three major frame/picture types can be used during encoding, i.e. I-frame, P-frame and B-frame:

- I-frame, as an intra-coded picture, is a complete picture and can be encoded and decoded independently, like a JPG image file.

- P-frame, as a predicted picture, is not a complete frame and only contains the image changes compared to the previous frame. If the reference frame is lost, the P-frame cannot be decoded and displayed.

- B-frame, as a bidirectional predicted picture, contains the changes between the previous and following reference frames. With more reference frames, the compression ratio can be higher. However, the B-frame can only be decoded when the previous and following reference frames are available.

A Group of Pictures (GOP) includes a collection of successive video frames. The first frame in a GOP is an I-frame. And the following frames can be P-frames or B-frame.

For example, as shown in figure 1, a GOP contains five frames and the first frame is I frame. The second frame is P frame and refers to the I frame. Similarly, the third frame is P frame, referring to the second frame. The forth frame is a B frame which refers to both the previous and following frames. When delivering the GOP from the application server to a client, the I frame can be directly decoded and displayed. The P frame can be decoded only when the previous frame arrived and is successfully decoded. The B frame can be decoded only when both the previous and following frames arrive and are successfully decoded.

Figure 1 Frame types and GOP

2.2 Other Traffic Characteristics

Most of the objectives of this study item are closely related with the XR traffic characterisitcs as follows.

1) Periodic and jitter

Data burst of video frame are always periodic with variable packet size. For example, audio frame ususally have a periodicity of 20ms, while video stream can have different frame rate, e.g., 60/90/120 fps. Besides, pose/control packets are always periodic (e.g., 4ms) with fixed packet size. Due to the unpredictable jitter, the packet arrival time might not be exacly periodic, which leads to transmission delay or packet loss. 5GS should take both periodicity and jitter into consideration for XR and media service.

2) Frame importance

For XR and media service, different types of frame coexist and have different importance. For example, in Group of Picture (GOP), the first I-frame is the most important frame, other P-frames are encoded based on the I-frame. That is, the decoding of P-frames depends on the decoding of I-frame in the same GOP. Therefore, frame importance should be provided to enable 5GS to perform efficient packet dropping, e.g., the non-transmitted part with low importance of GOP will be discarded at the transmitter if the packets with high importance of the GOP have been lost or exceed the delay budget.

3) Media unit granularity

XR applications impose requirements in terms of Media Units(e.g., video/audio frame/tile, Application Data Units etc.), rather than in terms of single packets/PDUs. Packets of one media unit need to be jointly processed for XR traffics. 5G system should be aware of the media unit in order to improve transmission performance. Basically, the media unit SN, the packet SN within the media unit, the total packet number of the media unit or the last packet indication should be provided to 5G system for each packet.

4) Traffic correlation

Some advanced XR or media services may include multiple traffic streams such as video, audio, haptic data or sensor data etc., which are called multi-modality communication. Different traffic streams with different QoS parameters are always mapped to different QoS flows. Packets within the same traffic stream with different packet charateristics (e.g., packet importance, frame type etc.) may also be classified and mapped into different QoS flows. These QoS flows belong to the same XR and media service have strong correlations, which should be coordinated for QoS requirement, packet processing and time synchronization etc.

In short, 5GS should be aware of all the paremeters related with traffic characterisitcis for XR and media service, which are defined as application-aware parameters. Generally, downlink traffic dominates XR and media service. In this paper, we mainly discuss how application-aware parameters are provided to 5GS, either over control plane by configuration, or over user plane together with each packets? E.g., parameters used to identify media unit should be provided for each packet over user plane, while the periodicity and traffic correlation can be provided over control plane by configuration.

2.3 Input from SA4

In SA4, the traffic models and quality evaluation methods for Media and XR services in 5G systems are discussed and captured in TR 26.926 [2]. The general architecture and system model for XR Traffic is shown in Figure 2. The packet traces (P-Traces) are defined as IP packets with associated metadata arrived at 5G system.

Figure 2 General architecture and system model for XR Traffic

The P-trace format together with semantics is defined in Table 1, where baseline paramters and cross-layer parameters are defined. The P-Trace may be extended based on new findings.

Table 1 P-Trace format

| Name | Type | Semantics |
| --- | --- | --- |
| Number | BIGINT | Unique packet number in the delivery |
| Baseline parameters |  |  |
| availability_time | BIGINT | Availability time of packet for next processing step relative to start time 0 in microseconds (0 means lost). |
| Size | BIGINT | packet size in bytes. |
| flow_id | BIGINT | an id to the flow in order to differentiate different QoS flows |
| Cross-Layer Parameter |  |  |
| buffer | BIGINT | The associated eye buffer 1=left 2=right In general, differentiates application traffic for different buffers, for example audio, video, left eye, right eye. |
| Delay | BIGINT | Delay observed of the packet in the last processing step (-1 means lost) |
| render_timing | BIGINT | the rendering generation timing associated to the media included in the packet. |
| number_in_unit | BIGINT | The number of the packet within the unit (slice), start at 1 |
| last_in_unit | BIGINT | Indicates if this is the last packet in the slice/unit 0=no, 1=yes |
| type | BIGINT | The data type of the unit 0 unknown For video 1=intra 2=inter |
| importance | BIGINT | assigned relative importance information (higher number means higher importance) |

From Table 1, we can see that “number_in_unit” and “last_in_unit” can be used to identify media unit. However, there’s no media unit SN in the P-trace profile. It needs further study whether the existing parameter contained in P-Trace profile can be used to deduce the media unit SN. E.g., packets with the same “render_timing” belongs to the same media unit, or packets with the same “Number” belongs to the same media unit. The “importance” IE can be used to identify the packet importance. It needs further study whether frame type (e.g., I-frame, P-frame etc.) and traffic type (e.g., video, audio, haptic etc. ) need to be provided to 5GS.

It is also stated in TR 26.926 that, the baseline and cross-layer parameters would not be available to RAN delivery in the incoming IP packets. As mentioned before, if the baseline and cross-layer parameters are provided over user plane, it needs further study on the signaling of the application-aware parameters within 5G system (via N3 or N9 interface) without impact on IP protocol layer.

3 Reference

[1] SP-211610, New SID on Study on architecture enhancement for XR and media services, China Mobile, Huawei, HiSilicon, Tencent, Xiaomi

[2] TR 26.926, 3rd Generation Partnership Project; Technical Specification Group SA; Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems; (Release 17)

4 Proposal

* * * * Start of Change * * * *

## 5.X	Key Issue X: <Traffic characteristics for XR and media service>

### 5.X.1	Description

All XR and media traffics, in spite of which codec was used, have some common characteristics. E.g., XR and media service are genearlly periodic with unpredictive jitter. Different XR frames have dependency with each other, which have different frame importance. Some advanced XR and media services may include multiple traffic streams such as video, audio, haptic data or sensor data etc., which are refer to as multi-modality communication. Different traffic streams with different QoS parameters are always mapped to different QoS flows. Packets of the same traffic stream with different packet charateristics (e.g., frame importance, frame type etc.) may also be classified and mapped into different QoS flows. These QoS flows belong to the same XR and media service have strong correlations, which should be coordinated for QoS requirement, packet processing and time synchronization etc.

The following aspects shall be studied to support 5GS to be aware of the traffic characterisitcs for XR and media service.

-  Study and identify which traffic characteristics of XR and media service are useful for aiding traffic handling and resource scheduling in 5GS in order to improve network resources usage and QoE of XR/media. ? Are the paremeters defined in P-trace format of SA4 enough? If not, what else parameters should be considered?

-  How to provide the traffic characteristics of XR and media service to 5GS? Study and identify which parameters should be provided over control plane, and which parameters should be provided over user plane?

-  For traffic characteristics of XR and media service that are provided over control plane, what is the impact on 5GC NFs? E.g.,

-  Whether and how the PCF receives the traffic characteristic and related QoS requirements?

-  Whether and how the traffic characteristic and related QoS requirements are considered by the PCF in the PCC rules?

-  whether and how the traffic characteristic and related QoS requirements in the PCC rules are considered by the SMF at derivation of the QoS parameters for a QoS flow

-  Whether, how and which media unit level QoS requirements and traffic characteristic is conveyed to RAN.

-  For  traffic characteristics of XR and media service that are provided for each packet over user plane, how to provide them within 5G system (via N3 or N9 interface) without impact on IP protocol layer?

-	How to efficiently aid the QoS handling (e.g. QoS policy, resource allocation) with regard to the traffic characteristics of XR/media.

NOTE1: Coordination with RAN and SA4 WGs may be needed for the above bullets.

NOTE2: The traffic characteristics of XR/media in this KI is not specific to media units.

*** NEXT CHANGE ***

Annex X(informative): Traffic characteristics of XR and media services

X.1 Frame and GOP

A video consists of a sequence of consecutive video frames. Each video frame is a picture encoded/compressed using different codec mechanisms (e.g. H.264/H.265/H.266, AV1 or AVS) for efficient storage and transmission. During encoding, the video spatial and temporal redundancy can be removed using, e.g. inter-frame/intra-frame prediction, motion estimation/compensation. Typically, three major frame/picture types can be used during encoding, i.e. I-frame, P-frame and B-frame:

- I-frame, as an intra-coded picture, is a complete picture and can be encoded and decoded independently, like a JPG image file.

- P-frame, as a predicted picture, is not a complete frame and only contains the image changes compared to the previous frame. If the reference frame is lost, the P-frame cannot be decoded and displayed.

- B-frame, as a bidirectional predicted picture, contains the changes between the previous and following reference frames. With more reference frames, the compression ratio can be higher. However, the B-frame can only be decoded when the previous and following reference frames are available.

A Group of Pictures (GOP) includes a collection of successive video frames. The first frame in a GOP is an I-frame. And the following frames can be P-frames or B-frame.

For example, as shown in figure 1, a GOP contains five frames and the first frame is I frame. The second frame is P frame and refers to the I frame. Similarly, the third frame is P frame, referring to the second frame. The forth frame is a B frame which refers to both the previous and following frames. When delivering the GOP from the application server to a client, the I frame can be directly decoded and displayed. The P frame can be decoded only when the previous frame arrived and is successfully decoded. The B frame can be decoded only when both the previous and following frames arrive and are successfully decoded.

Figure 1 Frame types and GOP

X.2 Other traffic characteristics

1) Periodic and jitter

Data burst of video frame are always periodic with variable packet size. For example, audio frame ususally have a periodicity of 20ms, while video stream can have different frame rate, e.g., 60/90/120 fps. Besides, pose/control packets are always periodic (e.g., 4ms) with fixed packet size. Due to the unpredictable jitter, the packet arrival time might not be exacly periodic, which leads to transmission delay or packet loss. 5GS should take both periodicity and jitter into consideration for XR and media service.

2) Frame importance

For XR and media service, different types of frame coexist and have different importance. For example, in Group of Picture (GOP), the first I-frame is the most important frame, other P-frames are encoded based on the I-frame. That is, the decoding of P-frames depends on the decoding of I-frame in the same GOP. Therefore, frame importance should be provided to enable 5GS to perform efficient packet dropping, e.g., the non-transmitted part with low importance of GOP will be discarded at the transmitter if the packets with high importance of the GOP have been lost or exceed the delay budget.

3) Media unit granularity

XR applications impose requirements in terms of Media Units(e.g., video/audio frame/tile, Application Data Units etc.), rather than in terms of single packets/PDUs. Packets of one media unit need to be jointly processed for XR traffics. 5G system should be aware of the media unit in order to improve transmission performance. Basically, the media unit SN, the packet SN within the media unit, the total packet number of the media unit or the last packet indication should be provided to 5G system for each packet.

4) Traffic correlation

Some advanced XR or media services may include multiple traffic streams such as video, audio, haptic data or sensor data etc., which are called multi-modality communication. Different traffic streams with different QoS parameters are always mapped to different QoS flows. Packets within the same traffic stream with different packet charateristics (e.g., packet importance, frame type etc.) may also be classified and mapped into different QoS flows. These QoS flows belong to the same XR and media service have strong correlations, which should be coordinated for QoS requirement, packet processing and time synchronization etc.

* * * * End of Change * * * *