3GPP TSG-WG SA2 Meeting #149E e-meeting  	S2-2200255

Elbonia, February 14 – 25, 2022

# Title:	[DRAFT] Reply LS on Clarification on the change of Target NSSAI

# Response to:	LS (S2-2200051/C3-216445) on Clarification on the change of Target NSSAI from CT3

# Release:	Rel-17

# Work Item:	eNS_Ph2

Source:	SA2

To:	CT3

Cc:

Contact Person:

Name:	Haiyang,Sun

Tel. Number:

E-mail Address:	sunhaiyang3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS 23.501 CR 3477

1. Overall Description:

SA2 would like to thank CT3 for their LS on Clarification on the change of Target NSSAI.

Specific to the CT3 questions in C3-216445/S2-2200051, SA2 would like to provide the following feedback:

Question: Can SA2 confirm that the target NSSAI may be generated more than once during the lifetime of the UE registration (i.e. the change of target NSSAI may happen)? For example, if the subscribed NSSAI or requested NSSAI changes, will the Target NSSAI be changed accordingly? Can SA2 indicate if there are other situations that require the trigger to be invoked.

SA2 Answer: SA2 has agreed that the Target NSSAI at least includes all or a subset of Rejected NSSAI for RA and conditionally includes all or a subset of Allowed NSSAI, as defined in clause 5.3.4.3.3 of TS 23.501. The Target NSSAI can be sent more than once based on AMF logic. The AMF does not forward the Target NSSAi to other AMFs, but can keep it stored locally in the AMF. The Target NSSAI is only possibly affected by the Requested NSSAI, which may contain S-NSSAI(s) that are not available in the current TA of the UE. If the subscribed NSSAI is changed, re-registration providing new Requested NSSAI from UE may be triggered.

The agreed CR to clarify the Target NSSAI is attached.

2. Actions:

To CT3 group.

ACTION: 	SA2 asks CT3 group to take the above feedback into account.

3. Date of Next TSG SA WG2 Meetings:

See https://portal.3gpp.org/Home.aspx?tbid=385&SubTB=385#/