3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200267

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	New Key Issue: Parameter mapping between DetNet controller and 5G system

Document for:	Approval

Agenda Item:	9.6

Work Item / Release:	FS_DetNet/ Rel-18

Abstract: This pCR proposes a new Key Issue to study which parameters should be mapped to 5G parameters and how to process those parameters

# 1. Introduction

According to the SP-211633, the study on Extensions to the TSC Framework to support DetNet targeted at R18 allows 5GC to investigate improvements of TSC framework for support of DetNet, which is standardized in IETF. And one of the objectives that this study attempts to address is:

Study whether and how to enable 3GPP support for DetNet such that a mapping is provided between the central DetNet controller entity (as defined in IETF) and the 5G system. Mapping involves translation of DetNet traffic profile and flow specification to 5GS QoS parameters and TSCAI.

The DetNet controller communicates with the 5G system through TSCTSF. QoS requirements and traffic specifications for DetNet flows may be provided by the DetNet controller. The TSCTSF or other network functions should map the parameters provided by the DetNet controller into 5G QoS parameters or other parameters e.g. TSCAI to configure flow transferring path for the DetNet traffic.

This key issue should study which parameters provided by the DetNet controller should be mapped into 5G parameters. It should also be studied which 5G parameter and which network function should be used for the parameter mapping.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-46.

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[X1]	IETF RFC 8939: "Deterministic Networking (DetNet) Data Plane: IP".

[X2]	IETF RFC 8655: "Deterministic Networking Architecture".

[X3]	IETF RFC 9016: " Flow and Service Information Model for Deterministic Networking (DetNet)".

# * * * * Second change * * * * all new

# * * * * Third change * * * * all new

## 5.X Key Issue #X: Parameter mapping between DetNet controller and 5G system

### 5.X.1 Description

This key issue will study which parameters should be mapped to 5G parameters and how to process those parameters.

The DetNet controller communicates with the 5G system through TSCTSF. DetNet Flow-Related Parameters may be provided by the DetNet controller as defined in RFC 9016 [X3]. The TSCTSF should map the parameters provided by the DetNet controller into 5G QoS parameters or other parameters e.g. TSCAI to configure flow transferring path for the DetNet traffic.

This KI will address:

- What information the 5GS acting as an DetNet IP Data Plane Node needs to expose to the DetNet Controller so that the Controller can perform path and worst-case end-to-end delay calculations for DetNet IP traffic flows served by the 5GS,

-	What information the 5GS acting as an IP Data Plane Node needs to receive from the Controller after the Controller decides to setup an DetNet IP data flow to be served by the 5GS with specific QoS requirements,

This includes work on Which parameters provided by the DetNet controller should be mapped into which 5G parameters.

-	What parts of the required information can already be supported by the existing R17 5G exposure APIs and what additional information needs to be defined for them

# * * * * End of changes * * * *