3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200267

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	New Key Issue: Parameter mapping between DetNet controller and 5G system

Document for:	Approval

Agenda Item:	9.6

Work Item / Release:	FS_DetNet/ Rel-18

Abstract: This pCR proposes a new Key Issue to study which parameters should be mapped to 5G parameters and how to process those parameters

# 1. Introduction

According to the SP-211633, the study on Extensions to the TSC Framework to support DetNet targeted at R18 allows 5GC to investigate improvements of TSC framework for support of DetNet, which is standardized in IETF. And one of the objectives that this study attempts to address is:

Study whether and how to enable 3GPP support for DetNet such that a mapping is provided between the central DetNet controller entity (as defined in IETF) and the 5G system. Mapping involves translation of DetNet traffic profile and flow specification to 5GS QoS parameters and TSCAI.

The DetNet controller communicates with the 5G system through TSCTSF. QoS requirements and traffic specifications for DetNet flows may be provided by the DetNet controller. The TSCTSF or other network functions should map the parameters provided by the DetNet controller into 5G QoS parameters or other parameters e.g. TSCAI to configure flow transferring path for the DetNet traffic.

This key issue should study which parameters provided by the DetNet controller should be mapped into 5G parameters. It should also be studied which 5G parameter and which network function should be used for the parameter mapping.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-46.

# * * * * First change * * * *

-

# * * * * Second change * * * * all new

# * * * * Third change * * * * all new

## 5.X Key Issue #X: Provisioning DetNet configuration from the DetNet controller to 5GS

### 5.X.1 Description

A DetNet controller may provide DetNet configuration as defined in ietf-detnet YANG module to the 5GS acting as a DetNet node. This key issue will study how to map the DetNet configuration.

The DetNet controller communicates with the 5G system through TSCTSF.. DetNet Flow-Related Parameters may be provided by the DetNet controller as defined in draft-ietf-detnet-yang [V]. The TSCTSF should map the parameters provided by the DetNet controller into 5G QoS parameters or other parameters e.g. TSCAI to configure 5GS flows for the DetNet traffic.

This KI will address:

- - Whether the communication between the DetNet CPF and the 5GS acting as Detnet Node may be direct from the DetNet CPF to a TSCTSF and/or may go via the NEF

- Which parameters provided by the DetNet controller should be mapped into which 5G parameters.

- How the 5GS finds the PDU Sessions corresponding to the given DetNet configuration

- What mechanisms are used in 5GS to configure the system according to the configuration provided by the DetNet controller.

- How the 5GS should treat a DetNet configuration for a DetNet flow when a corresponding PDU Session cannot be found for the source/destination IP address

- Whether and how the 5GS authorizes a request from DetNet controller; what IETF specifications the 5GS should support for authentication and authorization of the DetNet controller.

- Whether a subscription is required for a UE to be controlled by DetNet controller, and in this case, how the subscription is enforced.

# * * * * End of changes * * * *