3GPP TSG-SA WG2 Meeting #149E (e-meeting)	S2-2200316r01

# Feburary 14-25, 2022, Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0497 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Dispersion Analytic for identification of Top-Heavy UEs in a location |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2022-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TS 23.288 states in section 6.10.1 “Dispersion is the percentage of activity that a UE, or group of UEs, or any UE, generates at a location or a slice during a period of interest. This enables the operator to rank hot locations or slices by the various activities and to identify the top contributors (i.e. UEs) for that activity. When heavy UEs at a particular location or slice are detected/predicted and reported to a policy entity, their QoS attributes, or access to an area of interest or a slice can then be limited”. Also, in this section it is stated that the client may include as Analytics Filter Information a filter for Top-Heavy UEs. In addition, TS 23.503 states in section 6.1.1.3 that the PCF may request the NWDAF to report the Dispersion analytics in a congested area for Top Heavy UEs, and then perform AM policy modifications for those UEs reported as heavy users. However, the output for this analytic doesn’t include the list of UEs which are identified as Top-Heavy UEs, so the client is not able to identify which are the UEs considered as Top-Heavy UEs. The same consideration may be also applied to Fixed, Camper and Traveller. In addition, it is not clear in TS 23.288 the usage of “Any UE” in combination with filter information for Data Volume Dispersion analytics, there is contradictory information. In some place it is said "Any UE" is only supported in combination with Analytics Filter Information S-NSSAI and Dispersion Analytics type "Data Volume Dispersion Analytics" but: - Dispersion for a location should also allow “Any UE”. It was already described in 6.10.1 the analytic may be used to determine that a hot spot is formed in a slice or area of interest. - Usage of “Any UE” should not be only restricted to filters containing S-NSSAI and Dispersion Analytics type "Data Volume Dispersion Analytics", it should be also allowed for filters including an area of interest, Top-Heavy UEs, Fixed, Camper or Traveller. In other way it is not possible to identify the Top-Heavy UEs (or fixed, camper or traveller) in a location. In addition, it is already said in 6.10.1 (NOTE 2) when using “Any UE”, in order to alleviate the load, filters as area of interest, Top-Heavy, Fixed, can be used |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend the dispersion analytics output information with a list of UEs. When the analytics client includes in the request a filter for having the Top-Heavy UEs, Fixed, Camper or Traveller UEs the NWDAF will include in the list of SUPIs the UEs which have been identified with that category. Also clarify the usage of “Any UE” in combination with filter information and editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The analytics client is not able to identify the UEs which have been classified as Top-Heavy UEs, Fixed, Camper or Traveller UEs so can’t apply different actions for them. Misalignment between TS 23.288 and 23.503. Contradictory information about the usage of “Any UE”. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.1, 6.10.3.1, 6.10.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

6.10.1	General

Dispersion analytics identifies the location (i.e. areas of interest, TAs, cells) or network slice(s) where a UE, or a group of UEs, or any UE disperse most (if not all) of their data volume and sessions transactions (i.e. MM and SM messages).

The NWDAF shall be able to provide dispersion statistics or predictions and shall be able to collect UE dispersion related information from NFs.

Dispersion is the percentage of activity that a UE, or group of UEs, or any UE, generates at a location or a slice during a period of interest. This enables the operator to rank hot locations or slices by the various activities and to identify the top contributors (i.e. UEs) for that activity. When heavy UEs at a particular location or slice are detected/predicted and reported to a policy entity, their QoS attributes, or access to an area of interest or a slice can then be limited.

-	Data volume dispersion - The percentage of data traffic volume that a UE, or a group of UEs, or any UE, generated at a location or in a slice during the period of interest.

-	Transactions dispersion - The percentage of MM and SM messages that a UE, or a group of UEs, or any UE, generated at a location or in a slice during the period of interest.

The operator may classify and assign one of the three mobility classes per dispersion characteristics for a UE or a group of UEs, or any UE as fixed, camper or traveller. In addition, the operator may classify a UE as a Top-Heavy UE in case its dispersion percentile rating, at a location or a slice, is higher than a threshold value defined by the operator (e.g., 0.9 percentile), see percentile ranking in the output tables, clause 6.10.3. The classification is based on either Data volume dispersion or Transactions dispersion.

-	Data-Classification - fixed, camper, traveller data-classification per thresholds assigned by the operator.

EXAMPLE:	When a UE disperses, during the period of observation, most (threshold=95%) of its data at a location or a slice, the dispersion data-classification of the UE at that location or slice is "fixed".

-	Transaction-Classification - fixed, camper, traveller transactions-classifications per thresholds assigned by the operator.

EXAMPLE:	When a UE disperses, during the period of observation, (threshold=40%) of its session transactions at a location or a slice, the dispersion transaction-classification of the UE is camper at that location or slice.

Based on the dispersion analytics, the analytics consumer can determine that a data volume or transactions hot spot is formed when the volume of data or amount of transactions dispersed by most of the UEs at the area of interest or slice exceeds a certain data or transaction threshold established through trend analysis or operator policy.

The consumer of dispersion analytics may indicate in its request:

-	Analytics ID = "Dispersion Analytics";

-	Target of Analytics Reporting: a single UE, any UE, or a group of UEs. "Any UE" is only supported in combination with Analytics Filter Information (S-NSSAI, Area(s) of Interest, Top-Heavy UEs indication, Fixed, Camper indication or Traveller indication) and Dispersion Analytics type "Data Volume Dispersion Analytics";

-	Analytics Filter Information:

-	optional list of TA(s), Area(s) of Interest, Cells, or S-NSSAI(s), Application ID(s), Top-Heavy UEs, Fixed UEs, Camper UEs, Dispersion Analytics type (Data Volume Dispersion Analytics (DVDA) or Transactions Dispersion Analytics (TDA) or both); and

-	optional list of analytics subsets that are requested (see clause 6.10.3);

-	Preferred level of accuracy of the analytics;

-	Preferred level of accuracy per analytics subset (see clause 6.10.3);

-	Preferred order of results for the list of UE Dispersion Analytics information:

-	ordering criterion:

-	For DDA, one of the following: "time slot start", "data dispersion", "data classification", "data ranking", or "data percentile ranking";

-	For TDA, one of the following: "time slot start", "transaction dispersion", "transaction classification", "transaction ranking", or "transaction percentile ranking"; and

-	order: ascending or descending;

-	Analytics target period indicating the time period over which the statistics or predictions are requested;

-	Optionally, maximum number of objects and maximum number of SUPIs;

-	Optionally, preferred granularity of location information: TA level or cell level; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

NOTE 1:	If one or multiple ID(s) are included in the filter information, the output analytics will be reported per Application ID, otherwise the output analytics is aggregated information per AOI or slice independent of the applications.

NOTE 2:	Care needs to be taken with regards to load when requesting analytics for "Any UE". This could be achieved by utilizing event filters (e.g. Area of Interest for AMF) and possible analytics filters including Top-Heavy UEs, and/or Fixed and/or Camper UEs and/or Analytics Reporting Information (e.g. SUPImax), or sampling ratio as part of Event Reporting Information. The load of analytics for "Any UE" can be alleviated by limiting the number of S-NSSAI in the analytics filter.

* * * * Next Change * * * *

6.10.3.1	Data Volume Dispersion Analytics

The data volume dispersion analytics results provided by the NWDAF can be for a UE or group of UEs or “any UE” data volume dispersion statistics as defined in table 6.10.3.1-1 and/or UE data volume dispersion predictions as defined in table 6.10.3.1-2. If the analytics covers multiple UEs, the Data dispersed, Data ranking, and Data percentile ranking parameters are calculated as the aggregated (i.e., summed up) data volume. When the Target of Analytics Reporting is set as "any UE" the data volume dispersion analytics results provided by the NWDAF can be for all UEs data volume dispersion statistics at a location and/or data volume dispersion predictions.

Table 6.10.3.1-1: Data volume dispersion statistics bound by location

| Information | Description |
| --- | --- |
| UE group ID or UE ID or list of UE IDs (1..SUPImax) | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, or a list of UEs for which for which the statistic applies (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > UE location (1..max) | Observed location statistics. |
| >> UE location | TA or cells where the UE or UE group or list of UEs dispersed its data (see NOTE 3). |
| >> Application ID (0..max)(NOTE 7) | To identify the application (NOTE 6). |
| >>> Data volume dispersed (NOTE 4) | Data volume dispersed at this location. |
| >> Data mobility classification (NOTE 4) | Data mobility classification of the UE for this location: fixed, camper, traveller (see NOTE 5). |
| >> Data usage ranking (NOTE 4)(NOTE 7) | Ranking of UE data usage at this location (See NOTE 2). |
| >> Data usage percentile ranking (NOTE 4) (NOTE 7) | Percentile ranking of the UE or UE group in the Cumulative Distribution Function of data usage for the population of all UEs served at the location. |
| >> Ratio | Percentage of UEs in the group at the location and observation time (in the case of UE group). |
| NOTE 1: When Target of Analytics Reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. When Target of Analytics Reporting is a UE group Id, an internal group Id will be included. When Target of Analytics Reporting is a UE group Id or “Any UE” and a filter for Top-Heavy UEs, fixed, camper or traveller is included, the NWDAF will include the list of UEs matching the filter. This information element is not present when Target of Analytic Reporting is “Any UE” and no filter for Top-Heavy UEs, fixed, camper or traveller is included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE, group of UEs or list of UEs. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its data dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. NOTE 3: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information. NOTE 4: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and "preferred level of accuracy per analytics subset". NOTE 5: This parameter is only provided for Target of Analytics Reporting set to single UE. NOTE 6: When Application ID is not included in the input, cardinality is zero and data volume dispersed is provided per UE location and applies across all the applications in that location. NOTE 7: This parameter is not provided for Target of Analytics Reporting set to “Any UE”. |  |

Table 6.10.3.1-2: Data volume dispersion prediction bound by location

| Information | Description |
| --- | --- |
| UE group ID or UE ID or list of UE IDs(1..SUPImax) | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, or a list of UEs for which for which the prediction applies (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > UE location (1..max) | Predicted location during the analytics target period. |
| >> UE location | TA or cells where the UE or UE group or list of UEs is predicted to disperse its data (see NOTE 3). |
| >> Application ID (0..max)(NOTE 7) | To identify the application (NOTE 6). |
| >>> Data volume dispersion (NOTE 4) | Data volume dispersion prediction at this location. |
| >>>> Confidence | Confidence of this prediction (i.e. data volume to be dispersed at this location). |
| >> Data mobility classification (NOTE 4) | Data mobility classification of the UE for this location: fixed, camper, traveller (see NOTE 5). |
| >>> Confidence | Confidence of this prediction (i.e. mobility classification at this location). |
| >> Data usage ranking (NOTE 4) (NOTE 7) | Ranking of UE data usage at this location (See NOTE 2). |
| >> Data percentile usage ranking (NOTE 4) (NOTE 7) | Percentile ranking of the UE or UE group in the Cumulative Distribution Function of data usage for the population of all UEs served at the location. |
| >>> Confidence | Confidence of this prediction (i.e. percentile ranking at this location). |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When Target of Analytics Reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. When Target of Analytics Reporting is a UE group Id, an internal group Id will be included. When Target of Analytics Reporting is a UE group Id or “Any UE” and a filter for Top-Heavy UEs, fixed, camper or traveller is included, the NWDAF will include the list of UEs matching the filter. This information element is not present when Target of Analytic Reporting is “Any UE” and no filter for Top-Heavy UEs, fixed, camper or traveller is included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE, group of UEs or list of UEs. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its data dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. NOTE 3: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information. NOTE 4: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and "preferred level of accuracy per analytics subset". NOTE 5: This parameter is only provided for Target of Analytics Reporting set to single UE. NOTE 6: When Application ID is not included in the input, cardinality is zero and data volume dispersion prediction is provided per UE location and applies across all the applications in that location. NOTE 7: This parameter is not provided for Target of Analytics Reporting set to “Any UE” |  |

The data volume dispersion analytics results provided by the NWDAF can also be for a UE or group of UEs or “any UE” data volume dispersion statistics at a given slice as defined in Table 6.10.3.1-3 and/or data volume dispersion predictions as defined in Table 6.10.3.1-4. If the analytics covers multiple UEs, the Data dispersed, Data ranking, and Data percentile ranking parameters are calculated as the aggregated (i.e. summed up) data volume. When the Target of Analytics Reporting is set as "any UE" the data volume dispersion analytics results provided by the NWDAF can be for all UEs data volume dispersion statistics at a given slice and/or data volume dispersion predictions:

Table 6.10.3.1-3: Data volume dispersion statistics bound by slice

| Information | Description |
| --- | --- |
| UE group ID or UE ID or a list of UE IDs(1..SUPImax) | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, or a list of UEs for which the statistic applies (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Slice (1..max) | Observed slice statistics. |
| >> S-NSSAI | Slice where the UE or group of UEs or list of UEs dispersed data. |
| >> Application ID (0..max)(NOTE 6) | To identify the application at the slice (NOTE 5). |
| >>> Data volume dispersed (NOTE 3) | Data volume dispersed at this slice. |
| >> Data mobility classification (NOTE 3) | Data mobility classification of the UE at the slice: fixed, camper, traveller (see NOTE 4). |
| >> Data usage ranking (NOTE 3)(NOTE 6) | Ranking of UE data usage at this slice. (See NOTE 2). |
| >> Data usage percentile ranking (NOTE 3) (NOTE 6) | Percentile ranking of the UE or UE group in the Cumulative Distribution Function of data usage for the population of all UEs served by the slice. |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When Target of Analytics Reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. When Target of Analytics Reporting is a UE group Id, an internal group Id will be included. When Target of Analytics Reporting is a UE group Id or “Any UE” and a filter for Top-Heavy UEs, fixed, camper or traveller is included, the NWDAF will include the list of UEs matching the filter. This information element is not present when Target of Analytic Reporting is “Any UE” and no filter for Top-Heavy UEs, fixed, camper or traveller is included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE, the group of UEs or the list of UEs. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its data dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. NOTE 3: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and "preferred level of accuracy per analytics subset". NOTE 4: This parameter is only provided for Target of Analytics Reporting set to single UE. NOTE 5: When Application ID is not included in the input, cardinality is zero and data volume dispersed is provided per slice and applies across all the applications in the slice. NOTE 6: This parameter is not provided for Target of Analytics Reporting set to “Any UE” |  |

Table 6.10.3.1-4: Data volume dispersion prediction bound by slice

| Information | Description |
| --- | --- |
| UE group ID or UE ID or list of UE IDs(1..SUPImax) | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, or a list of UEs for which the prediction applies (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Slice (1..max) | Predicted slice during the Analytics target period. |
| >> S-NSSAI | Slice where the UE or UE group or list of UEs is predicted to disperse its data. |
| >> Application ID (0..max)(NOTE 6) | To identify the application at the slice (NOTE 5). |
| >>> Data volume dispersion (NOTE 3) | Data volume dispersion prediction at this slice. |
| >>>> Confidence | Confidence of this prediction (i.e. data volume to be dispersed at this slice). |
| >> Data mobility classification (NOTE 3) | Data mobility classification of the UE at this slice: fixed, camper, traveller (see NOTE 4). |
| >>> Confidence | Confidence of this prediction (i.e. mobility classification at this slice). |
| >> Data usage ranking (NOTE 3)(NOTE 6) | Ranking of UE data usage at this slice. See NOTE 2. |
| >> Data usage percentile ranking (NOTE 3)(NOTE 6) | Percentile ranking of the UE or UE group in the Cumulative Distribution Function of data usage for the population of all UEs served by the slice. |
| >>> Confidence | Confidence of this prediction (i.e. percentile ranking at this slice). |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When Target of Analytics Reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. When Target of Analytics Reporting is a UE group Id, an internal group Id will be included. When Target of Analytics Reporting is a UE group Id or “Any UE” and a filter for Top-Heavy UEs, fixed, camper or traveller is included, the NWDAF will include the list of UEs matching the filter. This information element is not present when Target of Analytic Reporting is “Any UE” and no filter for Top-Heavy UEs, fixed, camper or traveller is included. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE, group of UEs or list of UEs. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its data dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. NOTE 3: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and "preferred level of accuracy per analytics subset". NOTE 4: This parameter is only provided for Target of Analytics Reporting set to single UE. NOTE 5: When Application ID is not included in the input, cardinality is zero and data volume dispersion prediction is provided per slice and applies across all the applications in the slice. NOTE 6: This parameter is not provided for Target of Analytics Reporting set to “Any UE”. |  |

Table 6.10.3.1-6: Data volume dispersion prediction bound by slice (for any UE)

| Information | Description |
| --- | --- |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Slice (1..max) | Predicted slice during the Analytics target period. |
| >> S-NSSAI | Slice where the UE is predicted to disperse its data. |
| >> Data volume dispersion | Data volume dispersion prediction at this slice. (see NOTE 1). |
| >>>> Confidence | Confidence of this prediction (i.e. data to be dispersed at this slice). |
| NOTE 1: Utilized bandwidth (i.e. average throughput) predication at the slice can be calculated by using the Data volume dispersion in the Duration. |  |

NOTE:	The Application ID in the Data Volume Dispersion Analytics is optional. When the Application ID is missing, the Data Volume Dispersion Analytics is applied across all the applications in an AOI or a slice.

* * * * Next Change * * * *

6.10.3.2	Transactions Dispersion Analytics

The transactions (MM and MS messages) dispersion analytics results provided by the NWDAF can be for a UE or group of UEs transactions dispersion statistics as defined in Table 6.10.3.2-1 and/or UE transactions dispersion predications as defined in Table 6.10.3.2-2. If the analytics covers multiple UEs, the Transactions dispersed, Transactions ranking, and Transactions percentile ranking parameters are calculated as the aggregated (i.e. summed up) number of transactions.

Table 6.10.3.2-1: Transactions dispersion statistics bound by location

| Information | Description |
| --- | --- |
| UE group ID or UE ID or list of UE IDs (1..SUPImax) | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, or a list of UEs for which for which the statistic applies (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > UE location (1..max) | Observed location statistics. |
| >> UE location | TA or cells where the UE or group of UEs or list of UEs dispersed its transactions (see NOTE 3). |
| >> Transactions dispersed (NOTE 4) | Transactions amount dispersed at this location. |
| >> Transactions mobility classification (NOTE 4) | Transactions mobility classification of the UE for this location: fixed, camper, traveller (see NOTE 5). |
| >> Transactions ranking (NOTE 4) | Ranking of transactions amount at this location. See NOTE 2. |
| >> Transactions percentile ranking (NOTE 4) | Percentile ranking of the UE or UE group in the Cumulative Distribution Function of transactions for the population of all UEs served at the location. |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When Target of Analytics Reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. When Target of Analytics Reporting is a UE group Id, an internal group Id will be included. When Target of Analytics Reporting is a UE group Id and a filter for Top-Heavy UEs, fixed, camper or traveller is included, the NWDAF will include the list of UEs matching the filter. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE, group of UEs or list of UEs. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its transactions are dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. NOTE 3: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information. NOTE 4: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and "preferred level of accuracy per analytics subset". NOTE 5: This parameter is only provided for Target of Analytics Reporting set to single UE. |  |

Table 6.10.3.2-2: Transactions dispersion prediction bound by location

| Information | Description |
| --- | --- |
| UE group ID or UE ID or list of UE IDs (1..SUPImax) | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, or a list of UEs for which for which the prediction applies (see NOTE 1). |
| Time slot entry (1..max) | List of predicted time slots. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > UE location (1..max) | Predicted location during the analytic target period. |
| >> UE location | TA or cells where the UE or group of UEs or list of UEs is predicted to disperse its transactions (see NOTE 3). |
| >> Transactions dispersion (NOTE 4) | Transaction amount predicted to be dispersed at this location. |
| >>> Confidence | Confidence of this prediction (i.e. data to be dispersed at this location). |
| >> Transactions mobility classification (NOTE 4) | Transaction mobility classification of the UE for this location: fixed, camper, traveller (see NOTE 5). |
| >>> Confidence | Confidence of this prediction (i.e. mobility classification at this location). |
| >> Transactions ranking (NOTE 4) | Ranking of transactions amount at this location. See NOTE 2. |
| >> Transactions percentile ranking (NOTE 4) | Percentile ranking of the UE or UE group in the Cumulative Distribution Function of transactions for the population of all UEs served at the location. |
| >>> Confidence | Confidence of this prediction (i.e. percentile ranking at this location). |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When Target of Analytics Reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. When Target of Analytics Reporting is a UE group Id, an internal group Id will be included. When Target of Analytics Reporting is a UE group Id and a filter for Top-Heavy UEs, fixed, camper or traveller is included, the NWDAF will include the list of UEs matching the filter. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE, group of UEs or list of UEs. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its transactions are dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. NOTE 3: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information. NOTE 4: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and "preferred level of accuracy per analytics subset". NOTE 5: This parameter is only provided for Target of Analytics Reporting set to single UE. |  |

The transactions dispersion analytics results provided by the NWDAF can be UE or group of UEs transactions dispersion statistics at a given slice as defined in table 6.10.3.2-3 and/or transactions dispersion predictions as defined in table 6.10.1.4.2-4. If the analytics covers multiple UEs, the Transactions dispersed, Transactions ranking, and Transactions percentile ranking parameters are calculated as the aggregated (i.e. summed up) number of transactions.

Table 6.10.3.2-3: Transactions dispersion statistics bound by slice

| Information | Description |
| --- | --- |
| UE group ID or UE ID or a list of UE IDs (1..SUPImax) | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, or a list of UEs for which the statistic applies (see NOTE1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Slice (1..max) | Observed slice statistics. |
| >> S-NSSAI | Slice where the UE or group of UEs or list of UEs dispersed its transactions. |
| >> Transactions dispersed (NOTE 3) | Transactions amount dispersed at this location. |
| >> Transactions mobility classification (NOTE 3) | Transaction mobility classification of the UE at the slice: fixed, camper, traveller (see NOTE 4). |
| >> Transactions ranking (NOTE 3) | Ranking of transactions amount at this slice. See NOTE 2. |
| >> Transactions percentile ranking (NOTE 3) | Percentile ranking of the UE or UE group in the Cumulative Distribution Function of transactions for the population of all UEs served by the slice. |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When Target of Analytics Reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. When Target of Analytics Reporting is a UE group Id, an internal group Id will be included. When Target of Analytics Reporting is a UE group Id and a filter for Top-Heavy UEs, fixed, camper or traveller is included, the NWDAF will include the list of UEs matching the filter. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE, the group of UEs or the list of UEs. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its transactions are dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. NOTE 3: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and "preferred level of accuracy per analytics subset". NOTE 4: This parameter is only provided for Target of Analytics Reporting set to single UE. |  |

Table 6.10.3.2-4: Transactions dispersion prediction bound by slice

| Information | Description |
| --- | --- |
| UE group ID or UE ID or list of UE IDs (1..SUPImax) | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, or a list of UEs for which the prediction applies (see NOTE1). |
| Time slot entry (1..max) | List of predicted time slots. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Slice (1..max) | Predicted slice during the Analytics target period. |
| >> S-NSSAI | Slice where the UE or group of UEs or list of UEs is predicted to disperse its transactions. |
| >> Transactions dispersion (NOTE 3) | Transaction volume to be dispersed at this slice. |
| >>> Confidence | Confidence of this prediction (i.e. transactions to be dispersed at this slice). |
| >> Transactions mobility classification (NOTE 3) | Transaction mobility classification of the UE at this slice: fixed, camper, traveller (see NOTE 4). |
| >>> Confidence | Confidence of this prediction (i.e. mobility classification at this slice). |
| >> Transactions ranking (NOTE 3) | Ranking of transactions amount at this slice. See NOTE2. |
| >> Transactions percentile ranking (NOTE 3) | Percentile ranking of the UE or UE group in the Cumulative Distribution Function of transactions for the population of all UEs served by the slice. |
| >>> Confidence | Confidence of this prediction (i.e. percentile ranking at this slice). |
| >> Ratio | Percentage of UEs in the group (in the case of UE group). |
| NOTE 1: When Target of Analytics Reporting is an individual UE, a single UE ID (i.e. SUPI) will be included. When Target of Analytics Reporting is a UE group Id, an internal group Id will be included. When Target of Analytics Reporting is a UE group Id and and a filter for Top-Heavy UEs, fixed, camper or traveller is included, the NWDAF will include the list of UEs matching the filter. The NWDAF will provide the dispersion analytics result of one or a list of time slots to the service consumer(s) for the UE group of UEs or list of UEs. NOTE 2: UE is ranked high (i.e. 1), medium (2) and low (3) when its transactions are dispersed, during the period of observation at the location, is higher than N1% or at the range of N1% to N2% or less than N2%, respectively. The percentage values of N1 and N2 are subject to operator's configuration. NOTE 3: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and "preferred level of accuracy per analytics subset". NOTE 4: This parameter is only provided for Target of Analytics Reporting set to single UE. |  |