3GPP TSG-SA2 Meeting #149E	S2-2200319r02

# Elbonia, February 14 - 25, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0500 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Analytics ID on Service Experience |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 28/01/2022 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | NWDAF can provide observed service experience analytics and accept both RAT type and Frequency as Analytics filter and include these parameters in the output. However, it is not possible for the analytic consumer to request the Observed Service Experience (OSE) analytics for any available combination of RAT Type and Frequency. This is valuable for a consumer such as the PCF that need to know the OSE over the available RAT types and Frequency values at NWDAF to be able to select the RFSP index value for a UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Description of Observed Service Experience analytics is updated to allow a service consumer to request the analytics per any available combination of RAT Type and Frequency. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification of the OSE Analytics ID |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1, 6.4.3, 6.4.4, 6.4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * 1st Change * * *

### 6.4.1	General

This clause specifies how NWDAF can provide Observed Service Experience (i.e. average of observed Service MoS and/or variance of observed Service MoS indicating service MOS distribution for services such as audio-visual streaming as well as services that are not audio-visual streaming such as V2X and Web Browsing services) analytics, in the form of statistics or predictions, to a service consumer.

The Observed Service Experience analytics may provide one or more of the following outputs:

-	Service Experience for a Network Slice: Service Experience for a UE or a group of UEs or any UE in a Network Slice;

-	Service Experience for an Application: Service Experience for a UE or a group of UEs or any UE in an Application or a set of Applications;

-	Service Experience for an Edge Application over a UP path: Service experience for a UE or a group UEs or any UE in an Application or a set of Applications over a specific UP path (UPF, DNAI and EC server);

-	Service Experience for an Application over a RAT Type or Frequency or both: Service experience for a UE or a group of UEs in an Application or a set of Applications over a RAT Type or over a Frequency or both as defined in Table 6.4.1-1.

Therefore, Observed Service experience may be provided as defined in clause 6.4.3. For example, individually per UE or group of UEs, or globally, averaged per Application or averaged across a set of Applications on a Network Slice.

The service consumer may be an NF (e.g. PCF, NSSF, AMF), AF, or the OAM.

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = "Service Experience";

-	Target of Analytics Reporting: one or more SUPI(s) or Internal Group Identifier(s), or "any UE";

-	Analytics Filter Information as defined in Table 6.4.1-1;

-	optionally, maximum number of objects and maximum number of SUPIs;

-	optionally, preferred level of accuracy of the analytics;

-	optionally, preferred level of accuracy per analytics subset (Network Slice instance service experience, Application service Experience);

-	optionally, preferred order of results for the list of Application Service Experiences and/or Slice instance service experiences: "ascending" or "descending"; and

-	optionally, preferred granularity of location information: TA level or cell level.

-	optionally, Reporting Threshold, which applies only for subscriptions and indicates conditions on the Service Experience to be reached in order to be notified by the NWDAF (see Table 6.4.3-1).

Table 6.4.1-1: Analytics Filter Information related to the observed service experience

| Information | Description | Mandatory |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Application | Network Slice | Edge Applications over a UP path | Application over RAT Type and frequency |
| Application ID (0...max) | The identification of the application(s) for which the analytics information is subscribed or requested. | Y | N | Y | Y |
| S-NSSAI | When requesting Service Experience for a Network Slice: identifies the Network Slice for which analytics information is subscribed or requested. When requesting Service Experience for an Application: identifies the S-NSSAI used to access the application together with the DNN listed below. | N | Y | N | N |
| NSI ID(s) | Identifies the Network Slice instance(s) for which analytics information is subscribed or requested. | N | N | N | N |
| Area of Interest | Identifies the Area (i.e. set of TAIs), as defined in TS 23.501 [2] for which the analytics information is subscribed or requested. | N | N | N | N |
| DNN | When requesting Service Experience for an Application, this is the DNN to access the application. | N | N | N | N |
| DNAI (NOTE 1) | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. | N | N | Y | N |
| RAT Type (NOTE 2) | Identifies the RAT type. | N | N | N | Y |
| Frequency (NOTE 2) | Identifies the Frequency value(s) (e.g., high, low) | N | N | N | Y |
| Application Server Address(es) (NOTE 1) | List of IP address(s)/FQDN(s) of the Application Server(s) that a UE, group of UEs, or 'any UE' has a communication session for which Service Experience Analytic information is requested. | N | N | Y | N |
| UPF anchor ID (NOTE 1) | Identifies the UPF where a UE has an associated PDU session | N | N | N | N |
| NOTE 1: These parameters can be provided when a consumer requires analytics for an edge application over a UP path. NOTE 2: A service consumer can provide either a RAT Type or a Frequency or a specific combination of RAT Type and Frequency. A service consumer can also provide multiple instances of RAT Type or multiple instances of Frequency or multiple combinations of RAT type and Frequency. A service consumer can also provide “any” RAT type indication， “any” Frequency value indication or “any” indication for all the RAT type and Frequency value the NWDAF has received for the application. |  |  |  |  |  |

NOTE:	A service consumer can use the Area of Interest in order to reduce the amount of signalling that the analytics subscription or request generates.

-	An Analytics target period that indicates the time window for which the statistics or predictions are requested;

-	In a subscription, the Notification Correlation Id and the Notification Target Address;

-	Optionally, Reporting Thresholds, which apply only for subscriptions and indicate conditions on the level to be reached for the Service Experience (see clause 6.4.3) in order to be notified by the NWDAF.

The NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.4.3.

NWDAF collects the network data from AF (directly or via NEF) and from other 5GC NF(s) in order to calculate and provide statistics and predictions on the observed service experience to a consumer NF or to OAM.

Based on the Analytics Filter information in Table 6.4.1-1 and the Target of Analytics Reporting provided by the service consumer in the analytics subscription or request, NWDAF determines whether service experience analytics should be delivered for:

i)	Application(s)

ii)	Network Slice;

iii)	both Application(s) and Network Slice;

iv)	Edge Applications over a UP path;

v)	Application(s) over RAT Type(s) and/or Frequency value(s).

If NWDAF is unable to differentiate based on the analytics subscription or request, it provides service experience analytics for both Application(s) and Network Slice.

If service experience for both Application(s) and Network Slice is desired but the Target of Analytics Reporting or Analytics Filter information values (e.g. Area of Interest) need to be different, separate subscriptions/requests may be provided by the service consumer.

* * * Next Change * * *

### 6.4.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics.

-	Service Experience statistics information is defined in Table 6.4.3-1.

-	Service Experience predictions information is defined in Table 6.4.3-2.

Table 6.4.3-1: Service Experience statistics

| Information | Description |
| --- | --- |
| Slice instance service experiences (0..max) (NOTE 1) | List of observed service experience information for each Network Slice instance. |
| > S-NSSAI | Identifies the Network Slice |
| > NSI ID | Identifies the Network Slice instance within the Network Slice. |
| > Network Slice instance service experience | Service experience across Applications on a Network Slice instance over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) for which the slice instance service experience applies. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Network Slice service experience analytics applies. |
| > Validity period | Validity period for the Network Slice service experience analytics as defined in clause 6.1.3. |
| Application service experiences (0..max) (NOTE 1) | List of observed service experience information for each Application. |
| > S-NSSAI | Identifies the Network Slice used to access the Application. |
| > Application ID | Identification of the Application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| > UE location | Indicating the UE location information (e.g. TAI list, gNB ID, etc) when the UE service is delivered (see NOTE 4). |
| > UPF Info | Indicating UPF serving the UE. |
| > DNAI | Indicating which DNAI the UE service uses/camps on. |
| > DNN | DNN for the PDU Session which contains the QoS flow. |
| > Application Server Instance Address | Identifies the Application Server Instance (IP address of the Application Server) or FQDN of Application Server. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) with the same application service experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |
| > RAT Type (NOTE 2) | Indicating the list of RAT type(s) for which the application service experience analytics applies. |
| > Frequency (NOTE 2) | Indicating the list of carrier frequency value(s) of UE's serving cell(s) where the application service experience analytics applies. |
| NOTE 1: Whether analytics subset is included or not is determined by the Analytics Filter Information. NOTE 2: When “any” value has been provided in the request (e.g. “any” RAT type, “any” frequency, or “any” for all the RAT type and frequency indication), the NWDAF provides an instance of the Application service experience per combination of RAT Type(s) and/or Frequency value(s) having the same Service Experience. |  |

Table 6.4.3-2: Service Experience predictions

| Information | Description |
| --- | --- |
| Slice instance service experiences (0..max) (NOTE 1) | List of observed service experience information for each Network Slice instance. |
| > S-NSSAI | Identifies the Network Slice |
| > NSI ID | Identifies the Network Slice instance within the Network Slice (see NOTE 2). |
| > Network Slice instance service experience | Service experience across Applications on a Network Slice instance over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) for which the slice instance service experience applies (see NOTE 3). |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs) (see NOTE 3). |
| > Spatial validity | Area where the Network Slice service experience analytics applies. |
| > Validity period | Validity period for the Network Slice service experience analytics as defined in clause 6.1.3. |
| > Confidence | Confidence of this prediction. |
| Application service experiences (0..max) (NOTE 1) | List of predicted service experience information for each Application. |
| > S-NSSAI | Identifies the Network Slice used to access the Application. |
| > Application ID | Identification of the Application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| > UE location | Indicating the UE location information (e.g. TAI list, gNB ID, etc) when the UE service is delivered (see NOTE 5). |
| > UPF Info | Indicating UPF serving the UE (see NOTE 5). |
| > DNAI | Indicating which DNAI the UE service uses/camps on. |
| > DNN | DNN for the PDU Session which contains the QoS flow. |
| > Application Server Instance Address | Identifies the Application Server Instance (IP address of the Application Server) or FQDN of Application Server. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) with the same application service experience (see NOTE 3). |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs) (see NOTE 3). |
| > Spatial validity | Area where the Application service experience analytics applies (see NOTE 6). |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |
| > Confidence | Confidence of this prediction. |
| > RAT Type (NOTE 7) | Indicating the list of RAT type(s) for which the application service experience analytics applies. |
| > Frequency (NOTE 7) | Indicating the list of carrier frequency value(s) of UE's serving cell(s) where the application service experience analytics applies. |
| NOTE 1: Whether analytics subset is included or not is determined by the Analytics Filter Information. NOTE 2: The NSI ID is an optional parameter. If not provided the Slice instance service experience indicates the service experience for the S-NSSAI. NOTE 3: The SUPI list and Ratio in the service experience information for an application can be omitted, if the corresponding parameter(s) is/are provided and are assigned with the same value(s) in the service experience information for the slice instance which the application belongs to. Otherwise, the SUPI list and Ratio are mandatory to be provided for an application service experience. NOTE 4: If the consumer NF is an AF, the item "DNN" and "UPF info" shall not be included, and the NEF is responsible for translation of SUPI to GPSI, internal group identifiers to external ones, and UE location to geographical area, by querying UDM, prior to contacting the AF. NOTE 5: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information. NOTE 6: The Spatial validity is present in the output parameters if the consumer provided the Area of Interest as defined in Table 6.4.1-1. NOTE 7: When “any” value has been provided in the request (e.g. “any” RAT type, “any” frequency, or “any” for all the RAT type and frequency indication), the NWDAF provides an instance of the Application service experience per combination of RAT Type(s) and/or Frequency value(s) having the same Service Experience. |  |

The number of Service Experiences and SUPIs are limited respectively by the maximum number of objects and the Maximum number of SUPIs provided as part of Analytics Reporting Information by the NWDAF Service Consumer.

* * * Next Change * * *

### 6.4.4	Procedures to request Service Experience for an Application

Figure 6.4.4-1: Procedure for NWDAF providing Service Experience for an Application

This procedure allows the consumer to request Analytics ID "Service Experience" for a particular Application. The consumer includes both the Application ID for which the Service Experience is requested and indicates that the Target of Analytics Reporting is "any UE". If the Target for Analytics Reporting is either a SUPI or an Internal-Group-Id the procedure in clause 6.4.6 applies. At the same time, for an Application ID, a set of initial QoS parameter combinations per service experience window (e.g. one is for 3<Service MOS<4 and another is for 4<Service MOS<5) is defined in PCF (e.g. by configuration of operator policies) that may be updated based on the Service Experience reported by NWDAF.

1.	Consumer NF sends an Analytics request/subscribe (Analytics ID = Service Experience, Target of Analytics Reporting = any UE, Analytics Filter Information that may include one or more of the following as defined in Table 6.4.4-1(Application ID, S-NSSAI, DNN, Application Server Address(es), Area of Interest, RAT type(s), Frequency value(s)), Analytics Reporting Information=Analytics target period) to NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

2a. NWDAF subscribes the service data from AF in the Table 6.4.2-1 by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe service (Event ID = Service Experience information, Application ID, Event Filter information), Target of Event Reporting = Any UE) as defined in TS 23.502 [3].

NOTE 1:	In the case of trusted AF, NWDAF provides the Area of Interest as a list of TAIs to AF. In the case of untrusted AF, NEF translates the requested Area of Interest provided as event filter by NWDAF into geographic zone identifier(s) that act as event filter for AF.

2b.	NWDAF subscribes the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation.

2c. With these data, the NWDAF estimates the Service experience for the application.

NOTE 2:	QoE measurements from the applications are based on outcome of the ongoing SA5 Rel-16 WID "Management of QoE measurement collection" which addresses how to collect the QoE measurements from the applications in the UE.

3.	The NWDAF provides the data analytics, i.e. the observed Service Experience (which can be a range of values) to the consumer NF by means of either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1, indicating how well the used QoS Parameters satisfy the Service MoS agreed between the MNO and the end user or between the MNO and the external ASP.

NOTE 3:	The call flow only shows a request-response model for the interaction of NWDAF and consumer NF for simplicity instead of both request-response model and subscription-notification model.

NOTE 4:	The non-real time data information from AF includes the service experience data (see Table 6.4.2-1), which indicates the service quality during the service lifetime.

If the consumer NF is a PCF and it determines that the application SLA is not satisfied, it may take into account the Observed Service Experience and the operator policies including SLA and required Service Experience (which can be a range of values) to determine new QoS parameters to be applied for the service, as defined in clause 6.1.1.3 and clause 6.2.1.2 of TS 23.503 [4].

If the consumer NF is an AF (e.g. MEC or other Application Server), it may use the Observed Service Experience related network data analytics to determine whether the user experience can be satisfied. If not, the AF may determine to adjust service parameters, e.g. for a video service this may be bit rate, frame rate, codec format, compression parameter, screen size, etc., to better match the network conditions and achieve better user experience.

If the consumer NF is SMF, PCF or AF/Application Server, it may take into account the Observed Service Experience analytics per UP path (i.e. UPF and/or DNAI and/or AS instance address as defined in Table 6.4.3-1) to perform the following procedures:

-	The consumer SMF determines to (re)selects UP paths, including UPF and DNAI, as described in clause 4.3.5 of TS 23.502 [3]. In addition, the SMF may (re)configure traffic steering, updating the UPF regarding the target DNAI with new traffic steering rules.

-	The consumer AF/Application Server determines to adjust service parameters, e.g. service parameters of video for adjustment may be bit rate, frame rate, codec format, compression parameter, screen size, etc. In addition, the AF/ Application Server may provide an updated list of DNAI(s) for SMF to perform relocation when appropriate.

-	The consumer PCF may provide an updated list of DNAI(s) for SMF to perform relocation upon AF request.

* * * Next Change * * *

### 6.4.6	Procedures to request Service Experience for a UE

Figure 6.4.6-1 depicts procedure for NWDAF providing Service Experience for an application for a UE or a group of UEs.

Figure 6.4.6-1: Procedure for NWDAF providing Service Experience for an application for a UE or a group of UEs

The procedure in clause 6.4.4 applies with the following additions. The consumer needs to request the Analytics ID "Service Experience" for a UE identified by a SUPI or a group of UEs identified by an Internal-Group-Id. The consumer includes both the Application ID for which their Service Experience is requested and the Target of Analytics Reporting for a SUPI or an Internal-Group-Id. Analytic Filter Information can be set according to clause 6.4.1. When NEF is the NF service consumer, the NEF translates a GPSI into a SUPI or an External-Group-Id into an Internal-Group-Id then includes it in the Target of Analytics Reporting.

* * * End of Changes * * *