3GPP TSG-SA WG2 Meeting #149E (e-meeting)	S2-2200372

# February 14 – 25, 2022, Elbonia											     (revision of S2-22xxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0075 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on MBS Session Update procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clauses 7.1.1.6 and 7.1.1.7 describes that the AF can update the MBS service area. If the updated MBS session is for local multicast services or location dependent multicast services, the AF may perform a Service Announcement towards UEs to update the MBS service area. However, the description about Service Announcement is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clauses 7.1.1.6 and 7.1.1.7: - Add description about Service Announcement to update the MBS service area. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MBS Session Update procedures are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6, 7.1.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

#### 7.1.1.6	MBS Session Update without PCC

This procedure is used by the AF to update the MBS service area and/or update QoS of an MBS Session. Updating QoS of an MBS Session may lead to addition of new MBS QoS Flow(s). The procedure applies to both multicast and broadcast communications unless otherwise stated.

Figure 7.1.1.6-1: MBS Session Update without PCC

1.	AF of content provider initiates MBS Session Update to a NEF/MBSF, e.g. to update MBS service area and/or update service requirement, or to activate or deactivate an MBS session. AF may provide updated information for an MBS session (identified by MBS session ID) by sending MBS Session update request ([MBS Session ID], MBS information, AF Identifier). MBS information may include service requirements, MBS service area information, and media information.  The service requirements adjustment may lead to addition of new MBS QoS Flow(s), removal of existing MBS QoS Flow(s) or update of existing MBS QoS Flow(s).

2.	NEF checks authorization of AF.

3.	NEF/MBSF forward the MBS Session Update request to MB-SMF.

4.	The MB-SMF derives any updated QoS parameters locally.

5-6.	MB-SMF may need to update MB-UPF, e.g. if new MBS QoS Flow is to be created, or existing MBS QoS Flow is to be deleted.

7.	For broadcast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.3.3. For multicast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.2.5 (for service activation/deactivation), 7.2.6 (for QoS updates) and 7.2.x (for service area updates).

8.	If an MBS service area is being updated, the MB-SMF stores the new service area in its profile at the NRF.

9.	MB-SMF responds to the MBS Session Update.

10.	NEF/MBSF responds to the MBS Session Update.

For local multicast services and location dependent multicast services, the AF may perform a Service Announcement towards UEs to update the MBS service area before step 1 or after step 10.

* * * * Start of Next Change * * * *

#### 7.1.1.7	MBS Session Update with PCC

Figure 7.1.1.7-1: MBS Session Update with PCC

1-2.	Same as in Figure 7.1.1.6-1.

For updates of MBS service area and/or MBS session activation/deactivation steps 3 to 6 apply

3.	NEF/MBSF forward the MBS Session Update request to MB-SMF, removing any updates not related to MBS service area and/or MBS session activation/deactivation

4.	For broadcast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.3.3. For multicast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.2.5 (for service activation/deactivation), and 7.2.x (for service area updates).

5.	If an MBS service area is being updated, the MB-SMF stores the new service area in its profile at the NRF.

6.	MB-SMF responds to the MBS Session Update.

For other updates of the service description and QoS related updates, steps 7 to 12 apply.

7.	NEF/MBSF updates the MBS policy Authorization for the MBS session at the PCF and provides the input received from the AF, removing any updates related to MBS service area and/or MBS session activation/deactivation, by sending Npcf_MBSPolicyAuthorization_Update Request message (MBS Session ID, service requirement).

8.	Based on the input received in step 7, the PCF may provide updated policy rules to the MB-SMF by issuing Npcf_MBSPolicyControl_UpdateNotify request message including the updated policy information about the MBS Session.

9-10.	Same as steps 5-6 in Figure 7.1.1.6-1.

11.	For broadcast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.3.3. For multicast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.2.6 (for QoS updates).

12.	The PCF sends the Npcf_MBSPolicyAuthorization_Update response to the NEF/MBSF.

13.	Same as step 10 in Figure 7.1.1.6-1.

For local multicast services and location dependent multicast services, the AF may perform a Service Announcement towards UEs to update the MBS service area before step 1 or after step 13.

* * * * End of Changes * * * *