SA WG2 Meeting #149e	S2-2200455r03

# February 14th – 25th, 2022; Elbonia

Source: 	Thales, Xiaomi, [Huawei?], [HiSilicon?], [Samsung?]

Title: 	FS_5GSAT_Ph2 TR KI on description of discontinuous coverage modelling

Document for: 	Approval

Agenda Item: 	9.24

Work Item / Release:	FS_5GSAT_Ph2 / Rel-18

Abstract of the contribution: Propose new key issues for supporting discontinuous coverage.

# 1	Discussion

The FS_5GSAT_Ph2 SID justification read

There are features proposed in RAN WG having 5GC impact, i.e. Discontinuous coverage. Dynamic support of discontinuous coverage is required for initial NGSO constellation deployment but as well to support evolution of the constellations such as loss of satellites, different releases supported in a given constellation. UE may have access to satellite service coverage only at specific time and places due to sparse constellation. UE location may not be timely aware of by the network to enable efficient paging, due to which mobility management mechanism needs to be enhanced. Moreover, UE may not always have to stay awake for the sake of power efficiency, especially for MIoT UE. Hence, the prediction, mechanisms on awareness & notification of UE wake-up time and data storage & forwarding for UEs temporarily out of coverage may be needed. There is also an important aspect of discontinuous coverage related to dynamic GEO systems where beams/cells are intermittent due to use of dynamic beam configuration.

The FS_5GSAT_Ph2 SID objectives read

Work task #1: Discontinuous coverage

- WT#1.1: Architectural enhancements to support discontinuous coverage for mobility enhancement (e.g. paging enhancement)

- WT#1.2: Architectural enhancements considering prediction, awareness & notification of UE wake-up time, power saving optimizations.

This contribution intends to introduce the modelling of discontinuous coverage. Before any further enhancements, it is essential to get aware of the coverage information of a UE. In Rel-17, Tracking Area or RAT specific MME configuration has been introduced to support UEs using a RAN that provides discontinuous coverage (e.g. for satellite access with discontinuous coverage).

However, the TA or RAT granularity is too coarse. Especially, for satellite access, the satellite coverage is much larger than the terrestrial one. Basing on the TA or RAT granularity is not accurate and the coverage status in UE and CN side may be different.

Besides, how the CN gets aware of the coverage information is still not clear. For example, the RAN node, the OAM or the UE can derive the coverage information and deliver it to the CN or the CN can just derive the coverage information based on the satellite ephemeris for further enhancements. Therefore, study on the awareness of coverage information for UE in discontinuous coverage is needed in Rel-18.

NOTE:	R01 is provided to merge the contents of S2-2200941 & S2-2201171, and provide a baseline for further discussion during SA2#149e.

# 2 Proposal

It is proposed to update TR 23.700-28 on FS_5GSAT_Ph2 as follows

# 5	Key Issues

## 5.X	Key Issue #X: Description of discontinuous coverage modelling.

### 5.X.1	Description

In order to support a UE using a satellite access that provides discontinuous coverage, it is essential to get aware of the coverage information of a UE. In Rel-17, Tracking Area or RAT specific MME configuration has been introduced in EPS. However, it is too coarse and may lead to inconsistency awareness of the coverage status between UE and the CN, especially for the satellite network where the coverage area is much larger. How 5GS becomes aware of the coverage information for the UE is still not clear.

The key issue intends to study the essential issues for discontinuous coverage modelling. At least the following aspects need to be further studied:

- 	Characterization of the discontinuity in terms of nature, timing (triggering event, periodicity), other

- 	The granularity of coverage information in discontinuous satellite coverage, e.g. TA, RAT, camped cell, other, etc., considering the satellite cell coverage is large comparing with TN cell.

-	How the 5GC becomes aware of the coverage information for a UE in discontinuous coverage, e.g. directly from RAN/UE, derived based on the satellite ephemeris, other

Note: for backward compatibility, solutions for this KI shall leverage on R17 solution, based on Tracking Area and RAT specific MME configuration, introduced to support discontinuous coverage for satellite access in EPS.