3GPP TSG-SA WG2 Meeting #149E	S2-2200466

Elbonia, February 14 - 25, 2022

# Title:	[Draft] Reply LS on MBS session restoration for NG-RAN failure with or without restart

# Response to:	(S2-2200199/C4-220308) LS on MBS session restoration for NG-RAN failure with or without restart

# Release:	Rel-17

# Work Item:	5MBS

Source:	Ericsson [will be SA2]

To:	CT4

Cc:	RAN3

Contact Person:

Name:	Robbie Ling

Tel. Number:

E-mail Address:	Robbie.Ling@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

SA2 thanks CT4 for the LS on MBS session restoration for NG-RAN failure with or without restart. SA2 discussed the LS and would like to provide following feedback:

With NF (service) Set deployment and relevant restoration procedures specified for CUPS interfaces, CT4 considers that the MBS session can be seamlessly restored when there is a failure in 5GC entities related to a MBS session, though CT4 needs to document this in their specification However, CT4 believes further specification is required to restore an MBS session when detecting a NG-RAN failure with restart, as well as to stop the delivery of MBS data towards an NG-RAN that fails without restart for unicast transport. It was pointed out during the discussion that additional use cases exist, not linked to NG-RAN failure/restart, where an existing MBS session would also need to be started in an NG-RAN, e.g. when a (new) NG-RAN is switched on in a TAI or is reconfigured by OAM to serve a new TAI that is part of an MBS service area of an existing broadcast MBS session, where similar procedures as for an NG-RAN restart could possibly apply. CT4 kindly asks SA2 to provide feedback on whether they will specify a procedure to start an MBS session (for an existing MBS session) in a (new) NG-RAN for the aforementioned non restoration related use cases, and feedback if any on the procedures to restore an MBS session upon detection of a NG-RAN failure/restart.

SA2 response:

Broadcast and multicast need to be discussed separately.

For multicast, SA2 assumes that (re)started NG RAN nodes will obtain information about UEs in multicast MBS sessions as part of the PDU session information and then contact the MB-SMF to establish shared delivery and obtain additional information about the MBS multicast session.

For broadcast, it has been proposed in SA2 that the AMF can handle the restoration, start or reconfiguration of RAN nodes mostly autonomously based on stored information about the broadcast MBS session as the MB-SMF should not require knowledge about RAN nodes. SA2 suggests that CT4 also discusses such alternatives. SA2 recognises that NG RAN or AMF triggered interactions with the MB-SMF may be required during ongoing broadcast sessions, for instance to terminate or start MBS data delivery during ongoing MBS broadcast sessions, or to notify the MB-SMF about (re) started or removed NG RAN nodes. SA2 intends to further study this issue and agree possibly needed related procedures at the next meeting.

.

For the restoration procedure, as it is usually under CT4’s remit, SA2 respectfully asks CT4 to further progress the work and suggest possible improvements. SA2 can make necessary alignment with CT4, if needed.

2. Actions:

To CT4:

ACTION: 	SA2 kindly asks CT4 further progress the work on MBS session restoration for NG-RAN failure  and suggest possible improvements..

3. Date of Next TSG SA WG2 Meetings:

https://portal.3gpp.org/Home.aspx?tbid=385&SubTB=385#/