SA WG2 Meeting #S2-149E	S2-2200492r04

14 - 25 Februrary, 2022, E-Meeting

Source: 	Ericsson, OPPO

Title: 	Solution (WT#1): Enable efficient mobility via equivalent SNPNs

Document for: 	Agreement

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_ph2 / Rel-18

Abstract of the contribution: This paper proposes a solution to address the Key Issue #X: Support for enhanced mobility between SNPNs without new network selection

Discussion

In order to enhance a more efficient idle and connected mode mobility between SNPNs without new network selection, key issue #X is introduced.

This solution considers the following two different network deployments and the UE mobility between the involved SNPNs:

-	UE accesses multiple SNPNs using Rel-17 credentials holder function.

-	UE accesses multiple SNPNs belonging to the same administrative entity (e.g. the same enterprise).

Proposal

Add the following solution to TR 23.700-08.

*** BEGIN CHANGES ***

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[X]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

*** NEXT CHANGES ***

6.x	Solution #x: Enable efficient mobility via equivalent SNPNs

6.X.1	Introduction

The solution addresses key issue #x "Enhanced mobility between SNPNs without new network selection".

The solution utilizes a list of SNPN identities (i.e. a list of combinations of PLMN ID and NID) to enable UE with one single SNPN subscription efficiently access different SNPNs without performing new network selection. The list is implemented by the similar logic as the list of equivalent PLMNs, as specified in TS 23.501 [X] clause 5.18.2a.

The solution also re-use existing function as specified in TS 23.501 [X] clause 5.18.1, where different combination of PLMN ID and NID can point to the same 5GC.

6.X.2	Functional Description

In existing specification, the UE can receive a list of equivalent PLMNs from core network. Such equivalent PLMN list will assist UE to select a cell from another PLMN during mobility without the need to perform new network selection.

In this solution, a list of equivalent SNPNs is proposed to facilitate the idle and connected mobility between SNPNs, reusing similar functions which are defined for PLMN:

-	The core network can provide a list of SNPN identities to the UE that the UE consider as equivalent to the registered SNPN.

-	The list of equivalent SNPNs can be prepared based on AMF local configuration.

-	The UE stores the list of equivalent SNPNs, and update or delete the list at the end of each registration procedure in the same way as done with the list of equivalent PLMNs.

-	The stored list consists of a list of equivalent SNPNs’ identities as downloaded by the network plus the SNPN identity of the registered SNPN that downloaded the list. When the UE is switched off, the UE shall keep the stored list so that it can be used for SNPN selection after switch on.

-	The lists of equivalent SNPNs is stored and used per SNPN subscription by the UE.

-	These SNPNs in the list shall be regarded by the UE as equivalent to each other for SNPN selection and cell (re)selection.

-	The list can also be provided from AMF to NG-RAN for the purpose of connected mode mobility.

6.X.3	Procedures

When UE accesses SNPNs using the credentials holder function as depicted in Figure 6.x.3-1, the following applies:

-	SNPN-1 and SNPN-2 are identified by different SNPN identity, i.e. combination of PLMN ID and NID.

-	SNPN-1 and SNPN-2 is the Credentials Holder(CH) for each other:

-	UE of SNPN-1 can register in SNPN-2, where using SNPN-1 as the CH.

-	UE of SNPN-2 can register in SNPN-1, where using SNPN-2 as the CH.

-	NFs from one SNPN are only authorized to consume services from the other SNPN which is acting as CH, as depicted in TS 23.501 [X] Figure 5.30.2.9.3-1.

-	UE from each SNPN has only the subscription/credential from the respective SNPN:

-	UE of SNPN-1 has only subscription/credential from SNPN-1.

-	UE of SNPN-2 has only subscription/credential from SNPN-2.

-	When UE holds the list of equivalent SNPNs(SNPN-1 and SNPN-2), it efficiently makes cell (re)selection when doing idle mode mobility between the SNPN-1's NG-RAN and SNPN-2's NG-RAN, without the need to perform network selection.

Figure 6.x.3-1 UE access multiple SNPNs using CH

When UE access multiple SNPNs belonging to the same administrative entity as depicted in Figure 6.x.3-2, the following applies:

-	SNPN-1 and SNPN-2 are identified by different SNPN identity, i.e. combination of PLMN ID and NID.

-	SNPN-1 and SNPN-2 belong to the same administrative entity. A common 5GC is used and can be managed by the same administrative entity to support both SNPN identities of SNPN-1 and SNPN-2.

-	NFs of one SNPN can be authorized to consume services from NFs of the other SNPN.

-	UE from each SNPN has only the subscription from the respective SNPN.

-	When UE is provided with the list of equivalent SNPNs(SNPN-1 and SNPN-2), it efficiently makes cell (re)selection when doing idle mode mobility between the SNPN-1's NG-RAN and SNPN-2's NG-RAN, with all the mobility and session context transferred, but without the need to perform network selection. PDU session continuity is supported.

-	When NG-RAN is provided with the list of equivalent SNPNs(SNPN-1 and SNPN-2) in the MRL, it make uses of such info to achieve connected mode mobility between SNPN-1 and SNPN-2.

In the case of handover or network controlled release to a shared network:

-	When multiple SNPN IDs are broadcasted in a cell selected by NG-RAN, NG-RAN selects a target SNPN, taking into account the list of SNPN IDs which are equivalent to the serving SNPN in the Mobility Restriction from the AMF.

-	For Xn based HO procedure, Source NG-RAN indicates the selected SNPN ID to the target NG-RAN.

-	For N2 based HO procedure, the source NG-RAN indicates a selected SNPN ID to the AMF in the HO required message. Source AMF uses the selected SNPN ID information supplied by the source NG-RAN to select the target AMF. The source AMF should forward the selected SNPN ID to the target AMF. The target AMF indicates the selected SNPN ID to the target NG-RAN so that the target NG-RAN can select target cells for future handover appropriately.

Serving SNPN ID if changed is indicated to the UE as part of the UE registration procedure.

Figure 6.x.3-2 UE access multiple SNPNs belonging to the same administrative entity

6.X.4	Impacts on services, entities, and interfaces

UE and AMF support of equivalent SNPN list in NAS.

NG-RAN and AMF support of equivalent SNPNs in NGAP.

Minor impact on the following clauses in TS 23.501 [X]:

-	5.18.1 NOTE 3

-	5.18.2a

*** END CHANGES ***