3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200580r01

Elbonia, February 14th – 25th, 2022	(merge of S2-2200580, S2-2200754, S2-2200837, S2-2201125)

Source:	vivo, China Telecom, Nokia, Interdigital Inc.

Title:	Key Issue: 5GC supports authorization in PIN

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	FS_PIN / Rel-18

Abstract: this paper proposes a key issue on 5GC supports authorization in PIN.

1. Introduction

According to the SP-211643, the study on Personal IoT Networks, the following work tasks needs to be studied:

- To study the management of a PIN, e.g., create PIN, authorizing/de-authorizing PIN Elements, authorizing/de-authorizing PIN Elements with Management Capability (PEMC), authorizing/de-authorizing PIN Elements with Gateway Capability (PEGC), establishing duration of the PIN, etc.

The management and communication of PIN needs to be authorized, most of the authorization can be done over application layer, but the authorization needs to be enforced by 5GC, and 5GC also can support some basic authorization, e.g., whether a UE subscribed PIN feature, whether a UE supports to be a PEGC, etc.

The main related stage 1 requirements are specified in TS 22.261 and assumed relevant requirements for the objective mentioned above are:

- The 5G system shall support mechanisms for a network operator or authorized 3rd party (e.g., a PIN User) to create, remove and manage a PIN, including:

-	Authorizing/deauthorizing PIN Elements;

-	Authorizing/deauthorizing PIN Elements with Management Capability;

-	Authorizing/deauthorizing PIN Elements with Gateway Capability;

-	Configure PIN Elements to enable service discovery of other PIN Elements;

-	Authorize/deauthorise if a PIN Element can use a PIN Element with Gateway Capability to communicate with the 5GS;

-	Authorize/deauthorise for a PIN Element(s):

-	which other PIN Element it can communicate with,

-	which applications/service or service in that PIN it can access,

-	which PIN Element it can use as a relay.

-	Authorize/deauthorise a UE to perform service discovery of PIN Elements over the 5G network;

-	Configure a PIN Element for external connectivity e.g.via 5G system;

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-88.

# * * * * First change * * * *

5	Key Issues

5.X	Key Issue #X: 5GC supports authorization for PIN

5.X.1	Description

The owner of a PIN may configure authorization information for the PIN, e.g., whether a PINE can communicate with other PINEs or with a specific data network, whether a UE is allowed to act as a PEMC, whether a PEGC is allowed to perform relay, etc.

The following aspects will be studied:

-	How 5GC supports authorization in a PIN, including but not limited to following aspects:

-	How to authorize/de-authorize PIN Elements to communicate within the PIN.

-	How to authorize/deauthorize a PIN element to communicate with other PIN elements or 5GS

-	How to authorize/de-authorize PIN Elements with Management Capability (PEMC) to manage the PIN and provide configuration for the PIN and PIN Elements.

-	How to authorize/de-authorize PIN Elements with Gateway Capability (PEGC) to provide connectivity to and from the 5G network for other PIN Elements that is not capable to access the 5G network.

-	Whether and how a UE that is capable of acting as a PEGC and/or PEMC can inform the 3GPP network about its capabilities. How to associate the UE identified as PEGC and/or PEMC to a specific PIN. Both the aspects of static and dynamic association of UEs capable of acting as PEGC and/or PEMC with a specific PIN shall be studied.

-	Whether and how the 3GPP network can authenticate and/or authorize a PEGC and/or PEMC capable UE to act as a GW and/or Manager of a specific PIN and also study the behaviour of the UE and the network upon failure of such authorization.

-	When there are multiple PEGC capable UEs present in a specific PIN, whether the PIN Gateway UE can change from one UE to another UE and how a PEGC capable UE or a PEMC capable UE can indicate to the 3GPP network about such change of a PIN GW. Whether and how the 3GPP network shall control/authorize such change of PIN GW for the specific PIN.

-	How to authorize/de-authorize PIN Element to connect to and from the 5G network via the PEGC.

-	How to authorize/de-authorize PIN Element to access applications and/or service in the PIN

-	How 5GC enforces the authorization result for a PIN.

# * * * * End of changes * * * *