3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200592r03

Elbonia, February 14th – 25th, 2022

Source:	Huawei, HiSilicon

Title:	Key Issue: efficient mobility and service continuity

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR / Rel-18

Abstract: It is proposed to define new key issue for efficient mobility and service continuity for WT#1.

1. Introduction

Based on the discussions in S2-2200591, the WT#1 is proposed to focus on two mobility scenarios (i.e. mobility within the same donor and mobility between different donors) for UEs inside the vehicle, and the corresponding new key issue for efficient mobility and service continuity is defined.

2. Text Proposal

It is proposed to capture the following changes into. TR 23.700-05.

# * * * * First change * * * *(all new text)

5.X	Key Issue #X: Efficient mobility and service continuity when served by VMR

5.X.1	General description

When the moving vehicles are equipped with mobile base station relays, the mobile base station relays can provide 5G coverage and communication to UEs (inside the vehicle and/or in its vicinity), and connected wirelessly to the 5G network via RAN (donor) nodes. When one or a group of UEs are already served by the mobile base station relay, there are two mobility scenarios to be studied as the following:

- Scenario A (mobility within the same donor): When the UEs are continuously served by a mobile base station relay (e.g. inside the vehicle and/or in its vicinity), this mobile base station relay within the vehicle is moving around within a limited geographical area while keeping connecting with the same donor RAN node. In this case, the UE keeps the connection with the same gNB-DU of the mobile base station relay, and there is no change ofthe donor-CU.

- Scenario B (mobility between different donors): When the UEs are continuously served by a mobile base station relay (e.g. inside the vehicle and/or in its vicinity), this mobile base station relay within the vehicle is moving around over a long distance. The mobile base station relay node connects with a different donor RAN node if the vehicle keeps moving. In this case, the UE keeps the connection with the same gNB-DU of the mobile base station relay, but there ischange of the donor-CU.

NOTE 1:	For the above scenarios, whether the cell information in the System Information Broadcast (e.g. Cell ID, TAC) changes has RAN dependency.

NOTE 2:	For the above scenarios, whether UE needs to perform the legacy handover procedures has RAN dependency.

1) Scenario A                              2) Scenario B

Figure 5.X.1-1: Scenarios for efficient mobility and service continuity

The following aspects need to be studied for UEs served by the mobile base station relay in the case of mobility within the same donor-CU and mobility between different donors-CU:

-	Whether and how to enhance current procedures of mobility and service continuity for a UE or a group of UEs, to efficiently deliver the data. Following aspects need to be considered in potential solutions:

-	If the TAC in the System Information Broadcast changes, whether and how to reduce the frequent Mobility Registration Update initiated by UE in case of mobility.

-	If the handover is needed due to cell changes, whether and how to efficiently manage the mobility for group UEs (e.g., during handover).

NOTE 4:	Mechanisms related to mobility management and service continuity have RAN dependency and should align with the progress of RAN WGs.

# * * * * End of changes * * * *