3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200600r05

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

Title:	Key Issue: MBS data reception in RRC Inactive mode

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract: this paper proposes a key issue on MBS data reception in RRC Inactive mode.

1. Introduction

This document proposes key issues on MBS data reception by UEs in CM-CONNETED with RRC Inactive state.

The 5MBS phase 2 SID SP-211645 (approved in SA#94E) contains the following working task:

WT#1. How to enable support for and enhancement related to end-to-end MBS traffic delivery for, including large number of UEs: WT#1.1 Enabling UE's receiving Multicast MBS Session data in RRC states in RRC Inactive state; NOTE 1: RRC Inactive state support requires collaboration with RAN WGs.

WT#1.1 aims to explore the potential enhancement on top of Rel-17 mechanism, that is, rather than rejecting connection requests, NG RAN could move UEs to RRC Inactive state, but those UEs are still able to receive the MBS session data.

For information, the approved RAN WID NR_MBS_enh for Rel-18 (RP-213568) includes the following:

-	"Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3]

-	PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2]

-	Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE.  (Seamless/lossless mobility is not required) [RAN2, RAN3]".

.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-47.

# * * * * First change * * * *

5	Key Issues

5.A	Key Issue #A: Multicast MBS data reception in RRC Inactive state

5.A.1	Description

In order to provide MBS service to more UEs in a cell, NG RAN could enable UEs within an MBS multicast session to receive MBS session data while in CM-CONNECTED with RRC Inactive state.

The following aspect will be studied for multicast:

-	Whether, how and what MBS assistance information to provide from 5GC to RAN for an MBS session allowing UEs in CM-CONNECTED with RRC Inactive state to receive MBS content, including the aspect

which 5GC NF(s) determine the MBS assistance information and how they do so;

-	Whether and how to enhance the current procedures (including mobility related procedures) for MBS session with members in RRC Inactive state.

NOTE 1:	During the study of this key issue, the conclusion needs to consult with RAN WGs..

NOTE 2:	RAN WG will determine how the switching for the UEs belongs to MBS session between CM-CONNECTED state to CM-CONNECTED with RRC Inactive state is performed by the RAN node (and vice versa).

# * * * * End of changes * * * *