3GPP TSG-WG SA2 Meeting #149E e-meeting  	S2-2200645r01

Elbonia, Feb 14 – 25, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

Title:	New key issue for management of 5GS network timing resiliency

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract: This contribution discusses 5G timing resiliency requirement defined by SA1 on work task #1.1 and proposes new key issues for this study.

# 1. Introduction/Discussion

As FS_5TRS_URLLC Work Task #1.1 described,

Study how to report 5GS network timing synchronization status (such as divergence from UTC and 5GS network timing source degradation) to UEs and 3rd party applications (AFs):

- Study how RAN and 5GC learn about 5GS network timing synchronization status to be able to inform UEs and AFs.

- Study if additional information needs to be provided to UEs and AFs to inform about 5GS network timing synchronization status.

The WT targets to support the 5GS timing resiliency service performance management, including reporting, monitoring, service exposure etc., as described in TS22.261. Particularly in the following aspects:

The 5G system shall be able to maintain accurate time synchronization as appropriate for the supported applications in the event of degradation or loss of the primary timing reference (e.g., GNSS).

The 5G system shall be able to support mechanisms to indicate to devices (e.g., UEs, applications) that there is an alternate time source available for use (e.g., 5G system internal holdover capability, atomic clock, Sync over Fiber, TBS, GNSS), taking into account the holdover capability of the devices.

Observation 1: FS_5TRS_URLLC needs to study how to maintain the 5GS network timing capability to UEs and AFs in the event of degradation or loss of the primary timing reference, and how to provide alternative time source available for 5GS network timing synchronization.

The 5G system shall support a mechanism to report timing errors such as divergence from UTC and time sync degradation to UEs and 3rd party applications.

Observation 2: FS_5TRS_URLLC needs to study how to report timing synchronization status to UEs and AFs.

The 5G system shall support a mechanism for a 3rd party application to request resilient timing with specific KPIs (e.g., accuracy, interval, coverage area).

Observation 3: FS_5TRS_URLLC needs to study how to provide 5GS network timing meeting the 5G timing resiliency performance requirements, like KPIs defined in TS 22.261.

# 2. Text Proposal

It is proposed to add the following key issue into the TR 23.700-25 V0.0.0

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

[y]	3GPP TS 22.261: “Service requirements for the 5G system; Stage 1”.

# * * * * Second change * * * *

# 5	Key Issues

## 5.X	Key Issue #X: management of 5GS network timing resiliency

### 5.X.1	Description

To maintain the time synchronization service meeting resiliency performance requirements as defined in TS 22.261 [y] for UEs and AFs in the event of degradation or loss of the primary timing reference.

This key issue aims at addressing the following points:

-	How and what to  report of 5GS network timing synchronization status to enable timing resiliency for 5GS network timing sychronization.

# * * * * Third change * * * *

# * * * * Fourth change * * * *

# * * * * Fifth change * * * *

# * * * * End of changes * * * *