SA WG2 Meeting #149e	S2-2200758r03

# February 14 – 25, 2022; Elbonia               			(revision of S2-220)

Source: 	Lenovo, Motorola Mobility, Nokia, Nokia Shanghai Bell

Title: 	Architectural assumptions for ATSSS_Ph3

Document for: 	Approval

Agenda Item: 	9.3

Work Item / Release:	FS_ATSSS_Ph3 / Rel-18

Abstract of the contribution:

# Proposal

It is proposed to add the following architectural assumptions and requirements into TR 23.700-53.

Start of Changes

# 4	Architectural Assumptions and Requirements

Editor's note: This clause will list general architectural assumptions and principles for this study.

The study in this technical report is based on the following architectural requirements and assumptions:

-	The study takes the Rel-17 ATSSS architecture (see TS 23.501 [2]) as a baseline.

-	It is assumed that an MA PDU Session is required for supporting all ATSSS capabilities. Only ATSSS enhancements using an MAP PDU Session are considered.

-	The study may impact the core network (5GC and/or EPC) but it shall not impact the access network (e.g., NG-RAN, W-5GAN, etc.).

-	All ATSSS steering functionalities (existing and new) reside in the UE and in an UPF. Steering functionalities outside the UPF are not considered.

-	Any new steering functionality shall be based on existing IETF protocols or extensions of such protocols (i.e. MP-QUIC, MP-DCCP).

-	New ATSSS capabilities shall  either able to co-exist with the existing ATSSS capabilities defined in Rel-17 or any dependencies between new and existing capabilities shall be explicitly defined.

-	The study considers, not only ATSSS-capable UEs, but also ATSSS-capable 5G-RGs. The conclusions of the study identify which aspects / solutions are applicable only to ATSSS-capable UEs and which are applicable only to ATSSS-capable 5G-RGs.

NOTE: Solutions related to 5G-RG will be brought to BBF attention for considerations

End of Changes