3GPP TSG-WG SA2 Meeting #149E e-meeting	S2-2200801r09

Elbonia, February 14 – 25, 2022

Source:	China Telecom, vivo, Ericsson, Samsung, Huawei

Title:	FS_eNA_ph3 KI for WT#3.7: NWDAF-assisted URSP

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_ph3 / Rel-18

Abstract of the contribution: Proposal for a new key issue about NWDAF assisted URSP.

Discussion

UE Route Selection Policy (URSP) is used by UE to determine how the traffic of a detected application can route, e.g. associated to an established PDU Session, establishment of a new PDU Session. The URSP may be pre-configured in the UE or may be provisioned to UE from PCF and it cannot be adjustable in time. However, the route selection descriptor of URSP can be variable according to time or location or other factors for the same application. With the analytics from NWDAF, it would help PCF to select the best connection (e.g. 3GPP or non-3GPP Access Type, S-NSSAI value, DNN value…) for the application(s) to get better resource utilization for the network or better experience for specific UE.

This new key issue is to address the WT#3.7: NWDAF-assisted URSP in SP-211650.

Proposal

Proposal: It is proposed to add a new key issue about supporting UE route selection by adjusting URSP based on assistance analytics information from NWDAF.

The following changes are proposed for TR 23.700-81.

* * * * Start of Changes * * * *

5.X	Key issue #X: < NWDAF-assisted URSP >

NWDAF can already provide some network analytics to PCF for policy decisions as described in clause 6.1.1.3 in TS 23.503 [x]. However, further investigation is required on whether and how analytics can be used to assist in the generation of URSP rules.

In this key issue, the following aspects need to be studied:

- Whether any and which components of the URSP rules can benefit from analytics.

- Whether and how existing Analytics IDs, or new Analytics ID(s) can be used to assist in the generation of URSP Rules.

- What procedures trigger the subscription to these Analytics IDs.

- Whether a new (set of) interactions(s) are required between NWDAF and other 5GC NF(s) to assist in the generation of URSP Rules, and how to define the new interactions.

- What information should be collected as input by the NWDAF for these Analytics IDs.

* * * * End of Changes * * * *