SA WG2 Meeting #S2-149e(e-meeting)	S2-2200819r09

14 - 25 February 2022, Electronic Meeting

Source:	vivo, CATT, China Telecom, China Mobile, Inspur, NTT DOCOMO, Samsung, Huawei, Rakuten Mobile, Lenovo Qualcomm? ZTE?

Title:	New KI on enhancement of NWDAF with the benefit from location service

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3 / R18

Abstract: Propose a key issue on the enhancement of NWDAF and what benefit can NWDAF acquire from location service.

# 1. Introduction

According to the SP-211650, the study on Enablers of Network Automation, the Work Task #4.2 needs to be studied:

WT#4.2: NWDAF enhancements considering the finer granularity of location information than TA and cell level

NOTE:    Coordinated activities between the study FS_eLCS_PH3 and this study are needed if NWDAF need to get Location information.

As per the discussion paper, the study of the enhancement of NWDAF with the benefits from location service is required.

# 2. Proposal

This paper proposes to add a new key issue as follows: (all new text).

* Start of change *

## 5.X	Key Issue X: The enhancement of NWDAF with the benefit from location service

### 5.X.1	Description

The NWDAF can retrieve and collect UE location information and then provide some valuable analytics to NWDAF consumers, e.g. UE mobility analytics, QoS Sustainability Analytics as defined in TS 23.288. However, the UE location information that NWDAF can obtain is only TA/cell granularity in R17.

The horizontal accuracy and the vertical accuracy of the existing location service can reach a granularity level finer than TA and cell level. Meanwhile, some extra information (e.g. speed, heading) can also be provided by the location service. But whether these LCS related information are beneficial and sufficient to NWDAF is not clear, and how can NWDAF obtain these LCS related information? Therefore, a KI to study how NWDAF benefits from location service is required.

In this key issue, the following aspects will be studied:

- Identify how an NWDAF determines that location information with finer granularity than cell/TA level is required in output analytics? What finer granularity location information needs to be retrieved as input data? From where?

- Identify which analytics ID(s) are benefit from the input finer granularity location information and how to enhance the existing or define new analytics ID(s) based on the input finer granularity location information

- Identify how NWDAF acquires the finer granularity location information. Whether and how the functionality and services of NWDAF or other NFs are needed to be enhanced.

NOTE: 	Coordinated activities with the eLCS_ph3 study are needed.

NOTE: 	Some examples of UE location different than cell/TA level are described in clause 4.2 and 4.3, TS 22.071 [X]

* Next change *

# References

[X]	3GPP TS 22.071: " Location Services (LCS)".

* End of change *