3GPP TSG-WG SA2 Meeting #149E e-meeting  	S2-2200880

# Elbonia, February 14 – 25, 2022	(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3545 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multi NSACF support for roaming and multi service area |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, China Mobile, China Telecom, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2022-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Which S-NSSAI subject to NSAC at AMF/SMF is configured at the NSACF. For the non-roaming case, this mechanism is possible with a large amout of configuration work. However in the roaming case, this is impossible considering the potential roaming partner and the related different HPLMN S-NSSAI. It is suggested that from the subscription data the AMF/SMF is aware whether the subscribed S-NSSAI is subject to NSAC control. One of the EN is the NSAC is realated to roaming UE, i.e. the NSAC for roaming UE is controlled by the HPLMN. Considering the amount of the AMF/SMF and the impact to the AMF/SMF, it is proposed to let the V-NSACF proxy the request towards H-NSACF. Then from the AMF/SMF view, it always communicate with the V-NSACF. And the V-NSACF may communicate with the H-NSACF if the the interaction with H-NSACF is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add the roaming support for NSAC with HPLMN control. This includes following part: - Subscription based NSAC controlled. This is to avoid too much configuration data at the AMF/SMF. - V-NSACF communicate with the H-NSACF for the NSACF controlled by the HPLMN if the interaction with H-NSACF is needed. - Adding the PLMN ID for the NSACF discovery and seletion. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NSAC controlled by the HPLMN is not supported and unclear how the NF in the VPLMN know that S-NSSAI is subject to NSAC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7, 5.15.6, 5.15.11.0, 5.15.11.3, 6.3.22 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

4.2.7	Reference points

The 5G System Architecture contains the following reference points:

N1:	Reference point between the UE and the AMF.

N2:	Reference point between the (R)AN and the AMF.

N3:	Reference point between the (R)AN and the UPF.

N4:	Reference point between the SMF and the UPF.

N6:	Reference point between the UPF and a Data Network.

N9:	Reference point between two UPFs.

The following reference points show the interactions that exist between the NF services in the NFs. These reference points are realized by corresponding NF service-based interfaces and by specifying the identified consumer and producer NF service as well as their interaction in order to realize a particular system procedure.

N5:	Reference point between the PCF and an AF or TSN AF.

N7:	Reference point between the SMF and the PCF.

N8:	Reference point between the UDM and the AMF.

N10:	Reference point between the UDM and the SMF.

N11:	Reference point between the AMF and the SMF.

N12:	Reference point between AMF and AUSF.

N13:	Reference point between the UDM and Authentication Server function the AUSF.

N14:	Reference point between two AMFs.

N15:	Reference point between the PCF and the AMF in the case of non-roaming scenario, PCF in the visited network and AMF in the case of roaming scenario.

N16:	Reference point between two SMFs, (in roaming case between SMF in the visited network and the SMF in the home network).

N16a:	Reference point between SMF and I-SMF.

N17:	Reference point between AMF and 5G-EIR.

N18:	Reference point between any NF and UDSF.

N19:	Reference point between two PSA UPFs for 5G LAN-type service.

N22:	Reference point between AMF and NSSF.

N23:	Reference point between PCF and NWDAF.

N24:	Reference point between the PCF in the visited network and the PCF in the home network.

N27:	Reference point between NRF in the visited network and the NRF in the home network.

N28:	Reference point between PCF and CHF.

N29:	Reference point between NEF and SMF.

N30:	Reference point between PCF and NEF.

NOTE 1:	The functionality of N28 and N29 and N30 reference points are defined in TS 23.503 [45].

N31:	Reference point between the NSSF in the visited network and the NSSF in the home network.

NOTE 2:	In some cases, a couple of NFs may need to be associated with each other to serve a UE.

N32:	Reference point between a SEPP in one PLMN or SNPN and a SEPP in another PLMN or SNPN; or between a SEPP in a SNPN and a SEPP in a CH/DCS, where the CH/DCS contains a UDM/AUSF.

NOTE 3:	The functionality of N32 reference point is defined in TS 33.501 [29].

N33:	Reference point between NEF and AF.

N34:	Reference point between NSSF and NWDAF.

N35:	Reference point between UDM and UDR.

N36:	Reference point between PCF and UDR.

N37:	Reference point between NEF and UDR.

N38:	Reference point between I-SMFs and between V-SMFs.

N40:	Reference point between SMF and the CHF.

N41:	Reference point between AMF and CHF in HPLMN.

N42:	Reference point between AMF and CHF in VPLMN.

NOTE 4:	The functionality of N40, N41 and N42 reference points are defined in TS 32.240 [41].

N43:	Reference point between PCFs.

NOTE 5:	The functionality of N43 reference point is defined in TS 23.503 [45].

NOTE 6:	The reference points from N44 up to and including N49 are reserved for allocation and definition in TS 32.240 [41].

N50:	Reference point between AMF and the CBCF.

N51:	Reference point between AMF and NEF.

N52:	Reference point between NEF and UDM.

N55:	Reference point between AMF and the UCMF.

N56:	Reference point between NEF and the UCMF.

N57:	Reference point between AF and the UCMF.

NOTE 7:	The Public Warning System functionality of N50 reference point is defined in TS 23.041 [46].

N58:	Reference point between AMF and the NSSAAF.

N59:	Reference point between UDM and the NSSAAF.

N80:	Reference point between AMF and NSACF.

N81:	Reference point between SMF and NSACF.

N82:	Reference point between NSACF and NEF.

N83:	Reference point between AUSF and NSSAAF.

N84:	Reference point between TSCTSF and PCF.

N85:	Reference point between TSCTSF and NEF.

N86:	Reference point between TSCTSF and AF.

N87:	Reference point between TSCTSF and UDR.

NOTE 8:	The reference points from N90 up to and including N95 are reserved for allocation and definition in TS 23.503 [45].

N88:	Reference point between two NSACFs in same PLMN.

N89:	Reference point between two NSACFs in different PLMN.

The reference points to support SMS over NAS are listed in clause 4.4.2.2.

The reference points to support Location Services are listed in TS 23.273 [87].

The reference points to support SBA in IMS (N5, N70 and N71) are described in TS 23.228 [15].

The reference points to support AKMA (N61, N62 and N63) are described in TS 33.535 [124].

The reference points to support 5G ProSe are described in TS 23.304 [128].

The reference points to support 5G multicast-broadcast services are described in TS 23.247 [129].

The reference points to Support Uncrewed Aerial Systems (UAS) connectivity, identification and tracking are described in TS 23.256 [136].

The reference points to support SBA in GBA and GBA push (N65, N66, N67 and N68) are described in TS 33.220 [140] and TS 33.223 [141].

# * * * * Next change * * * *

5.15.6	Network Slicing Support for Roaming

For roaming scenarios:

-	If the UE only uses standard S-NSSAI values, then the same S-NSSAI values can be used in VPLMN as in the HPLMN.

-	If the VPLMN and HPLMN have an SLA to support non-standard S-NSSAI values in the VPLMN, the NSSF of the VPLMN maps the Subscribed S-NSSAIs values to the respective S-NSSAI values to be used in the VPLMN. The S-NSSAI values to be used in the VPLMN are determined by the NSSF of the VPLMN based on the SLA. The NSSF of the VPLMN need not inform the HPLMN of which values are used in the VPLMN.

Depending on operator's policy and the configuration in the AMF, the AMF may decide the S-NSSAI values to be used in the VPLMN and the mapping to the Subscribed S-NSSAIs.

-	The UE constructs Requested NSSAI and provides the mapping of S-NSSAIs of the Requested NSSAI to HPLMN S-NSSAIs if the mapping is stored in the UE, as described in clause 5.15.5.2.1.

-	The NSSF in the VPLMN determines the Allowed NSSAI without interacting with the HPLMN.

-	the HPLMN may provide NSSRG Information as part of the Subscription information as described in clause 5.15.12.

-	The Allowed NSSAI in the Registration Accept includes S-NSSAI values used in the VPLMN. The mapping information described above is also provided to the UE with the Allowed NSSAI as described in clause 5.15.4.

-	If, depending on subscription information, the S-NSSAI values are subject to NSAC,  the AMF or SMF in VPLMN triggers a request for NSAC for these S-NSSAI values as described in clause 5.15.11.3.

-	In PDU Session Establishment procedure, the UE includes both:

(a)	the S-NSSAI that matches the application (that is triggering the PDU Session Request) within the NSSP in the URSP rules or within the UE Local Configuration as defined in clause 6.1.2.2.1 of TS 23.503 [45]; the value of this S NSSAI is used in the HPLMN; and

(b)	an S-NSSAI belonging to the Allowed NSSAI that maps to (a) using the mapping of the Allowed NSSAI to HPLMN S-NSSAIs; the value of this S-NSSAI is used in the VPLMN.

For the home routed case, the V-SMF sends the PDU Session Establishment Request message to the H-SMF along with the S-NSSAI with the value used in the HPLMN (a). If the S-NSSAI values are subject to NSAC, the V-SMF or H-SMF triggers a request for NSAC for these S-NSSAI values as described in clause 5.15.11.3.

-	When a PDU Session is established, the CN provides to the AN the S-NSSAI with the value from the VPLMN corresponding to this PDU Session, as described in clause 5.15.5.3.

-	The Network Slice instance specific network functions in the VPLMN are selected by the VPLMN by using the S-NSSAI with the value used in the VPLMN and querying an NRF that has either been pre-configured, or provided by the NSSF in the VPLMN. The Network Slice specific functions of the HPLMN (if applicable) are selected by the VPLMN by using the related S-NSSAI with the value used in the HPLMN via the support from an appropriate NRF in the HPLMN, identified as specified in clause 4.17.5 of TS 23.502 [3] and, for SMF in clause 4.3.2.2.3.3 of TS 23.502 [3].

# * * * * Next change * * * *

5.15.11.0	General

The Network Slice Admission Control Function (NSACF) monitors and controls the number of registered UEs per network slice and/or the number of PDU Sessions per network slice for the network slices that are subject to Network Slice Admission Control (NSAC). The NSACF is configured with the maximum number of UEs and/or the maximum number of PDU Sessions allowed to be served per S-NSSAI subject to NSAC. The action associated to reaching a maximum value may be to reject a UE registration or to reject a PDU session or an operator may rather configure the NSACF to just generate charging events upon reaching the maximum value. The NSACF is also configured with information indicating the access type(s) for which NSAC applies for the S-NSSAI (i.e. 3GPP Access Type, Non-3GPP Access Type, or both).

The subscription information the AMF or SMF receives from the UDM may include the S-NSSAIs in the subscription information whether NSAC applies.

The NSACF also provides event-based Network Slice status notifications and reports to the consumer NFs (e.g. AF).

The NSACF may be responsible for one or more S-NSSAIs. There may be one or multiple NSACFs deployed in a network as follows:

-	If the network is configured with a single service area, there is a single NSACF configured with the maximum number of UEs per network slice and/or the maximum number of PDU Sessions per network slice, which are valid in the network.

-	If the network is configured with multiple service areas, an NSACF may be deployed on a service area basis, which can be one NSACF instance or one NSACF Set. Each NSACF is configured with the maximum number of UEs per network slice and/or the maximum number of PDU Sessions which are valid in the service area.

If multiple NSACFs are configured in a PLMN for a network slice, a NSACF is configured with a maximum number of UEs and/or a maximum number of PDU Sessions specific for the service area and the Network Slice.

NOTE:	When multiple NSACFs are deployed, how the maximum number of UEs per network slice and the maximum number of PDU Sessions per network slice is distributed among multiple NSACFs, is implementation specific.

To ensure that a single counter applies across service areas for the number of UEs per Network Slice and/or the number of PDU Sessions per network slice for a S-NSSAI, an operator may designate one NSACF to act as primary NSACF across all service areas for a S-NSSAI and the other NSACFs act as proxy NSACF towards this NSACF for the S-NSSAI. In this case the configuration of the maximum values allowed for this S-NSSAI is only in the primary NSACF. The proxy NSACF manage the NSAC at the related service area, e.g. one VPLMN, i.e. V-NSACF, or one defined service area within one PLMN. For one service area there are only one proxy NSACF. To improve the signalling efficiency when the proxy NSACF proxies the NSAC request towards the primary NSACF, the primary NSACF may allocate a local  maximum values and piggyback it as the response to the NSAC request. If that, until the local maximum value is expired, the proxy NSACF is not required to proxy the NSAC request to the primary NSACF.

Subject to operator policy and national/regional regulations, network slices which are used for Emergency service or for Critical and Priority services (e.g. MCS, MPS) may be exempted from NSAC.

When the AMF receives a Registration Request for an Emergency Registration or with a Registration Request with an Establishment Cause indicating a priority services (e.g. MPS, MCS), the AMF may accept the registration request without applying NSAC, i.e. the AMF triggers NSAC procedure, but the response from the NSACF is ignored at the AMF.

When the SMF receives a PDU Session Establishment Request for an emergency PDU Session or a PDU Session Establishment Request with a priority header, the SMF may accept the PDU Session Establishment Request without applying NSAC, i.e., the SMF triggers NSAC procedure, but the response from the NSACF is ignored at the SMF.

Alternatively, when NSAC is exempted for the S-NSSAI, the AMF and the SMF skip the corresponding NSAC procedure for the S-NSSAI, i.e. this UE (respectively PDU Session) is not counted towards the maximum number of UEs (respectively PDU Sessions).

The support of NSAC for the S-NSSAI used for SNPN Onboarding as described in clause 5.30.2.10 is optional and subject to Onboarding Network operator policies.

If the S-NSSAI used for SNPN Onboarding or Emergency services is rejected due to NSAC, the UE associates the rejected S-NSSAI for NSAC to SNPN Onboarding or to Emergency services as described in TS 24.501 [47].

# * * * * Next change * * * *

5.15.11.3	Network Slice Admission Control for Roaming

In the case of roaming, depending on operator's policy, a roaming agreement or an SLA between the VPLMN and the HPLMN, and for S-NSSAIs subject to NSAC in the subscription data, NSAC for roaming UEs can be performed by the VPLMN or HPLMN. The following principles apply:

For NSAC of roaming UEs for maximum number of UEs per network slice and/or maximum number of PDU Sessions per network slice, the following principles shall be used:

-	When the AMF or SMF retrieves the subscription data from the UDM, the subscription data includes the information on whether the subscribed S-NSSAI is subject to NSAC.

.

For NSAC of roaming UEs for maximum number of UEs per network slice, the following principles shall be used:

-	For maximum number of UEs, if the admission controlled is managed by the VPLMN, a NSACF in the VPLMN is configured with the maximum number of allowed roaming UEs per mapped S-NSSAI in the HPLMN for each S-NSSAI in the HPLMN that is subject to NSAC.

-	The admission control is either managed by the VPLMN, or HPLMN which is based on proxying the NSAC interactions of the V-NSACF with the H-NSACF for the HPLMN S-NSSAI.

-	The AMF triggers a request to a V-NSACF in the VPLMN, which handles the PLMN ID and S-NSSAI of the HPLMN.

-	For maximum number of UEs, if the admission controlled is managed by the HPLMN,  the V-NSACF performs the proxy NSAC request towards the H-NSACF of the HPLMN S-NSSAI. The H-NSACF may take into account the VPLMN PLMN ID for NSAC decisions.

-	For maximum number of UEs, if the admission controlled is managed by the VPLMN, the V-NSACF  locally enforce a configured maximum number of UEs for the HPLMN NSSAI, rather than using the proxy-based method..

For network slice admission control of roaming UEs for maximum number of PDU Sessions per network slice, the following principles shall be used:

-	For Home Routed PDU sessions, the SMF in the HPLMN interacts with the NSACF for the S-NSSAI of the PDU session.

-	For LBO PDU sessions, if the admission controlled is managed by the VPLMN, a NSACF in the VPLMN is configured with the maximum number of allowed PDU Sessions in LBO mode per mapped S-NSSAI in the HPLMN for each S-NSSAI in the HPLMN that is subject to NSAC.

-	For LBO PDU sessions, the admission control is either managed by the VPLMN, or HPLMN which is based on proxying the NSAC interactions of the V-NSACF with the H-NSACF for the HPLMN S-NSSAI.

-	For LBO PDU sessions, the SMF triggers a request to a V-NSACF in the VPLMN, which handles the PLMN ID and S-NSSAI of the HPLMN,

-	 If the admission control is managed by the HPLMN, the V-NSACF performs the proxy NSAC request towards the H-NSACF of the HPLMN S-NSSAI. The H-NSACF may take into account the VPLMN PLMN ID for NSAC decisions. In this case, the LBO and Home Routed PDU sessions for the same HPLMN S-NSSAI are subject to NSAC by the same HPLMN NSACF.

-	 If the admission control is managed by the HPLMN, the V-NSACF locally enforce a configured maximum number of LBO PDU sessions for the HPLMN S-NSSAI rather than using the proxy-based method.

.

.

‐	The AMF or SMF (in LBO roaming case) in the VPLMN provides both the S-NSSAI in the VPLMN and the corresponding mapped S-NSSAI in the HPLMN to the NSACF in the VPLMN. The NSACF in the VPLMN performs NSAC for both S-NSSAI in the VPLMN (if the S-NSSAI in the VLMN is also subject to NSAC) and the corresponding mapped S-NSSAI in the HPLMN based on the SLA between the VPLMN and the HPLMN. The NSACF in the VPLMN provide the mapped S-NSSAI in the HPLMN to the NSACF in HPLMN if the interaction between them is needed.

# * * * * Next change * * * *

6.3.22	NSACF discovery and selection

The NF consumers shall utilise the NRF to discover NSACF instance(s) unless NSACF information is available by other means, e.g. locally configured in NF consumers.

If the NSACF NF consumer is the AMF, the NSACF selection function in the AMF selects an NSACF instance based on the available NSACF instances, which are obtained from the NRF or locally configured in the AMF.

The following factors may be considered by the NF consumer for NSACF selection:

-	S-NSSAI(s).

-	HPLMN ID.

-	NSACF Serving Area information. The NSACF service area is related to the location of the NF consumer.

NOTE:	Each Serving Area is unique and unambiguously identified.

-	NSACF service capabilities:

-	Support monitoring and controlling the number of registered UEs per network slice for the network slice that is subject to NSAC.

-	Support monitoring and controlling the number of established PDU Sessions per network slice for the network slice that is subject to NSAC.

In the case of delegated discovery and selection in SCP, the NSACF NF consumer shall send all available factors to the SCP.

# * * * * End of changes * * * *