3GPP TSG-WG SA2 Meeting #149E e-meeting  	S2-2200882

# Elbonia, Feb 14 – 25, 2022	(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0090 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of MBS data forwarding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN3#114E meeting, regarding “22.3. Mobility with Service Continuity for UEs in RRC_CONNECTED State” , the following agreements were achieved: - “22.3.1. Mobility Between MBS Supporting Nodes” To support PDCP SN sync, support alt 2 (PDCP SN Sync for a common CU-UP) in Rel-17. To support PDCP SN sync, support alt 1 (PDCP SN Sync among RAN nodes with different CU-UP) in Rel-17. - “22.3.2. Mobility Between MBS Supporting and non-MBS Supporting Nodes” For when to stop data forwarding, agree to eliminate control plane solutions and continue working on user plane solutions. - “22.3.2. Mobility Between non-MBS Supporting and MBS Supporting Nodes”, the following agreements were achieved: Agree to continue working on solutions avoiding duplicates during the switch from DRB to MRB. Base on RAN3#114E meeting agreement, in RAN3#114E-bis meeting, further progresss on topic 22.3 includes: - For topic “22.3.2. Mobility Between MBS Supporting and non-MBS Supporting Nodes”. Based on the discussion in the summary of offline discussion R3-221079, majority companies support the following bullets: - The core network provides UE individual end marker in the UE associated NG-U tunnels to the source gNB. And then the source gNB forwards the end marker to the target gNB. - The UPF sends this UE-individual End Marker packet with the mapped QFI i.e. the QFI of the unicast QoS flow associated with the MBS flow. The gNB can identify from the received mapped QFI what is the corresponding MBS flow for which data forwarding should stop. - For topic “22.3.2. Mobility Between non-MBS Supporting and MBS Supporting Nodes”, further agreements include: RAN3 decided to minimize data loss and agreed on the solution to eliminate duplicates via using the same Core Network Sequence Numbers over both the unicast N3 tunnel and shared N3 tunnel for the multicast related handover from non-MBS supporting gNB to MBS supporting gNB. As per the above-mentioned progress in RAN3, it is the suitable time for SA2 to coordinate with RAN3 to specify the detail for MBS data forwarding. Therefore, it is proposed to specify the detail of MBS data forwarding in 23.247. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Coordinate with RAN3 progres, in clause 7.2.3.5, add some detail descriptions to further clarify how to stop MBS data forwarding and how to minize data loss. - In clause 7.2.3.2 and 7.2.3.3, the detail of how to perform the MBS data forwading, add reference to clause 7.2.3.5. Also, remove the “Editor’s Note Details on data forwarding, if applicable, needs to wait for RAN WGs” . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The detail of how to perform data forwading is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.2, 7.2.3.3, 7.2.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 7.2.3.2	Xn based handover from MBS supporting NG-RAN node

This clause describes an Xn based handover with MBS traffic delivered to the UE at the source NG-RAN node supporting MBS.

Figure 7.2.3.2-1: Xn based handover with MBS Session

The following additions apply compared to clause 4.9.1.2 of TS 23.502 [6]:

Before Handover:

The source NG RAN has been provided with MBS Session Resource information (including the MBS Session ID and multicast QoS flow information) and the UE Context information contains a mapping information within the PDU Session Resource associated with the MBS Session Resource, e.g. including mapped unicast QoS Flows associated with the multicast QoS flow(s) of the MBS Session Resource.

Handover Preparation Phase:

At Xn handover, the target NG-RAN is provided with MBS session information by the source NG-RAN which causes:

-	an MBS non-supporting target NG-RAN node to prepare the unicast resources according to associated QoS flow(s) information.

‐	an MBS supporting target NG-RAN node to allocate to the UE shared NG-RAN resources according to the MBS session information. If the 5GC Shared MBS traffic delivery for the indicated multicast MBS Session has not been established in target NG-RAN, target NG-RAN triggers setup of the resources for the 5GC Shared MBS traffic delivery, see clause 7.2.1.4 for details.

1.	Target NG-RAN to AMF: the target NG-RAN sends N2 Path Switch Request to AMF.

The target NG-RAN node, if MBS-capable, indicates it supports of MBS to SMF in N2 SM information. Per the received N2 SM information, the SMF knows whether the target NG-RAN node supports MBS and determines the delivery method, i.e., whether the 5GC Shared MBS traffic delivery or 5GC Individual MBS traffic delivery is used for MBS data transferring.

The SMF differentiates two cases:

Case A) The target NG-RAN supports MBS. Step 3 applies and step 4 is skipped.

3.	SMF to UPF: The SMF invokes N4 Session Modification procedure with the UPF (PSA) only for unicast PDU Session.

Case B) The target NG-RAN does not support MBS. Step 3 is skipped, step 4 applies.

4.	This steps is same as described in step 11 of clause 7.2.1.3.

The details of how to perform data forwarding refers to clause 7.2.3.5.

5.	SMF to AMF: The SMF responds to AMF through Nsmf_PDUSession_UpdateSMContext response.

6.	AMF to target NG-RAN: The AMF sends the path switch Ack to target NG-RAN.

#### 7.2.3.3	N2 based handover from MBS supporting NG-RAN node

This clause describes the N2 based handover with MBS traffic delivered to the UE at the source NG-RAN node supporting MBS.

Figure 7.2.3.3-1: N2 based handover with MBS Session

The following additions apply compared to clause 4.9.1.3 of TS 23.502 [6]):

1.	Source NG-RAN to S-AMF: Handover Required (RAN container (MBS Session information, associated PDU session information, associated QoS flow information and corresponding multicast QoS flow information)).

2.	S-AMF to T-AMF: The T-AMF is provided with associated PDU Session information and the MBS session related information.

4.	T-AMF to Target NG-RAN: The Target NG-RAN prepares the radio resource based on the received information:

-	If the Target NG-RAN does not support MBS, the MBS Session related information is not used. The Target NG-RAN uses the associated PDU Session information to allocate resource to deliver MBS data. The MBS data are transmitted via the associated QoS flows within the associated PDU Session.

-	If the Target NG-RAN supports MBS, the Target NG-RAN uses the multicast MBS Session related information to allocate RAN resources to deliver the MBS data. If 5GC Shared MBS traffic delivery for the indicated multicast MBS session has not been established towards the Target NG-RAN, the Target NG-RAN initiates the shared delivery establishment towards the MB-SMF via AMF as described in clause 7.2.1.4.

5.	Target NG-RAN to T-AMF: The target NG-RAN sends Handover Request Ack to T-AMF.

The target NG-RAN node, if MBS-capable, indicates it supports MBS to SMF in N2 SM information. Per the received N2 SM information, the SMF knows whether the target NG-RAN node supports MBS and determines the delivery method, i.e. whether the 5GC Shared MBS traffic delivery or 5GC Individual MBS traffic delivery is used for MBS data transferring.

12.	T-AMF to SMF: The AMF invokes Nsmf_PDUSession_UpdateSMContext request towards SMF, the message includes the received N2 SM information received from the target NG-RAN.

13-14.	Same as described in steps 3-4 of clause 7.2.3.2.

The details of how to perform data forwarding, refers to clause 7.2.3.5.

15.	SMF to T-AMF: The SMF sends the Nsmf_PDUSession_UpdateSMContext Request to the T-AMF.

# * * * * Second change * * * *

#### 7.2.3.5	Minimization of data loss

To minimize the data loss of the UE during the handover procedure the following functions apply:

-

For each MBS QoS flow, the MB-UPF adds a sequence number in each data packet of the MBS session sent by the MB-UPF and forwarded to all related NG-RAN nodes and UPFs via GTP-U tunnel.

Editor's note:	Details on data forwarding between RAN nodes, if applicable, and possible additional methods for minimizing data loss needs to wait for RAN WGs.

-	If source NG-RAN supports MBS and target NG-RAN does not support MBS, the multicast MBS session data is forwarded from source NG-RAN node to target NG-RAN node via the associated QoS flow within the associated PDU session.

To stop the MBS data forwarding, when the individual delivery is enabled, the SMF instructs the UPF to construct an end marker with the associated QoS flow QFI(s) and the MBS data packet sequence number, and to send the end marker packet to the source NG-RAN via the associated PDU session tunnel.  Based on the received end marker packet and the associated PDU session, the source NG-RAN determines the corresponding MBS QoS flow for which data forwarding can be stopped.

-	If source NG-RAN does not support MBS and target NG-RAN supports MBS, for Xn/N2 handover, the multicast MBS session data is forwarded to the target NG-RAN node as the data forwarding mechanism defined in TS 23.502 [6]. Directly after the handover the target NG-RAN node thus still receives MBS session data via individual delivery. The UPF forwards multicast MBS session data within the associated PDU session including the sequence number received from the MB-UPF to the target NG-RAN node. Shared delivery of MBS data towards the target RAN node is being established as described in Clause 7.2.3.4, and the target RAN node receives sequence numbers as part of the MBS data with sequence numbers via shared delivery.

NOTE:	Usage of the sequence number in RAN nodes is defined by RAN WGs. For N2 handover, the detail mechanism of how to transmit the received forwarded multicast MBS session data to UE is defined by RAN WGs.

# * * * * End of changes * * * *