3GPP TSG-SA WG2 Meeting #149E e-meeting 	S2-2200886

Elbonia, February 14 – 25, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	New Key issue for WTI#7: support of NSAC involved multi NSACF

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: This document adds the key issue for WT#7.

- Discussion

One of the ENS3 WT#7 is to support the multi NSACF, i.e.

“7. Study whether and how to enhance the support of NSAC when more than one NSACF is involved in enforcing a shared maximum allowed number of the UEs or PDU Sessions for a network slice in one PLMN or in roaming, in order to avoid fragmentation of the shared maximum allowed number.”

This paper add the related key issue description.

- Proposal

It is proposed to capture the following changes vs. TS 23.700-41.

PS, there are ongoing CR discussed in Rel-17 under the eNS2 WID. If this issue is resolved at Rel-17, the solution to this key issue is not needed at Rel-18.

FIRST CHANGE

5.X	Key Issue #X: Support of NSAC involved multi Service Area

5.X.1	Description

It is possible for one S-NSSAI the NSAC need involve multi NSACF. The potential case includes at least following case:

- Multi NSACF deployed within one PLMN: For NSACF deployment more than one service area are defined within one PLMN. For each service area one NSACF or NSACF set is selected for slice admission control. This include the control of the maximum allowed number of UE or PDU session.

- Roaming: when one user resides at the visit PLMN, the NSAC (Maximum PDU session) may be controlled by the V-NSACF (e.g. for LBO PDU session), or the H-NSACF (e.g. for HR PDU session).

- EPS interworking: when the user establish a HR PDN connection at the EPS network and move to 5GS later, the NSACF(Maximum UE number) selected by the SMF+PGW-C and AMF may be different.

To support NSAC executed in more than one NSACF, one mechanism is to distribute the global maximum allowed number among different NSACFs. However there are some problem on that mechanism, e.g. if the UE move across different service area, the session continuity is not guaranteed.

So for one S-NSSAI and multi NSACF is involved as mentioned above, how to enhance the NSAC to keep the maximum allowed number control but use the shared maximum allowed number more efficiently, especially considering the following case:

- How to enhance the initial registration of a UE with the indicated S-NSSAI considering the efficient usage of the global maximum allowed number of UE ?

- How to enhance the new PDU session establishment with the indicated S-NSSAI considering the efficient usage of the global maximum allowed number of PDU session?

- How to support session continuity when UE move across different service area considering the efficient usage of the global maximum allowed number of UE?

End of CHANGE