3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200910r17

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, KDDI, CATT, Ericsson? Futurewei? Intel, InterDigital, Lenovo? Motorola Mobility? Nokia? Nokia Shanghai Bell? OPPO, Samsung? Tencent Tencent Cloud vivo

Title:	New Key Issue: Media unit integrated packet handling

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract: Propose a new key issue to study QoS enhancement for media unit integrated treatment for WT #3.2

# 1. Introduction/Discussion

This contribution proposes a key issue to study the required QoS enhancement for media unit granularity treatment in 5G network as stated in WT#3.2:

WT#3.2:  Enhance QoS framework to support media units granularity (e.g., video/audio frame/tile, Application Data Unit, control information), where media units consist of PDUs that have the same QoS requirements.

Current 5G network provides QoS support in a QoS Flow level granularity. Each packet in a QoS flow are given the same QoS requirements, e.g. packet delay budget, packet error rate, etc. Within the QoS Flow, each packet is handled individually to fulfil the policy of the QoS Flow.

For XR/media services, a group of packets are used to carry payloads of specific media unit (e.g. frame, video slice).

In media layer, such a media unit are decoded/handled as a whole. For example, the frame/video slice may only be decoded in case all of the packets carrying the frame/video slice are successfully delivered. And a frame within a GOP can only be decoded by the client in case all referenced frames are successfully received. Hence the group of packets have inherent dependency on each other in media layer. Without considering such dependencies between the group of packets, 5GS may perform a scheduling with low efficiency. For example, the 5GS may randomly drop a packet but try to deliver other packets of the same frame/video slice which is useless to the client and thus waste radio resources.

Taking “frame” as an example, as shown in the figure below:

Time (ms)

Burst bitrate

Frame Interval (1/FR)

Burst length

Figure1： Example of downlink traffic for cloud XR

The characteristic of a group of packets for a frame includes:

- Depends on the size of frame, one XR media frame are delivered via different number of packets, e.g. ~400 IP packets for one media frame of 600KB.

- The group of packets of a frame are transferred as the periodic bursts, and the interval period between two frames are 1/frame rate, e.g. the interval is 16.7ms for a video of 60FPS.

- To ensure the MTP (Motion-To-Photon) requirements, the packets group of one media frame needs to be transferred to UE within the specified time budget, e.g. 9ms in order to further media decoding and presentation pressing at UE side. To ensure correctly decoding the media and user experience, the packet loss rate for the packet group of one media frame also has requirements, e.g. 99.9% reliability for one packet group.

If such dependency between packets of a media unit (e.g. frame, video slice,) can be considered, it’s possible to enhance the efficiency and promote user experience.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60.

# * * * * First change (all new texts) * * * *

## 5.x	Key Issue #x - Media Unit integrated packet handling

### 5.x.1	Description

The current 5G system provides QoS support in a QoS Flow level granularity. Each packet in a QoS flow is treated according to the same QoS requirements, e.g. packet delay budget, packet error rate, etc.

For XR/media services, a group of packets are used to carry payloads of specific a media unit (e.g. frame, video slice/tile).

In media layer, such a media unit is decoded/handled as a whole. For example, the frame/video slice may only be decoded in case all of the packets carrying the frame/video slice are successfully delivered. A frame within a GOP (Group of Pictures) can only be decoded by the client in case all referenced frames are successfully received. Hence the group of packets have inherent dependency on each other in media layer. Without considering such dependencies between the group of packets, 5GS may perform a scheduling with low efficiency. For example, the 5GS may randomly drop a packet but try to deliver other packets of the same frame/video slice which is useless to the client and thus waste radio resources.

Also audio samples, haptics applications or remote control operations may benefit if the 5GS considers the Media Unit characteristics.

If such dependency between packets of a media unit (e.g. frame, video slice) can be considered, it’s possible to enhance the efficiency and promote user experience.

This key issue proposes to study media unit integrated packet handling in 5G network, in which the group of packets belongs to a same media unit will be handled as a whole. The key issue includes the following aspects:

-	Which types of media unit shall be supported;

NOTE: Coordination with SA WG4 may be needed.

-	What information should be provided to the 5GS for media unit integrated packet handling, and how to provide;

-	How the 5GS identifies that a PDU belongs to a specific media unit., including the case where the PDUs belonging to the same media unit are encrypted or not;

NOTE: It is assumed that some header information necessary for the identification of PDUs is not encrypted.

-	Whether and how to enhance the QoS model and policy control for media unit integrated packet handling;

-	Whether and how to perform the media unit integrated packet handling in the UE, and/or UPF;

-	What information needs to be provided to the RAN and/or UPF to support media unit integrated packet handling in both the downlink and uplink direction.

-	Whether and how information for media unit integrated packet handling is provided to the UE.

NOTE: Coordination with RAN WGs may be needed.

# * * * * End of changes * * * *