3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200931r04

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, Interdigital Inc., ZTE, vivo, Ericsson

Title:	KI proposal for multi-path transmission using U2N Relay

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract: Addition of a new key issue to support multi-path transmission using U2N Relay.

# 1. Introduction/Discussion

The multi-path transmission using only one direct network communication path and only one indirect network communication path with UE-to-Network Relay can be used to improve the reliability or data rates of Remote UE’s data.

This contribution introduces the key issue for WT#8 in the SID.

WT#8.	Support of multi-path transmission using only one direct network communication path and only one indirect network communication path for UE-to-Network Relay for improved reliability or data rates.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-33.

# * * * * First change * * * *

## 5.X	Key Issue #X: Support of multi-path transmission for UE-to-Network Relay

### 5.X.1	General description

Multi-path transmission using only one direct network communication path and only one indirect network communication path with UE-to-Network Relay can be used to improve reliability or data rates for the remote UE. As illustrated in figure 5.X.1-1, a UE can use path #1 and path #2 for multi-path transmission, where path #1 is direct network communication path, and path #2 is indirect network communication path with UE-to-Network Relay.

Figure 5.X.1-1: Example scenario of multi-path transmission using UE-to-Network Relay

At least the following aspects needs to be studied in potential solutions:

-	How to authorize and trigger the connection establishment for multi-path transmission, including:

-	Whether and how to authorise a Remote UE to use the multi-path transmission for specific service(s) or QoS flow(s).

-	What information will be required forthe Remote UE or the network to trigger the multi-path connection establishment .

-	How to provide/update the rules for multiple-path transmission, e.g. duplication, splitting, aggregation, etc.

-	How to establish/modify/release the connection for multi-path transmission, at least including:

-	The procedure for multi-path establishment/modification/release.

NOTE 1:	Coordination with RAN WGs is needed for RAN dependency.

NOTE 2:	For security aspects, coordination with SA WG3 is needed.

NOTE 3: This KI covers both Layer-2 and Layer-3 UE-to-Network Relay cases.

# * * * * End of changes * * * *