SA WG2 Meeting #149E (e-meeting)	S2-2200992rev1

February 14 – 25, 2022, Elbonia	(revision of S2-220xxxx)

Source:	Nokia, Nokia Shanghai Bell.

Title:	New key issue: KI for objective #4 –Improved Network control of the UE behaviour control

Document for:	Approval

Agenda Item:	9.14 Study on Network Slicing Phase 3

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract of the contribution: This paper proposes KI for objective #4

Discussion

This paper proposes Key Issue for FS_eNS_Ph3 objective #4.

4.	Study whether and how to enhance the system to ensure network controlled behaviour of network slice usage including UE registration and PDU Session establishment (e.g. so that when performing NSAC the network slice can serve UEs/PDU Sessions with actual activity).

Proposal

It is proposed to agree the following changes into TR 23.700-41 on FS_eNS_Ph3.

* * * * Start of 1st Change (All new text) * * * *

5.X	Key Issue #X: Improved network control of the UE behaviour

5.X.1	Description

In the 5GS specifications up to rel-17, a UE Registers/Deregisters with a Network Slice and establishes/tears down PDU sessions based on its own policy taking into account network provided information such as the URSPs. However, this does not allow an operator e.g. to enforce that the UE only registers with a S-NSSAI when it is actually needed because an application is actively using it. A UE may in fact choose to register with all the Configured NSSAIs and then use the URSP just to decide with DNNs to connect to at run time. Also, it is not clear whether a UE can establish connectivity with a DNN based on own logic and URSPs at any time even ahead of usage.

Not all PDU Sessions using the same S-NSSAI are identical in terms of traffic as. For example some of them do not carry any traffic, the number of QoS flows they carry or amount of resources they consume differs. One potential reason could be that some UEs may act as relay or in particular an Industrial Gateway device. Also, different PDU sessions are likely not consuming the same or comparable amount of resources in case the same S-NSSAI can accommodate different traffic types (e.g., GBR and non-GBR) or addressing different services/DNNs. Hence, the counter representing the number of established PDU Session would not carry enough resource usage information.

Operators currently do not have the ability to request the UE to register or establish PDU Session with network slices based on e.g. actual usage (e.g. when NSAC is applied on the number of UEs, the operators may want the UE to deregister from the slice subject to NSAC and register with it based on actual usage.)

There is currently no standardized means for the operator to remove UE from a network slice or tear down of a PDU session based on e.g. actual usage.

This Key Issue will study following aspects:

- How to measure an activity of a UE and/or actual usage of a PDU Session that uses a given S-NSSAI so that consideration of resource usage is taken in account as part of the counting, e.g. considering UEs that have generated actual traffic while in active mode?

- How the network can control the UE and ensure the proper utilization of Slices in the system taking into account e.g. the above measurement result aspects.

* * * * End of Changes * * * *