3GPP TSG-SA WG2 Meeting #149E (e-meeting)	S2-2201026r03

# 14 – 25 February, 2022, Electronic meeting		                  (revision of S2-220xxxx)

Title:	[Draft] Response LS on NAS PDU delivery during PDU Session modification procedure

Response to:	S2-2200423 (R3-221475) LS on NAS PDU delivery during PDU Session modification procedure

Release:	Rel-16

Work Item:	TEI16

Source:	SA2

To:	RAN3

Cc:

Contact Person:

Name:                   Yunjing Hou

E-mail Address:   houyunjing (at) catt (dot) cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	Agreed version of S2-2201025

1. Overall Description:

SA2 thanks RAN3 for their LS on NAS PDU delivery during PDU Session modification procedure. SA2 discussed the option 1 and option 2, agreed the CR in the attachment to adopt the option 2.

Regarding the different understanding by RAN3, SA2 would like to provide following feedback:

- Understanding 1: The SMF would send PDU Session Modification Reject message to UE if the SMF found that the UE requested modification on the PDU session is rejected by NG-RAN node no matter other modifications e.g. TNL address update included in the same N2 message succeeds or not. - Understanding 2: The SMF would send PDU session Modification Reject message to UE only when all of the modification request included in the N2 message failed, i.e. upon receiving the PDU Session Resource Modify Unsuccessful Transfer IE in the PDU SESSION RESOURCE MODIFY RESPONSE message.

SA2 Response

SA2 confirms that understanding 2 is correct under the assumption that NG-RAN node does not deliver the NAS-PDU to the UE.

2. Actions:

To RAN3:

ACTION: 	SA2 kindly asks RAN3 to take the above information into account.

3. Date of Next TSG-SA2 Meetings:

TSG-SA2 Meeting #150-e		06 - 12 April 2022			Electronic Meeting