3GPP TSG-WG SA2 Meeting #149E e-meeting	S2-2201030

e-Meeting, 14th-25th Feb, 2022

# Title:	[Draft] Reply LS on Slice list and priority information for cell reselection

# Response to:	LS S2-2108996/ R2-2111310 on Slice list and priority information for cell reselection from RAN 2

# Release:	Rel-17

# Work Item:	NR_Slice-Core

Source:	SA2

To:	RAN2, CT1, RAN3

Cc:

Contact Person:

Name:	Zhu Jinguo

E-mail Address:	zhu dot Jinguo at ZTE dot com dot cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS23.501 CR 3482, CR331 and CR3539; TS23.502 CR 3334, CR3194 and CR3300

1. Overall Description:

SA2 would like to thank RAN2 for their reply on Slice list and priority information for cell reselection.

SA2 has discussed the granularities of the slice group. If a per TA granularity is to be enabled SA2 could not agree whether to keep the granularity consistent for the Registration Area or whether to separately per slice group indicate which Tracking Areas (TAs) the slice group is associated to. The per TA option was argued to be beneficial in case the meaning of the slice group would frequently change between TAs. SA2 could not reach consensus, and SA2 assumes that there would be a need to fallback to a per PLMN granularity in case there is no consensus. SA2 would like to ask.

Q1: whether RAN2 has any preferences on the options mentioned above.

With regards to the prioritization between network slices for cell reselection. There were different opinions on whether to keep the prioritization completely proprietary in the UE or whether to require some specific logic. The following was proposed as a compromise; if the UE prioritizes network slices that are either not registered (i.e. not in Allowed NSSAI) or without any established PDU Session then the UE, after a possible cell reselection, directly register the network slice or establish a PDU Session, respectively.

SA2 would like to ask.

Q2: if there are any concerns from RAN2 or CT1 with above proposed UE logic.

Furthermore, SA2 has agreed that one network slice can be associated to one or multiple network slice groups.

Please see the attached technically endorsed CRs describing the different options as mentioned above.

SA2 assumes that appropriate CRs will be sent to TSG SA as company CRs based on the feedback.

2. Actions:

To: RAN2, RAN3 and CT1

ACTION:

SA2 asks kindly RAN2, RAN3 and CT1 to take the above information into consideration and for RAN2 and CT1 to answer the above questions.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #150E                               April 4 – 8, 2022