3GPP TSG-WG SA2 Meeting #149e 	S2-2201045r01

# E-meeting, February 14-25 2022	(revision of S2-21XXXXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0509 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on service operation to retreive the number of UE and number of PDU session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2022-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The service name by NSACF to help NWDAF collects the number of UEs registered with a network slice or the number of PDU Sessions established on a network slice is not aligned with the NSACF services as defined in clause 5.2.21 of TS 23.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct service operation name to retreive the number of UE or number of PDU session of the network slicing |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NWDAF can’t retreive the number of UE or number of PDU session of the network slicing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.3.2A	Input data

The detailed information collected by the NWDAF is listed in Table 6.3.2A-1 and Table 6.3.2A-2.

Table 6.3.2A-1: OAM Input data for slice load analytics

| Information | Source | Description |
| --- | --- | --- |
| UE registered in a Network Slice/Network Slice instance | OAM | Mean number of UEs registered in a NW slice or NW slice instance as defined in TS 28.552 [8]. (NOTE 1). |
| PDU Session established on a Network Slice/Network Slice instance | OAM | Mean number of established PDU Sessions in a NW slice or NW slice instance as defined in TS 28.552 [8]. (NOTE 1). |
| Load of NFs associated to Network Slice instance | OAM | Resource utilization information of a Network Slice instance obtained from its constituent NF instances. NF instance load input data collection is described in clause 6.5, Table 6.5.2-1. |
| NOTE 1: 5GC performance measurements can be provided per S-NSSAI by OAM as defined in TS 28.552 [8]. Any 5GC performance measurements per NSI ID required further coordination with SA WG5. |  |  |

Table 6.3.2A-2: 5GC NF Input data for slice load analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamps | 5GC NF | A time stamp associated with the collected information. |
| UE registers/de-registers to a Network Slice/Network Slice instance | AMF(s) | AMF reports that a UE registered or deregistered to a S-NSSAI or to a S-NSSAI and NSI ID. |
| Number of UEs served by the AMF | AMF(s) | AMF reports the total number of UEs served by the AMF per S-NSSAI or per S-NSSAI and NSI ID. (NOTE 1) |
| PDU Session established/released on a Network Slice | SMF(s) | SMF reports that a PDU Session is established or released per S-NSSAI or per S-NSSAI and NSI ID. |
| Current number of UEs registered in a NW slice | NSACF | NSACF reports the number of UE registered at the S-NSSAI. |
| Current number of PDU Sessions established in a NW slice | NSACF | NSACF reports the number of PDU Sessions established at the S-NSSAI. |
| Load of NFs associated to Network Slice instance | NRF | Resource utilization information of a Network Slice instance obtained from its constituent NF instances. NF instance load input data collection is described in clause 6.5, Table 6.5.2-1. |
| NOTE 1: AMF reports the total number of registered UE in the AMF at each associated time stamp. NOTE 2: SMF reports multiple PDU Sessions when establishment or release happened at the same time, indicated by the time stamp. NOTE 3: Based on the internal logic, the NWDAF determines the source for the data collection. |  |  |

NWDAF collects input data on the number of UEs registered in a S-NSSAI or S-NSSAI and NSI ID combination using one of the following options:

-	Total number of UE registered to a S-NSSAI or to a S-NSSAI and NSI ID from each AMF(s) and/or from  NSACF:

-	Namf_EventExposure_Subscribe (Target for Event Reporting = "any UE", Event ID = "Number of UEs served by the AMF and located in "Area of Interest"", Event Filter information = S-NSSAI(s) or one or more of the tuple (S-NSSAI, NSI ID), Event reporting mode = periodic along with periodicity) as defined in clause 5.2.2.3.1 of TS 23.502 [3]; or

-	Nnsacf_SliceEventExposure_Subscribe (EventID = "Number of UE registered", EventFilter = "S-NSSAI", Event reporting mode = periodic along with periodicity) as defined in clause 5.2.21.4.2 of TS 23.502 [3].

-	Individual UE registration/deregistration to a S-NSSAI or to a S-NSSAI and NSI ID reported by AMF(s):

-	Namf_EventExposure_Subscribe (Target for Event Reporting = "any UE", Event ID = "UE moving in or out of a subscribed "Area of Interest", Event Filter information = S-NSSAI(s) or one or more of the tuples (S-NSSAI, NSI ID), Event reporting mode = reporting to a maximum number or a maximum duration) as defined in clause 5.2.2.3.1 of TS 23.502 [3].

NWDAF collects input data on the number of PDU Sessions established in a S-NSSAI using one of the following options:

-	Total number of PDU Sessions established in a S-NSSAI from each SMF(s) and/or NSACF:

-	Nnsacf_SliceEventExposure_Subscribe  (EventID = "Number of PDU sessions established", EventFilter = "S-NSSAI(s)", Event reporting mode = periodic along with periodicity) as defined in clause 5.2.21.4.2 of TS 23.502 [3].

-	Individual PDU Session Established or PDU Session Released in a S-NSSAI from SMF:

-	Nsmf_EventExposure_Subscribe (Target for Event Reporting = "any UE", Event ID = "PDU Session Establishment and/or PDU Session Release", Event Filter information = S-NSSAI(s), Event reporting mode = reporting to a maximum number or a maximum duration) as defined in clause 5.2.8.3.1 of TS 23.502 [3].

Next CHANGE

6.3.4	Procedures

Figure 6.3.4-1: Network Slice load analytics provided by NWDAF

Figure 6.3.4-1 shows the procedure for NWDAF to derive slice load analytics. The steps are described as follows:

1.	A consumer NF subscribes to/requests a NWDAF using Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operation (Analytics ID = Load level information and a set of Analytics Filters (e.g. S-NSSAI, NSI ID, Area of Interest)).

2.	[OPTIONAL] If the NWDAF does not have already the slice information, it gains the slice information from OAM (as described in clause 6.2.3) and discovers from NRF the AMF, SMF and NSSF instance(s) relevant to the Analytics Filters provided in the analytics subscription.

3.	[OPTIONAL] If the NSI ID(s) are not provided in the analytics subscription by the consumer NF, the NWDAF invokes Nnssf_NSSelection_Get service operation from NSSF to obtain the NSI ID(s) corresponding to the S-NSSAI in the subscription.

NOTE:	Step 4a to step 7 are conditional depending on the NWDAF internal logic that determines the source(s) of data collection.

4a.	[CONDITIONAL] The NWDAF may subscribe to input data in Table 6.3.2A-1 from the OAM according to the data collection principles from the OAM described in clause 6.2.3.

4b.	[CONDITIONAL] The NWDAF may collect input data from the NRF (see clause 6.5) to derive slice instance resource usage statistics and predictions for a Network Slice instance.

5.	[CONDITIONAL] The NWDAF may subscribe to the AMF(s) event exposure service to collect data on the number of UEs currently registered on certain Network Slice and, if available, its constituent Network Slice instance(s) as defined in clause 6.3.2A. If required, the NWDAF may also collect the corresponding UE IDs.

6.	[CONDITIONAL] The NWDAF may subscribe to the SMF(s) event exposure service to collect data on the number of PDU sessions established and/or released at the SMF on currently registered on certain Network Slice as defined in clause 6.3.2A. NWDAF can then use such collected data to determine the number of PDU sessions established on i) a Network Slice; and ii) if available, on a Network Slice instance by leveraging the data collected in step 5.

7.	[CONDITIONAL] The NWDAF may subscribe to the NSACF to collect data on either the number of UE registered in a S-NSSAI or the number of PDU sessions established in a S-NSSAI as defined in clause 6.3.2A.

8.	The NWDAF derives slice load analytics.

9.	The NWDAF delivers analytics to the consumer NF by invoking Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response service operations.

END of CHANGES