SA WG2 Meeting #149e	S2-2201064r02

# February 14th – 25th, 2022; Elbonia               		(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell, Xiaomi

Title: 	Enhancements to power savings for XR services

Document for: 	Approval

Agenda Item: 	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: This paper proposes a KI to address Work Task #4.2

# Discussion

The FS_XRM SID contains the following Work Task:

WT#4: Study potential enhancements of power management considering traffic pattern of media services:

- WT 4.1: Void.

- WT 4.2: Power saving enhancement e.g. support trade-off of throughput/latency/reliability considering device battery life, whether and how to enhance CDRX, considering XR/media traffic pattern.

WT#4: Study potential enhancements of power management considering traffic pattern of media services:

- WT 4.1: Void.

- WT 4.2: Power saving enhancement e.g. support trade-off of throughput/latency/reliability considering device battery life, whether and how to enhance CDRX, considering XR/media traffic pattern.

WT#4.2 addresses the scenario where information on the XR media traffic like pattern, periodicity and other key statistics as well as device battery status are exposed to the network to enhance power saving schemes operating in the RAN like CDRX and PDCCH monitoring mechanisms.

# 2 Proposal

It is proposed to update TR 23.700-60 as follows

# * * * * First change (all new text)* * * *

## x.1	Key Issue #X: Enhancements to power savings for XR services

x.1.1 	Description

The objective of this Key Issue is to study whether and how to improve power management schemes like CDRX to achieve the best trade-off among key performance indicators like latency and device battery lifetime.

Solutions for this Key Issue rely on the enhancements to power saving schemes to support XR services studied during RAN1 SI on XR evaluation in Release 17 and under investigation in Release 18 and builds on top of those features.

For this Key Issue the following areas should be studied:

- Which information (e.g., XR/media traffic characteristics, traffic pattern and statistics), if any, is needed by the RAN from the CN and/or the UE to enhance   power management (i.e., CDRX) ? If needed, how such information is collected?

NOTE:	This Key issue requires collaboration with RAN WGs.