SA WG2 Meeting #149e	S2-220XXXXr06

# February 14th – 25th, 2022; Elbonia               		(revision of S2-220)

Source: 	NTT DOCOMO, vivo, Nokia, CATT, Samsung

Title: 	KI(WT#3.2): Enhance trained ML Model sharing

Document for: 	Approval

Agenda Item: 	9.23

Work Item / Release:	FS_eNA_Ph3/ Rel-18

Abstract of the contribution: This contribution proposes a KI to address the enhanced trained ML Model interoperable with the AnLF.

1	Discussion

Based on the approved WT#3.2 and Rel-17 limitation, it is proposing a KI should address the ML model sharing between different vendors.

2 Proposal

It is proposed to update TR 23.700-81 on FS_eNA_Ph3 as follows

5	Key Issues

5.X	Key Issue #X: Enhance trained ML Model sharing

5.X.1	Description

As described in the below note from clause 5.1, TS 23.288 that in Rel-17 the ID(s) of the NWDAF containing MTLF(s) is locally configured in the NWDAF containing AnLF, and the NWDAF containing AnLF is only allowed to retrieve trained ML model(s) from the configured NWDAF containing MTLF(s).

NOTE 3:	In this Release of the specification an NWDAF containing AnLF is locally configured with (a set of) IDs of NWDAFs containing MTLF and the Analytics ID(s) supported by each NWDAF containing MTLF to retrieve trained ML models. An NWDAF containing AnLF uses NWDAF discovery for NWDAF  containing MTLF within the set of configured IDs of NWDAFs containing MTLF, if necessary. ML Model provisioning/sharing between multiple MTLFs is not supported in this Release of the specification.

In Rel-18, the following aspects are to be studied to enhance trained ML Model sharing between NWDAFs from different vendors:

- How to discover and select a NWDAF containing MTLF even from different vendors which can provide interoperable trained ML model(s), by a NWDAF containing AnLF.

- How to retrieve interoperable trained ML model(s) from a NWDAF containing MTLF even from different vendors by a NWDAF containing AnLF, including:

- Whether and what new functionality / function(s) is needed for support of trained ML model interoperability?

- What additional information should be exchanged between the NWDAF containing MTLF and the NWDAF containing AnLF, for support of trained ML model interoperability.

- Whether the need to define common characteristics for trained ML models from different vendors? e.g. using a unified data structure for trained ML models, to enable trained ML model sharing between NWDAFs from different vendors.

NOTE:	Coordination with SA5 is needed for ML model sharing aspects.

NOTE: Coordination with SA3 is needed for security aspects related to ML model sharing (e.g., ML model integrity, authorization of use, etc.) as investigated by this Key Issue.