SA WG2 Meeting #149e	S2-2201094r07

# February 14th – 25th, 2022; Elbonia               		(revision of S2-220)

Source: 	Intel, Nokia, Nokia Shanghai Bell

Title: 	FS_SFC KI related with WT3

Document for: 	Approval

Agenda Item: 	9.1

Work Item / Release:	FS_SFC / Rel-18

Abstract of the contribution: This key issue is proposed for WT#3.

1	Discussion

This key issue is proposed based on WT#3:

WT#3: Depending on the outcome of WT#2, specify possible enhancement to northbound APIs for allowing an AF to request network capability exposure functionalities, e.g. request a service function chain for a certain traffic flow, etc., based on service level agreement with the third party.

2 Proposal

It is proposed to update TR 23.700-18 on FS_SFC as follows:

*** BEGIN CHANGES ***

5.X	Key Issue #X:  Exposure to enable AF to request a service function chain

5.X.1	Description

For allowing an AF e.g. a third party AF to request SFC for traffic flow(s), etc., (when the AF belongs to a third party, this is based on service level agreement with the third party),this key issue will study solutions on:

- Whether the existing traffic steering policy and network capability exposure is enough  to fulfil the SA1 requirements.

- How to enable support for AF to request predefined SFC configured by the operator for traffic flow(s) related with UE targets.

- Whether or how to enhance network capability exposure functionalities based on solutions of the User plane related Key issue (WT#2).

- Based on the requests from the AF, what are the solutions and procedures to interact with network functions in the control plane of 5GC.

NOTE x: AF only request service function chains pre-defined by the operator by 5G network. The creation/modification of service function chains, e.g. deciding which SFs to be included in service function chain, and the order of SFs belongs to management aspects, is out of scope of SA2.

*** END CHANGES ***