SA WG2 Meeting #149e	S2-2201174r01

# February 14th – 25th, 2022; Elbonia               		(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	FS_UPEAS new KI: define requirements of UPF event exposure service(s)

Document for: 	Approval

Agenda Item: 	9.25

Work Item / Release:	FS_UPEAS / Rel-18

Abstract of the contribution: FS_UPEAS new KI: define requirements of UPF event exposure service(s)

# 1	Discussion

FS-UPEAS objectives contain:

WT#1: Study UPF event exposure service(s) registration/deregistration, and discovery via the NRF.

WT#2: Study UPF event exposure service(s) that would support, e.g.

-	Consumption of UPF exposure services by the PCF, NWDAF, CHF, NEF, Trusted AF and other NFs (if needed).

-	(To support the UPF exposure service, if needed) Use of SMF services, PCF services, NWDAF services, CHF services, NEF services, Trusted AF services by the UPF.

NOTE 1: This will not define solutions where UPF exposes information that it is not originator of, i.e. not re-expose information owned and exposed by other NFs.

-	Relevant Event IDs.

WT#3: Evaluate usage of UPF event exposure service(s) as defined in WT#2 also considering the architectural impacts

FS_XRM SP-211646 WT#2: Enhancements of network exposure to support interaction between 5GS and application：

….

WT#2.2:  Study exposure of 5GS QoS information (e.g., QoS capabilities) and network conditions to the Application to enable quick codec/rate adaptation help to provide desired QoE (e.g. such as assist in alleviating 5GS congestion).

# 2 Proposal

It is proposed to update 23.700-62 as follows

CHANGE (1) ALL text is new

## 5.Y	Key Issue #Y: define requirements of UPF event exposure service(s) .

### 5.Y.1	Description

This KI relates to the WT2 of the SID.

In R17 consumers (NEF, AF) of UPF event exposure subscribe to this UPF service via the SMF .

The key issue is to define requirements for UPF event exposure (including the related Event Id) and for each requirement to determine whether the consumer can directly contact the UPF for its subscription or whether the consumer needs to go via an intermediate function like the SMF. This key issue does not intend to work on how the direct subscription to UPF event could take place (e.g. potential usage of NRF etc…) which is handled by another key issue.

For this purpose, the KI will study

-	Which information the UPF can expose to NWDAF to facilitate data collection and analysis considering efficient sampling for the different analytics. R17 TS 23.288 [X] requirements are considered as the input for this item.

-	Which specific information the UPF can expose to NEF/Local NEF/ trusted AF. For this item the work will focus on requirements from R18 FS_ EDGE_Ph2 [Z], R18 FS_AIMLsys [M] and from R18 FS_XRM [Y].

CHANGE (2)

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[X]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[Y]	3GPP TR 23.700-60: “Study on architecture enhancement for XR and media services".

[Z]	3GPP TR 23.700-48: “5G System Enhancements for Edge Computing; Phase 2".

[M]	3GPP TR 23.700-80: “Study on 5G system support for AI/ML-based services”