SA WG2 Meeting #149e	S2-2201190r02

# February 14th – 25th, 2022; Elbonia               	    		 	 				   (revision of S2-220XXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0050 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removing inconsistency in the definition of ECS Address Configuration Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2022-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 23.548 § 6.5.2.1 contains: The ECS Address Configuration Information consists of one or more FQDN(s) and/or IP address(es) of Edge Configuration Server(s), and of an ECS Provider ID. It may be associated with spatial validity conditions. The SMF does not need to be aware of the internal structure of the ECS Address Configuration Information. 23.502 Table 4.15.6.3d-1: Description of ECS Address Configuration Information provided by the AF Parameters; Description. ECS Address Configuration Information; Consists of: - one or more FQDN(s) and/or IP Address(es) of Edge Configuration Server(s); - an ECS Provider ID. The identifier of the Edge Configuration Server (ECS) Provider (such as Edge Computing Service Provider); - Spatial Validity Condition per Edge Configuration Server(s), which may correspond to one of following alternatives: - a Geographical Service Area (see TS 23.558 [74]); - a list of TA(s); - a list of countries (list of MCC); where ECS Address Configuration Information is applicable.. Target; This may correspond to one of: - a group of UE identified by an external group Id; - any UE.. 24.008 § 10.5.6.3 (PCO) contains - 0032H (ECS IPv4 address) - 0033H (ECS IPv6 address) - 0034H (ECS FQDN) - 0035H (ECS provider identifier) - 0047H (Spatial validity condition for ECS IPv4 address) - 0048H (Spatial validity condition for ECS IPv6 address) - 0049H (Spatial validity condition for ECS FQDN); and 23.548 § 6.5.2.1 indication that the ECS Address Configuration Information may be associated with spatial validity conditions assumes that spatial validity conditions are not part of the ECS Address Configuration Information which is contradicting other specifications. This CR thus also aligns with the CT1 agreed CR 3298 to TS 24.008 (C1-220628) r01 removes 6.5.2.2, since there are no changes proposed in 6.5.2.2 |
| - |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | - ECS Address Provisioning information may contain spatial validity conditions. - The ECS Address Configuration Information is sent to the UE on a per PDU Session basis |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START CHANGES

#### 6.5.2.1	ECS Address Configuration Information

The ECS Address Configuration Information consists of one or more FQDN(s) and/or IP address(es) of Edge Configuration Server(s), and  an optional ECS Provider ID. It may also contain spatial validity conditions. It is further described in TS 23.502 [3]. A UE may receive multiple instances of ECS Address Provisioning information (e.g. corresponding to different ECS Provider ID).

The ECS Address Configuration Information is sent to the UE on a per PDU Session basis; The same PDU session can be used by multiple ECS providers.

The SMF does not need to be aware of the internal structure of the ECS Address Configuration Information.

END CHANGES