SA WG2 Meeting #149e	S2-2201232r04

# February 14th – 25th, 2022; Elbonia               		(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell, NTT DOCOMO, Lenovo, Motorola Mobility, Huawei, HiSilicon, Vivo, CATT

Title: 	WT#6 Key Issue Description and Scenario

Document for: 	Approval

Agenda Item:         9.11

Work Item / Release:	FS_EDGE_Ph2 / Rel-18

Abstract of the contribution: Key issue description and scenario on SID WT#6

1	Discussion

Approved SID has this WT#6: Investigate the potential need and solutions to influence of PSA-UPF and EAS (re)location for collection of UEs, e.g. in scenarios when UE(s) should use the same EAS and are not members of a pre-defined group.

This paper proposes description and scenario corresponding to this work task.

2 Proposal

It is proposed to add below text to EC TR 23.700-48

# * * * * Start of changes * * * *

5.X	KI#X: Influencing UPF and EAS (re)location for collections of UEs

5.X.1	Description

Editor's note:	This key issue corresponds to Work Task #6 in SP-211638 i.e. the below work task:

Investigate the potential need and solutions to influence of PSA-UPF and EAS (re)location for collection of UEs, e.g. in scenarios when UE(s) should use the same EAS and are not members of a pre-defined group.

In particular, the key issue will study the following aspects:

-	Define what a collection of UEs that should use the same EAS and/or same LDN and/or same DNAI is and how it is identified.

-	How can the 5GS determine which UEs belong to the collection of UEs.

-	How to decide on a common L-DN for the collection of UEs.

-	How to determine that the L-PSAs for the PDU Sessions that are used to access the EAS must be relocated.

-	How to handle coordination of the UPF and EAS (re)location for collections of UEs.

-   Whether and how existing mechanisms suffice.

- What information does the AF provide to network for assistingEAS discovery and re-discovery?

- EAS discovery and/or rediscovery procedure to (re-)discovery EAS for UEs belongs to the collection of UEs.

5.X.2	Scenarios

Editor's note:	This clause will document the scenarios (and potential associated use cases) applicable to KI#4, if any. This clause will be removed if left empty.

There are use cases that UEs belonging to a non-predefined dynamic group should be treated the same way, and members of the dynamic group is likely to change dynamically, e.g. UE could join/leave the group randomly. For example:

There are multiple use cases and scenarios such as:

Multi-user low latency Gaming: UEs identified by application (e.g. UEs within a geographical area) need to interact with each other for accessing to the same gaming service, the UEs needs to connect to the same application server or application servers associated with the same DNAI to reduce the interacting latency from application server side.. Also, due to maintenance purposes or due to overload situation EAS would require to be relocated to another one, thus moving all registered users to the new EAS.

Platooning: In case of platooning use case, all the member UEs involved has similar attributes and requirements, for example, all UEs in a particular platoon have similar mobility characteristics i.e. moving in the same direction and with similar speed, located in proximity to each other, and so on.

Below figure shows example scenario applicable to ad-hoc and dynamic group applications such as online gaming and platooning:

Figure 5.4.2-1: EAS (re)location for collections of UEs

- The stateful app1 (e.g. online multiplayer game) is deployed in the local part of the DN accessed via DNAI1 and DNAI2.

- Provisioning the game with efficient cost, it can include the following service scenarios:

- When the first player (UE1) initiates the match, PSA2 is selected to access APP Server1 via DNAI1. When UE2 joins the same match, PSA2 is selected to access APP Server1 via DNAI1 selected to access APP Server1 via DNAI1 although App Server 2 is closer to UE2 than APP Server1.

- When UE1 moves into the same area of UE2, the EAS relocation can be triggered for both UE1 and UE2 in order that both UE1 and UE2 within the same match are served in the same APP server.

- When the match ends, the binding of the players to the match is disassociated. The UE2 can be routed to APP Server 2 for the game accessed via DNAI2. If the UE1 moves back to the original area, UE1 can be routed to APP Server 1 for the game accessed via DNAI1.

The following scenarios will be investigated for this key issue:

Figure 1 Scenario#1: UEs in dynamic group connecting to the same EAS.

Figure 2 Scenario#2: UEs in dynamic group connecting to the EASs associated with the same DNAI

5.X.3	Assumptions

Editor's note:	This clause will document assumptions applicable to KI#4, if any. This clause will be removed if left empty.

# * * * * End of changes * * * *