SA WG2 Meeting #150e	S2-2202563

# April 6th – 12th, 2022; Elbonia                  	       (revision of S2-220xxxx)

Source: 	vivo

Title: 	FS_5WWC_Ph2 Update of Solution 4 for providing differentiated service for non-3GPP devices connected behind a 5G-RG

Document for: 	Approval

Agenda Item: 	9.13

Work Item / Release:	FS_5WWC_Ph2 / Rel-18

Abstract of the contribution: To update Solution 4 for supporting QoS differentiation for non-3GPP devices behind 5G-RG is proposed.

1	Discussion

To solve the following ENs in solution 4, this contribution updates the solution by adding the following:

This solution is used for Non Authenticable Non-3GPP (NAUN3) devices.

The 5G-RG is pre-configured or receives session policy for non-3GPP devices from the network during its PDU Session Establishment procedure. The session policy for non-3GPP devices describes the following information:

a) to establish a QoS flow for one category of non-3GPP devices;

b) the DNN / S-NSSAI corresponding to the PDU session of the QoS flow for non-3GPP devices;

c) the non-3GPP device category identifier. The non-3GPP device category identifier is used to identify one or a group of non-3GPP devices.

When the 5G-RG detects the connection of a non-3GPP device, the 5G-RG establishes a QoS flow for the non-3GPP device according to the session policy. If there is already a corresponding PDU Session (i.e. a PDU sessions with the same DNN/S-NSSAI), the 5G-RG performs PDU Session Modification procedure. If there is no corresponding PDU Sessions (i.e. no PDU sessions with the same DNN/S-NSSAI), the 5G-RG performs PDU Session Establishment procedure.

The corresponding procedures are added.

2 Proposal

It is proposed to update TR 23.700-17 Solution 4 as follows.

6.4	Solution 4: Solution of providing differentiated service for Non-3GPP devices connected behind a 5G-RG

6.4.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). (Sub) clause(s) may be added to capture details.

This solution provides differentiated service for non-3GPP devices connected behind a 5G-RG. In this solution, the 5G-RG receives QoS rules for the non-3GPP devices behind the 5G-RG. Then the 5G-RG uses the corresponding data transfer tunnel to transfer the packets of the non-3GPP devices according to the QoS rules.

This solution is used for Non Authenticable Non-3GPP (NAUN3) devices.

The 5G-RG is pre-configured or receives session policy for non-3GPP devices from the network during its PDU Session Establishment procedure. The session policy for non-3GPP devices describes the following information:

a) to establish a QoS flow for one category of non-3GPP devices;

b) the DNN / S-NSSAI corresponding to the PDU session of the QoS flow for non-3GPP devices;

c) the non-3GPP device category identifier. The non-3GPP device category identifier is used to identify one or a group of non-3GPP devices.

When the 5G-RG detects the connection of a non-3GPP device, the 5G-RG establishes a QoS flow for the non-3GPP device according to the session policy. If there is already a corresponding PDU Session (i.e. a PDU sessions with the same DNN/S-NSSAI), the 5G-RG performs PDU Session Modification procedure. If there is no corresponding PDU Sessions (i.e. no PDU sessions with the same DNN/S-NSSAI), the 5G-RG performs PDU Session Establishment procedure.

6.4.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

Figure 6.4.2-1 5G-RG establishes a QoS flow for one category of non-3GPP devices

1.	non-3GPP device connects to 5G-RG with non-3GPP access network (e.g. establishes WiFi association).

During this procedure, 5G-RG obtains non-3GPP device category identifier. The 5G-RG may need to map the non-3GPP device MAC address, WLAN SSID or physical Ethernet port identifier used by the non-3GPP device to non-3GPP device category identifier. How the 5G-RG performs such mapping is 5G-RG implementation dependent.

The 5G-RG may configure IP address (i.e. using IPv6 prefix delegation or local IPv4 address allocation mechanism) to the non-3GPP device.

2.	5G-RG sends NAS message to the AMF. The NAS message includes a PDU Session Modification Request, including PDU Session ID, 5G-RG ID and non-3GPP device's MAC address. Optionally, non-3GPP indication is also included.

The 5G-RG determines to establish a QoS flow for the category of non-3GPP devices according to the session policy for non-3GPP devices.

NOTE:	It is assumed that the 5G-RG already establishes a PDU session.

3.	The AMF sends N11 message to the SMF and forwards the PDU Session Modification Request.

4.	The SMF reports non-3GPP device category identifier to the PCF by initiating the SM Association Modification procedure.

The PCF provides QoS rules for the category of non-3GPP devices to the SMF.

5.

For Ethernet PDU Session type, the SMF updates the UPF with N4 Rules by indicating that this PDU session is also used to transfer the traffic related to the non-3GPP device's MAC address.

6.	The SMF generates PDU Session Modification message including the QoS rules for the non-3GPP device.

The SMF sends N11 message to the AMF, including the PDU Session Modification message, 5G-RG ID and non-3GPP device category identifier.

7.	The AMF sends NAS message to the 5G-RG, forwarding the PDU Session Modification message to the 5G-RG.

The 5G-RG maps the traffic of the non-3GPP device to the QoS flow of this PDU session according to the received QoS rule.

When the 5G-RG detects disconnection of the non-3GPP device, if there is no traffic of other non-3GPP devices maps to the QoS flow, the 5G-RG initiates PDU Session Modification procedure to release such QoS flow.

Figure 6.4.2-2 5G-RG establishes a PDU session for one category of non-3GPP devices

If the 5G-RG determines to establish a PDU session for the non-3GPP device according to the session policy for non-3GPP devices, the 5G-RG performs PDU session establishment procedure according to TS 23.502 clause 4.3.2.1 with the following differences:

In step 1, 5G-RG includes non-3GPP device category identifier in PDU Session Establishment Request.

During SM Policy Association Establishment procedure, in step 3, the SMF reports non-3GPP device category identifier to the PCF, and in step 4, the PCF sends PCC policy corresponding to the non-3GPP device category identifier.

In step 6 and 7, the SMF includes the non-3GPP device category identifier and QoS rule of the non-3GPP device in PDU Session Establishment Response.

The 5G-RG maps the traffic of the non-3GPP device to this PDU session according to the received QoS rule.

Figure 6.4.2-3 5G-RG receives session policy for non-3GPP devices

The 5G-RG receives the session policy for non-3GPP devices during its own PDU Session establishment procedure. The 5G-RG performs PDU session establishment procedure according to TS 23.502 clause 4.3.2.1 with the following differences:

In step 1, 5G-RG includes non-3GPP access indication in PDU Session Establishment Request to requests the session policy for non-3GPP devices behind it.

During SM Policy Association Establishment procedure, in step 3, the SMF reports non-3GPP access indication to the PCF and in Step 4, the PCF sends session policy for non-3GPP devices to the SMF.

In step 6 and 7, the SMF sends the session policy for non-3GPP devices to the UE in PDU Session Establishment Response.

6.4.3	Impacts on Existing Nodes and Functionality

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

5G-RG:

-	5G-RG requests for QoS rule  for the non-3GPP device behind it, by sending PDU session modification message to the network and receives the requested information from the network.

-	5G-RG receives session policy for non-3GPP devices from the network and establishes PDU Session or QoS flow based on the session policy for non-3GPP devices.

-	5G-RG includes non-3GPP access indication in PDU Session Establishment Request to requests the session policy for non-3GPP devices behind it.

SMF:

-	The SMF requests QoS rule for non-3GPP device from the PCF.

-	The SMF sends the non-3GPP access indication to the PCF and receives session policy for non-3GPP devices.

-	The SMF sends the session policy for non-3GPP devices to the UE in PDU Session Establishment Response.

PCF:

-	The PCF provides QoS rules for non-3GPP devices.

-	The PCF provides session policy for non-3GPP devices to the SMF.

************************** END OF CHANGE ***********************