SA WG2 Meeting #150E (e-meeting)	S2-2203389

February 14 – 25, 2022, Elbonia	(revision of S2-2202278r01)

Source:	LG Electronics

Title:	KI 1, New Sol: Exposing UE provided unavailability period via Loss of Connectivity event subscription

Document for:	Approval

Agenda Item:	9.27 Study on Seamless UE context recovery

Work Item / Release:	FS_SUECR / Rel-18

Abstract of the contribution: This paper proposes a new solution to Key Issue #1.

Discussion

Proposal

It is proposed to agree the following changes into TR 23.700-61 on FS_SUECR.

* * * * Start of 1st Change (All new text) * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 |  |  |  |
| X | X |  |  |  |

* * * * Start of 1st Change (All new text) * * * *

6.X	Solution #X: Exposing UE provided unavailability period via Loss of Connectivity event subscription

6.X.1	Introduction

This solution addresses key issue #1: Determination of unavailability period in 5GS for a specific UE.

6.X.2	Functional Description

This solution proposes to extend "Loss of Connectivity" event subscription to notify unavailability period of the UE if the UE provides unavailability period via NAS message. How the UE determines unavailability period is not defined by this solution and out of scope of 3GPP.

6.X.3	Procedures

Figure 6.X.3-1. Overall procedure for unavailability period report to the AF

1.	The UE is registered with 5GC.

2-7.	The AF subscribes "Loss of Connectivity" event to the NEF based on existing procedure.

8.	The UE may send NAS message (e.g. Registration, Deregistration) with unavailability period information, which is determined by the UE.

9.	If the AMF received unavailability period in the NAS message and there is "Loss of Connectivity" event subscription for the UE, the AMF includes unavailability period in the event notification report even though the UE is not deregistered.

10.	If the NEF receives unavailability period in the event notification report from the AMF, the NEF provide the received unavailability period to the AF.

6.X.4	Impacts on services, entities and interfaces

UE:

-	Provides unavailability period via NAS message.

AMF:

-	If the UE provides unavailability period via NAS message and there is "Loss of Connectivity" event subscription for the UE, the AMF triggers "Loss of Connectivity" event report and includes unavailability period during the "Loss of Connectivity" event report.

NEF:

-	Notifies unavailability period during the "Loss of Connectivity" event report to the AF if the AMF provided unavailability period in the "Loss of Connectivity" event report.

* * * * End of Changes * * * *