SA WG2 Meeting ##151-e	S2-2203665

16-20 May 2022, Electronic Meeting

3GPP TSG-RAN WG3 Meeting #116 Electronic	R3-223791

Online, 9th – 19th May 2022

Title:	Reply LS on handover issue for 5G access stratum time distribution

Response to:	R3-223034/ S2-2203596 LS On handover issue for 5G access stratum time distribution

Release:	Release 17

Work Item:	NR_IIOT_URLLC_enh-Core

Source:	RAN3

To:	SA2

Cc:	-

Contact Person:

#### Name:	Ma zijiang

E-mail Address:	ma.zijiang@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN3 would like to thank SA2 for their LS on handover issue on 5G access stratum time distribution.

RAN3 provides the following feedback on the SA2 LS:

Question: What is RAN3 view on transfer of such context during Xn/NGAP HO?

Answer:

RAN3 has specified the Time Synchronisation Assistance Information (i.e. the 5G access stratum time distribution indication and the Uu time synchronization error budget), which is transferred during handover as follows:

- For NG-based handover, the AMF provides the Time Synhronisation Assistance Information to the target gNB via the NGAP: HANDOVER REQUEST message.

- For Xn-based handover, the source gNB provides the Time Synchronisation Assistance information to the target gNB via the XnAP: HANDOVER REQUEST message. The AMF may update the information via the NGAP: PATH SWITCH REQUEST ACKNOWLEDGE message.

2. Actions:

To SA2:

ACTION: 	RAN3 respectfully asks SA2 to take the above into account.

3. Date of Next TSG-RAN WG3 Meetings:

3GPP RAN3#117-e			15 - 25 August 2022		Electronic Meeting

3GPP RAN3#117bis-e	10 - 18 October 2022	    	Electronic Meeting