SA WG2 Meeting #S2-151e(e-meeting)	S2-2203713

16 - 20 May 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Ericsson

Title:	Conclusion for KI#1

Document for:	Approval

Agenda Item:	9.8

Work Item / Release:	FS_RedCap_Ph2

Abstract of the contribution: Introduction of initial conclusion based on existing solutions and evaluations.

# 1. Introduction

This paper introduces the initial conclusion for KI#1

# 2. Discussion

Based on the current proposed solutions and initial evaluations in TR 23.700-68, they can be divided to two types:

A) Using existing CN data buffering capability and existing MT data/signalling handling in CN when UE is not reachable. When UE becomes reachable, the CN triggers the RAN paging even though the N3 exists.

B) Using existing CM-CONNECTED with RRC_INACTIVE and CM-IDLE mode handling. The gNB makes the decision based on its capability (e.g., data buffering capability for long eDRX) to decide which is the most appropriate state that UE shall enter.

Solutions #1, #3, #4, #5 and #6 (which is a converged solution based on Solutions #1,#3 and /#4) can be seen as type A), while solution #2 is type B).

Solutions #3 and #5 further consider that some data may can reach RAN and can be buffered in RAN, even the general principle in the solutions is that CN buffers the data when UE is not reachable due to long eDRX in CM-CONNECTED with RRC_INACATIVE state.

Some high level realizations of the functionalities:

- The handling proposed in type A solutions (e.g., though with slightly different names in different solutions) is about additional CN handling of MT data/signalling even for UE in RRC_INACTIVE state (e.g. CM-CONNECTED with RRC_INACTIVE with long eDRX), besides the normal MT data/signalling handling in RAN for UE in RRC_INACTIVE state and in CN for UE in CM-IDLE mode as proposed in type B solution. However, the additional MT data/signalling handling and existing MT data/signalling handling for RRC_INACTIVE state are not necessarily to be exclusive to each other, i.e., network can decide which one to apply at any given time.

- The Data buffering in RAN may happen for both type A solutions or type B solution, though type B solution may impose more capability requirement in RAN (e.g., extended buffering capability in RAN node) to get the full benefit of RRC_INACTIVE state. In such scenario, the data delivery from source RAN to target RAN needs to be addressed. The CN based data buffering tries to reduce such possibility of unnecessary data delivery and forwarding when UE is reachable after long eDRX. These different options may be needed for different network deployment/operator preference (e.g., based on the RAN architecture with support of CU and DU separation).

Based on above, it’s proposed to support the conclusions proposed below.

# 3. Proposal

This paper proposes the following conclusion points to TR 23.700-68 clause 8.

* Start of change *

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

## 8.1	Conclusions for Key Issue #1: Enabling long eDRX cycle support in RRC_INACTIVE state

The following aspects are concluded as principles for the normative work:

-	Based on the eDRX information for CM-IDLE mode provided by CN, the negotiation of eDRX applied for CM-CONNECTED with RRC_INACTIVE and its handling capability, the gNB either decides to keep the UE in CM-CONNECTED with RRC_INACTIVE and handles the MT data/signalling in RAN in the unreachable time as described in clause 6.2.3.2 of solution #2, or requests the CN for MT data/signalling handling in the unreachable time by notifying the CN when UE entering CM-CONNECTED with RRC_INACTIVE and long eDRX as described in clause 6.6.3.1 of solution #6.

-	In case of CM-CONNECTED with RRC_INACTIVE and there is data buffering in gNB, the UE context retrieval between target gNB and source gNB is supported. The UE context transfer via CN is supported (e.g., as described in clause 6.2.3.3 of solution #2) when there is no Xn interface between gNBs. In case gNB decides to request CN for MT data/signalling handling, e.g., due to MT signalling delivery failure or data buffering limitation, the gNB requests the CN to further handle MT data/signalling in the unreachable time in RRC_INACATIVE state as described in clause 6.6.3.1 of solution #6.

NOTE 1:	The details of the UE context retrieval procedure need to be synchronized with RAN WGs.

-	In case of CM-CONNECTED with RRC_INACTIVE with CN MT data/signalling handling, the gNB notifies CN with RRC State transition information as described in clause 6.6.3.2 of solution #6. CN applies the HLCOM functions when UE is unreachable.

* End of changes *