# 3GPP TSG-SA WG2 Meeting #151E	S2-2203714

Elbonia, 16 - 20 May 2022	(Revision of S2-xxxxxxx)

Title:	[Draft] LS On FS_REDCAP_Ph2 option feasibility

Response to:	-

Release:	Rel-18

# Work Item:	     FS_REDCAP_Ph2

Source:	Ericsson [to be SA2]

To:	RAN3, RAN2

cc:	CT4, CT1

Contact Person:	Qian Chen

E-mail Address:	qian (dot) xb (dot) chen (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 Rel-18 study FS_REDCAP_Ph2 is ongoing and approaching the conclusion phase. The study is focused on one key issue which enabling long eDRX>10.24s cycle support for UE in RRC_INACTIVE state. See more details in latest TR 23.700-68.

SA2 discussed mainly two different types of solutions:

A) 	RAN provides UE unreachability information (e.g. the eDRX information) in RRC_INACTIVE state to CN when UE enters RRC_INACTIVE with long eDRX and CN handles the MT data/signalling when UE is unreachable. MT data/signalling and RAN paging will be triggered when the UE becomes connected again or the UE is considered reachable. See solution#6 in TR for reference.

B) 	RAN handles MT data/signalling, and decides if UE needs to be changed to CM-IDLE (e.g. when buffering capability close to the limit). In case UE moves out the RNA area in unreachable time period and performs resume outside the RNA, the UE context retrieval between RAN nodes with data forwarding are supported via CN when there is no Xn interface. See soluiotn#2 in TR for reference.

Considering different deployment and capabilities in different network entities, SA2 considers it is beneficial to conclude a solution with parts from both types of solutions, i.e., RAN makes a decision based on deployment preferences and conditions if RAN wants CN handles MT data/signalling as in A). Even in case RAN decides to handle MT data/singling at first as in B), RAN can still keep the UE in RRC_INACATIVE state and switch to CN MT data/signalling handling as in A) due to certain limits. The data buffered previously in RAN can be delivery to UE as described in B).

SA2 would like to ask RAN2 and RAN3 on their views on the following aspects:

- Kindly provide RAN feedback for gNB providing UE eDRX in RRC_INACTIVE state information to CN for MT data/signalling handling when UE is not reachable in RRC_INACTIVE state.

- Kindly provide RAN view for including the UE context retrieval with data forwarding handling via CN in Rel-18.

2. Actions:

To RAN3, RAN2

ACTION: 	SA2 kindly asks RAN2 and RAN3 take the above information into consideration and provide their views on the two aspects.

3. Date of Next RAN3 Meetings:

SA2#152e                   August 22-26, 2022                                    Gothenburg, Sweden