SA WG2 Meeting #S2-151E	S2-2203724

16 - 20 May, 2022, E-Meeting

Source: 	Ericsson

Title: 	KI #1: Evaluation of Key Issue #1

Document for: 	Agreement

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This paper proposes an evaluation of Key Issue #1.

# Discussion

There is one solution (Solution #1) addressing KI#1.

The principles of the solution #1 includes support for equivalent SNPNs by re-using the logic defined for PLMNs.

The KI#1 includes the following:

Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection. This KI aims at studying impacts to the 5G System for scenarios where the UE has a subscription with each of the source and target SNPN or can access both the source and target SNPN using credentials from a Credentials Holder:

-	Study how to enable optimizations for idle mode mobility without new network selection in the inter-SNPN mobility case.

Solution#1 addresses this by providing the UE with a list of SNPN identities to the UE that the UE consider as equivalent to the registered SNPN during cell (re)selection avoiding the need to perform network selection at inter-SNPN change.

-	Study how to enable optimizations for connected mode mobility without new network selection in the inter-SNPN mobility case.

Solution#1 addresses this by allowing equivalent SNPNs belonging to the same administrative entity being included in the MRL sent to NG-RAN.

-	Whether and how session continuity can be supported in case of idle mode and connected mode mobility between SNPNs.

Solution#1 addresses this by enabling PDU session continuity for the case when the equivalent SNPNs belong to the same administrative entity.

-	Whether and which additional information transfer between SNPNs on top of Rel-17 is required for the above mentioned mobility scenarios.

Solution#1 addresses this by selected SNPN is forwarded between SNPNs.

# Proposal

Add the following solution to TR 23.700-08.

*** BEGIN CHANGES ***

# 7	Evaluation

Editor's note:	This clause provides an evaluation of the solutions.

## 7.1	Key Issue #1: Enabling support for idle and connected mode mobility between SNPNs without new network selection

The key issue #1 is addressed as follows:

-	Idle mode mobility:

-	Solution#1 addresses this by:

-	Providing the UE with a list of SNPN identities to the UE that the UE consider as equivalent to the registered SNPN during cell (re)selection avoiding the need to perform network selection at inter-SNPN change.

-	Connected mode mobility:

-	Solution#1 addresses this by:

-	Allowing equivalent SNPNs belonging to the same administrative entity being included in the MRL sent to NG-RAN.

-	PDU session continuity is enabled for the case when the equivalent SNPNs belong to the same administrative entity.

Solution #1 re-uses the principles of equivalent PLMNs functionality enabling the support of equivalent SNPNs and thereby provides enhancements to idle and connected mode mobility with minor impacts to the system.

*** END CHANGES ***