SA WG2 Meeting #S2-151E	S2-2203728

16 - 20 May, 2022, E-Meeting

Source: 	Ericsson

Title: 	KI #3, Sol#7: update to clarify the configuration of hosting network

Document for: 	Approval

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This paper proposes to remote the editor's note in solution#7.

# Discussion

Previously during meeting SA2#150E, it is agreed to replace Editor's note with a NOTE in Key Issue #3 regarding whether service agreement establishment is part of SA2 work. The new NOTE indicates that SA2 assumes the service agreement is already available.

Editor's note:	It is FFS whether above is to be studied in the scope of SA WG2 and whether SA WG2 works on the assumption that the relationship is already available.

NOTE 1:	SA WG2 works under the assumption that the relationships and the localized service agreements are already available by means outside SA2 scope i.e. SA2 is to agree where the information is made available.

In solution #7, there is an Editor's note regarding whether there is SA2 related work for the configuration of the hosting network.

Editor's note:	It is FFS whether there is SA WG2 work related to the configuration of hosting network. Also how the configuration of the hosting network can be done e.g. via OAM or via other mechanisms involving interactions with service provider.

Also, there is one aspect of Key Issue #3 to investigate which type of interaction between hosting network operator and localized service provider is needed. One example of the interaction is regarding "configuration of the hosting network".

What is required to enable communication between a network operator deploying a hosting network and a localized services provider:

-	Investigate which type of interaction (e.g. configuration of the hosting network, information reporting) is needed, in such relation to enable the localized services provider for making the best use of the hosting network; and

If the hosting network operator and the localized service provider are different entities, BSS(Business Support System) type of interaction is needed between a "Customer" and a "Provider". In localized service use case, the "Customer" here is the localized service provider who wishes to utilize the hosting network for delivering the localized service, and the "Provider" is the hosting network operator who provides the network resources for delivering the localized service. When the "Provider" has received the order from the "Customer", the order will be processed and the hosting network will be configured using OSS (Operations Support System) accordingly. For example, if the order is to create a new network slice in hosting network for the localized service, existing SA5 functions (e.g. TS 28.531, TS 28.541) can be re-used to make the configuration.

If the hosting network operator and the localized service provider are the same entity, the configuration of the hosting network for localized service is assumed to be internal activity, which can be addressed by OAM or OSS.

So, in short the configuration of a hosting network based on agreement/requirement from localized service provider is not in the scope of SA2 WG. Any potential enhancement related to localized service use case can be studied in other WGs (e.g. SA5).

# Proposal

Add the following update to TR 23.700-08.

*** BEGIN CHANGES ***

## 6.7	Solution #7: High level flow for localized service support

### 6.7.1	Introduction

This solution provides a high level and overall flow to enable localized service, and covers key issues KI#3/#4/#5/#6.

### 6.7.2	Functional Description

The steps shown in the Figure 6.7.3-1 describe a sequence of events among the involved entities. Such sequence is assumed to be applicable for enabling localized service in a typical scenario. The order of the steps and the occurrences of each step are not necessarily restricted.

This solution is assumed to be an umbrella solution without further details on how each steps are implemented.

### 6.7.3	Procedures

Figure 6.7.3-1: High level procedures for providing access to local service

H1.	For a hosting network to provide access to localized service, the service provider of the localized service needs to establish localized service agreement with the operator of a hosting network.

The service provider, according to clause 6.41.2.2 of TS 22.261 [2], can be network operators or 3rd party application providers. The services offered from service providers can be localized, and accessed via the hosting network based on the agreement between the different entities.

A service agreement with UE's home network operator is needed to enable, e.g.:

-	UE to receive and use configuration provided by a 3rd party service provider to discover and access a hosting network and localized services.

-	Charging for the use of localized service.

-	The interworking scenarios described in Annex G, clause G.1 of TS 22.261 [2].

-	Service/session continuity between home network and hosting network.

NOTE 1:	The service level agreements established between different entities are work assumptions for SA WG2.

H2.	The hosting network is configured based on the service agreements with localized service provider for the localized service, e.g. QoS, number of end users, time, location, network slicing, etc.

The configuration of home network can include, e.g. whether a subscriber of the home network is authorized to use localized service, whether home network services can be accessed via hosting network, etc.

NOTE X:	The configuration of hosting network based on agreement with localized service provider for localized service is out of SA WG2 study scope. Whether there is a gap that needs to be addressed in 3GPP is up to SA WG5.

H3.	End user/UE is prompted with localized service (e.g. via ticket of an event, commercial etc) , and starts to look for methods how to access the localized service. This could trigger activities on the application layer (e.g. login a web page, scan a QR code etc) and further triggers demand/request from the UE to obtain information related localized services.

NOTE 2:	The application layer activities are outside of 3GPP scope.

H4.	The service provider / hosting network / home network can co-ordinately deliver to the UE information related to localized service(s). This step can also involve the serving network of the UE in case UE is not currently served by home network. The information related to localized service(s) provided to UE in this step can also be unsolicited. In this case, step H3 can happen after, or during this step.

H5.	Based on the received information related to localized service(s), the end user/UE decides to accept and starts the process to discover/select the hosting network when the conditions of the localized service are about to be met (e.g. event time is approaching, end user enters the physical location etc).

H6.	UE connects to the hosting network, possibly with the authorization from home network, and prepares to access the localized service (e.g. User Plane setup, QoS negotiation, etc ).

H7.	UE temporarily stays in the hosting network to obtain the desired localized service and optionally home network services that are available via the hosting network.

H8.	When the temporary access to hosting network for localized service is about to be terminated due to for example event is over, end user has left the area, agreed quota is exhausted, etc, the UE returns from the hosting network.

H9.	Hosting network and/or the service provider collect and provide charging information to UEs' home network operator, depends on the localized service agreement.

NOTE 3:	Charging aspects is to be coordinated with SA WG5.

H10.	When the localized service agreement is terminated, each entity shown in the figure may need to roll back the previous setup, in order to for example maintain the privacy of an end user against hosting network, prevent a UE to re-access hosting network, release network resources etc. The operation of roll back depends how the localized service is agreed between entities, e.g. if it is a time limited or a geographic limited service, whether it is an one-time service for a single event, etc.

### 6.7.4	Impacts on services, entities, and interfaces

Impacts are expected to be described in other solutions.

NOTE:	Security aspects has dependences on SA WG3 work.

*** END CHANGES ***