SA WG2 Meeting #S2-151E	S2-2203730

16 - 20 May, 2022, E-Meeting

Source: 	Ericsson

Title: 	KI #4,KI #5, New Sol: Broker for credential provisioning and redirection to hosting network

Document for: 	Agreement

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This paper proposes a solution to address the Key Issue #4 and #5.

# Discussion

Since it is usually assumed that UE may not have prior relationship with hosting networks which providing access to the localized services, it is important for UE to know what type of credential can be used to access hosting networks and how to obtain them.

For localized service user case, UE may have regular network credential (e.g. a credential from a PLMN operator). Hosting network can either allow access using this existing regular network credential, or require a separated credential for accessing hosting network.

If UE can use existing network credential to access the hosting network, certain relationship and interface is needed between hosting network and the network owning the credential. For example, in case of SNPN type of hosting network, the hosting network needs to establish relationship with UE's home network via Credential Holder function (TS 23.501 clause 5.30.2.9.3). This may not fit for the localized service use cases as described in TR 22.284, where the hosting network usually is assumed to be small which covers limited geographic area and even a temporary network which is commissioned during the time period of the localized services and decommissioned when the localized services are over.

If separated credential is used, a challenge will be how UE to obtain such credential, and whether existing network credential can assist and facilitate such process of retrieving the credential, and whether UE understands how to use the newly obtained credential.

This document proposes solution to address Key Issue #4 aspect:

-	How the UE is provisioned with credentials (if required) to access the selected localised services provided via the hosting network.

This document also addresses Key Issue #5 aspect:

-	How home network determines the need to steer or instruct the UE, and how the home network steers or instructs the UE to select a hosting network for obtaining home network services or localized services or select a network for a specific service which is available from both hosting and home network.

# Proposal

Add the following solution to TR 23.700-08.

*** BEGIN CHANGES ***

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | X |  |  |  |  |  |
| 2 |  | X |  |  |  |  |
| 3 |  | X |  |  |  |  |
| 4 |  | X |  |  |  |  |
| 5 |  | X |  |  |  |  |
| 6 |  | X |  |  |  |  |
| 7 |  |  | X | X | X | X |
| 8 |  |  |  |  |  | X |
| 9 |  |  |  |  |  | X |
| 10 |  |  |  | X |  | X |
| 11 |  |  |  |  | X |  |
| 12 |  |  |  | X |  |  |
| 13 |  |  |  | X | X |  |
| 14 |  |  |  | X |  |  |
| 15 |  |  |  | X | X |  |
| 16 |  | X |  |  |  |  |
| 17 |  |  |  |  |  | X |
| 18 |  |  |  |  | X |  |
| X |  |  |  | X | X |  |

*** NEXT CHANGES ***

## 6.x	Solution #x: Broker for credential provisioning and redirection to hosting network

### 6.X.1	Introduction

This solution addresses the Key Issue #4 aspect regarding how UE is provisioned with credential to access hosting network, and the Key Issue #5 aspect regarding how to steer UE to the hosting network.

### 6.X.2	Functional Description

It is assumed that UE has a regular network credential from a home network (a credential from a PLMN operator).

Because hosting network in principle is assumed to be small and even temporary, it is not in practice possible for a hosting network to establish relationship with all UEs' home networks, as shown in Figure 6.X.2-1. There will be a gap regarding how to make use of UE's existing network credential for the access of hosting network, given that such UE normally has no prior relationship with hosting network.

So, this solution proposes a functional role "broker", which can be taken by a 3GPP network (e.g. home PLMN). The broker will authenticate the UE based on UE's existing home network credential, and provide UE with a separated credential and the associated information of the credential based on the UE's input. The hosting network is only required to have direct relationship/interfaces with the broker. The broker is able to assist hosting network for authenticating the UE when UE accesses the hosting network, as a trust anchor for the hosting network.

The broker can also steer/redirect UE to the hosting network based on the input from UE (e.g. selected localized service and/or selected hosting network).

Figure 6.X.2-1 Broker deployment

### 6.X.3	Procedures

Figure 6.X.3-1 Credential provisioning and redirection via broker

0.	It is assumed that the end user/UE has been provisioned with information related localized service, e.g. which hosting networks can provide access to the desired localized service, etc.

1.	UE registers in the network which has the functional role of "broker", using home network credential.

Editor's note:	It is FFS how UE is made aware of the broker functional role of a network.

2.	End user/UE makes a selection of one or more specific hosting network(s) to access the desired localized service, or one or more specific localized service.

3.	UE initiates PDU Session Establishment procedure, and requests for Provisioning Server(PVS) related information from the network.

The UE's request of PVS related information includes one or more identifier(s) of the selected hosting network(s), and/or one or more identifier(s) of the selected localized service(s).

The UE's request can be either as part of the UL NAS Transport message or inside the PDU Session Establishment Request message which is the payload container of the UL NAS Transport message.

4.	PDU Session Establishment is accepted by SMF.

AMF or SMF has a locally configured mapping table between [PVS information] and [Localized service identifier], or a mapping table between [PVS information] and [Hosting network identifier]. AMF or SMF can return the corresponding [PVS information] to the UE, based on UE's request and the configured mapping table.

-	Whether it is AMF or SMF to return the [PVS information] to UE, depends on if the UE's request is part of UL NAS Transport message, or is inside the PDU Session Establishment Request message.

-	The [PVS information] in the mapping table consists of

-	FQDN or IP address of the PVS

-	Optionally, the identifier of the hosting network associated with the FQDN/IP address, in case the mapping table is based on [Localized service identifier]

-	The type of the credential. For example, in case of SNPN type of hosting network, the credential type can be a native credential which is used to access the SNPN directly, or can be a credential from Credentials Holder(CH) where the broker can act as CH, or can be "Default UE credential" which is used to perform onboarding procedure with the SNPN where the broker can act as Default Credential Server (DCS).

5.	UE connects to the PVS for remote provisioning of credential, and stores information associated with the newly obtained credential, e.g. the corresponding hosting network, the type of the credential etc.

6.	The UE initiates the de-registration procedure. If the selected localized service(s) are about to start (or have started), the UE needs to select the hosting network.

The redirection to hosting network can be triggered as below:

-	The UE includes in the de-registration request the identifier of the selected hosting network for accessing a localized service, or

-	The UE indicates in the de-registration request message with cause "redirection is requested", in case in step 3 only single hosting network identifier is requested by UE and the request is handled by AMF

After de-registration accept, the AMF informs the NG-RAN via NGAP about the selected hosting network identifier, e.g. as new IE in the UE CONTEXT RELEASE COMMAND for that UE. If the NG-RAN is aware of neighboring cells within the UE’s selected hosting network identifier, the NG-RAN assists the UE in finding the hosting network by sending an RRC release with redirect message (frequency and optionally cell information) or RRC release with multiple frequencies to the UE.

-	If the UE does not receive any redirect information in the RRC release message, the UE performs cell/network search to find/select the hosting network.

Figure 6.X.3-2 Broker acting as CH/DCS

As shown in Figure 6.X.3-2, the broker can act as either CH (see TS 23.501 [3] clause 5.30.2.9.3) or DCS (see TS 23.501 [3] clause 5.30.2.10.2) towards hosting network for authenticating the UE to access the hosting network, depends on the credential type provided to the UE in Figure 6.X.3-1. So the hosting network at the most need to have a relationship with a broker, without the need to have relationship with multiple home networks.

### 6.X.4	Impacts on services, entities, and interfaces

UE:

-	Request PVS information by indicating the selected hosting network(s) or selected localized service(s).

-	Use the obtained credential based on the associated type to access hosting network.

-	Indicate to the network during de-registration a redirection to hosting network is preferred.

AMF/SMF:

-	Configure the mapping of [PVS information] and [Hosting network identifier]/[Localized service identifier].

-	Respond to UE with proper [PVS information].

AMF/RAN:

-	Redirect the UE to hosting network based on the input from UE.

*** END CHANGES ***