3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2203756

Elbonia, May 16th – 20th, 2022	(revision of S2-220xxxx)

Source:	China Telecom

Title:	Discussion on IPv4 problem in direct communication path switching from PC5 to Uu

Document for:	Discussion

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract: This contribution points out the problem of IPv4 traffic in direct communication path switching from PC5 to Uu.

1. Introduction

As shown in TS 23.304 clause 5.3.4, the principal for the PC5 unicast link of V2X communication described in TS 23.287 clause 5.2.1.4 is reused for the PC5 unicast link of the 5G ProSe direct communication, and new data unit types are supported (including IPv4, Ethernet and Unstructured). For unicast mode of 5G ProSe direct communication, DHCP-based IPv4 address allocation with one of the two UEs may act as a DHCP server.

In the current TR 23.700-33, Key Issue #3 contains the following bullet point:

-	How to support any IP, Ethernet or Unstructured PDU type for direct communication path switching.

However, the routing problem exists in the scenario of IPv4 type path switching. IPv4 addresses are divided into public IPv4 addresses and private IPv4 addresses. The IPv4 addresses allocated by DHCP server (one of the two UEs) in the PC5 unicast link are private IPv4 addresses, which is not applicable for the network.

2. Discussion

When two UEs in the PC5 link want to communicate through the public network (Uu path), it must go through Network Address Translation (NAT) to convert their private IPv4 addresses into a legal public IPv4 addresses before they can be accessed externally.

In the current TR 23.700-33 clause 6.17.2 step 5, UE1 and UE2 perform the Uu path preparation procedure, where UE1 and UE2 each trigger PDU Session establishment/modification procedure, if needed, to make the Uu path ready for Service A transmission. During this procedure UE1 and UE2 can get the IP address of the PDU session that will be used for the Service A. But in this step, IP addresses negotiated between UE are private IPv4 addresses. How UE1 and UE2 obtain the public IPv4 addresses of themselves and the other UE is still a problem. The same problem exists in TR 23.700-33 clause 6.22.2.1 step 5 and step 6, where IP address associated to the other UE’s PDU session are exchanged.

This paper proposes a potential method for path switching from PC5 to Uu to solve IPv4 problem. Application servers are set to handle the IPv4 traffic, which are responsible for the traffic between UE 1 and UE 2 transfer.

0.	When the traffic type in PC5 link between UE1 and UE2 is IPv4, the following procedures are performed.

1.	The ProSe Application Server configures the IP address or FQDN of the application server for UE 1 and UE 2.

2.	UE1 and UE2 register with the same application server according to the IP address or FQDN

3.	UE1 and UE2 obtain their respective service IDs from the application server.

4.	UE1 and UE2 obtain each other's service ID through PC5 negotiation.

5.	UE1 or UE2 sends a session establishment request including the target service ID to the application server.

Editor's note:	This method is out of 3GPP scope.

3. Conclusion and proposal(s)

It is proposed to discuss the IPv4 problem of direct communication path switching from PC5 to Uu and a potential method. Suggestion is as follow:

-	Add editor’s note like “IPv4 type is not considered in this solution.” in related solutions which aim to support any IP type for direct communication path switching.

4. References

[1]	3GPP TS 23.304: " Proximity based Services (ProSe) in the 5G System (5GS) ".

[2]	3GPP TS 23.287: " Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services ".

[3] 	3GPP TR 23.700-33: " Study on system enhancement for Proximity based Services".