SA WG2 Meeting #151e	S2-2203818

May 16th – 20th, 2022

Source: 	China Unicom, Huawei

Title: 	KI #1, Sol #3: Update to remove ENs

Document for: 	Approval

Agenda Item: 	9.11

Work Item / Release:	FS_Edge_ph2/ Rel-18

Abstract of the contribution: Updated solution proposal for KI#1 in roaming scenario.

1 Discussion

In the last meeting, this solution left 6 ENs to be addressed:

Editor's note:	It is FFS whether and how to support charging for the local traffic of a PDU session that supports local traffic routing to access an EHE in the VPLMN.

Editor's note:	How the solution work in case of DNS security is FFS.

Editor's note:	It is FFS whether and how the V-SMF configures the DNS message handling rule considering H-EASDF as the DNS server for the DNS resolution for traffic routed to the HPLMN.

Editor's note:	It is FFS how the V-SMF knows the N6 traffic routing information.

Editor's note:	It is FFS how V-AF and V-SMF know each other's address and whether this assumes involvement of a V-PCF.

Editor's note:	this clause is FFS.

The updated solution addresses all of these remaining ENs.

2 Proposal

It is proposed to update TR 23.700-48 as follows.

Start of change

## 6.3	Solution 03 (KI#1): EAS (re)discovery procedure in roaming scenario

### 6.3.1	Description

KI#1 proposes the scenario: Accessing EHE in a VPLMN when roaming. This solution addresses one of these specific scenarios, i.e., UE accessing V-EHE for a PDU session with a PSA in HPLMN and assumes that HPLMN does not have the knowledge of EAS deployment information in VPLMN.

This solution proposes EAS (re)discovery procedure in V-EHE by transmitting a newly defined EC enabling indicator between V-SMF and H-SMF and using the V-EASDF. The UL-CL functionality is used to combine the features of HR and LBO roaming and steer local traffic to the local V-PSA. To minimize the impact of UE, UE is not aware of DNS server changing in HR roaming scenario.

NOTE 1:	According to the roaming agreement between HPLMN and VPLMN, DNS security issue (i.e., using EASDF IP replacement mechanism) can be resolved based on mutual trust between operators.

NOTE 2:	This EC enabling indicator means that VPLMN can apply the traffic offload related to EC service (e.g. UL-CL insertion for EC traffic) without any further HPLMN control.

### 6.3.2	Procedures

#### 6.3.2.1	EAS discovery procedure in roaming scenario

When a UE that has established an HR PDU session expects to use edge computing service, the UE may send a DNS query to the DNS server, the corresponding EAS discovery procedure is shown in figure 6.3.2.1-1:

Figure 6.3.2.1-1: EAS discovery procedure accessing V-EHE in roaming scenario

1.	UE requests PDU session establishment for HR roaming as described in clause 4.3.2.2.2 of TS 23.502 [9]. During the PDU Session Establishment procedure, the H-SMF may consider the UE subscription information to select a H-DNS server  for the PDU Session. H-SMF sends H-DNS server IP address included in PCO to UE via V-SMF.

H-SMF may indicate to the UE either that for the PDU Session the use of the EDC functionality is allowed or that for the PDU Session the use of the EDC functionality is required.

If V-SMF does not store the EC enabling indicator per PDU session level, V-SMF sends an EC enabling indicator request to H-SMF.

H-SMF responses a positive EC enabling indicator to V-SMF to indicate that the HPLMN authorizes VPLMN to manage the EC service according to the roaming agreement between these two operators and UE is unaware of potential DNS server changing. H-SMF also sends the IP address of H-DNS server to V-SMF.

According to the roaming agreement, a FQDN list for EC service in VPLMN is preconfigured on the V-SMF. And V-SMF selects V-EASDF based on the local configuration. V-SMF drives related DNS message handling rule based on EAS deployment information provided by V-PLMN AF and/or local configuration and sends this DNS message handling rule to V-EASDF via Neasdf_DNSContext_Create request.

NOTE 1:	V-EASDF selection is triggered in HR PDU session establishment procedure based on the roaming agreement and the positive EC enabling indicator.

V-SMF uses the FQDN list to derive a traffic routing rule. This traffic routing rule is provided to V-UPF (with ULCL functionality), e.g., V-UPF routes DNS Queries for an FQDN (range) query to V-EASDF, and routes other traffic to H-PSA.

V-SMF configures V-UPF with EASDF IP replacement information (i.e., H-DNS server IP address and port number, V-EASDF IP address and port number). In uplink direction, V-UPF replaces the destination address of the DNS query with corresponding FQDN from H-DNS server to V-EASDF; in downlink direction, V-UPF replaces the source address of the DNS response with corresponding FQDN from V-EASDF to H-DNS server so that the UE does not aware DNS server changing.

NOTE 2:	This configuration refers to the option D of session breakout connectivity model in TS 23.548 and assumes that V-UPF (with ULCL functionality) steering is based on L4 information (i.e. DNS port number) and that V-UPF (with ULCL functionality) has visibility of the DNS traffic (i.e. FQDN in the DNS Query message).

2.	UE sends DNS query to H-DNS server.

3a. If the DNS query does not match the FQDN list, V-UPF delivers the DNS query to H-PSA via N9 tunnel and H-PSA delivers the DNS query to H-DNS server.

3b. If the DNS query matches the FQDN list, V-UPF delivers the DNS query to V-EASDF using EASDF IP replacement. The following EAS discovery procedure is based on the step 3b.

4.

If the DNS Query message matches a DNS message detection template of DNS message handling rule for reporting, the V-EASDF sends the DNS message report to V-SMF by invoking Neasdf_DNSContext_Notify Request.

5.

V-SMF responds with Neasdf_DNSContext_Notify Response.

6.	V-SMF updates DNS message handling rule if needed. V-SMF sends this DNS message handling rule to V-EASDF via Neasdf_DNSContext_Update request.

7. V-EASDF responds with Neasdf_DNSContext_Update response.

8. V-EASDF handles the DNS query according to the DNS message handling rule and sends the DNS query to the DNS server in VPLMN, the V-DNS server returns the DNS response including EAS IP address and FQDN to V-EASDF.

9. V-EASDF sends the DNS response to the V-SMF by invoking Neasdf_DNSContext_Notify request including EAS information if the EAS IP address or the FQDN in the DNS response matches the DNS message detection template provided by the V-SMF as described in clause 6.2.3.2.2 of TS 23.548 [3], and V-EASDF buffers this DNS response.

10. V-SMF responds with Neasdf_DNSContext_Notify Response.

11. Based on EAS information received from the V-EASDF in Neasdf_DNSContext_Notify and other UPF selection criteria, V-SMF may determine the DNAI and determine the associated N6 traffic routing information for the DNAI based on local configuration. V-SMF may perform V-UL CL and V-PSA selection and insertion as described in TS 23.502 [9].

V-SMF configures the V-UL-CL with CN tunnel info provided by V-UPF and V-PSA and with AN tunnel info provided by serving AN. The traffic routing rules are provided to V-UL-CL based on EAS information, e.g., V-UL-CL routes the traffic to V-PSA if the IP packet has a destination address of V-EAS and routes other traffic to V-UPF.

V-SMF configures the Usage Report Rule on V-PSA for traffic charging. V-PSA collects and reports the charging information between UE and V-EAS based on the corresponding URR.

12. V-SMF invokes Neasdf_DNSContext_Update Request (DNS message handling rule). The DNS message handling rule with the Control Action "Send the buffered DNS response(s) message to UE" indicates the V-EASDF to send the DNS response buffered in step 9 to UE via V-UPF.

13. V-EASDF responds with Neasdf_DNSContext_Update response.

14. If it is indicated to send the buffered DNS response to UE in step 12, the V-EASDF sends the DNS response to the V-UPF. V-UPF replaces the source address from V-EASDF to H-DNS server in the DNS response based on the V-SMF instructions and sends this DNS response to the UE via V-UL CL.

15.	For traffic charging between UE and V-EAS, V-PSA collects and reports the charging information based on the corresponding URR.

#### 6.3.2.2	EAS rediscovery procedure in roaming scenario

The support for EAS rediscovery indication procedure enables the UE to refresh stale EAS information stored locally so that the UE can trigger EAS discovery procedure to discover new EAS information as described in clause 6.2.3.3 of TS 23.548 [3]. The corresponding EAS rediscovery procedure is shown in figure 6.3.2.2-1:

Figure 6.3.2.2-1: EAS rediscovery procedure accessing V-EHE in roaming scenario

0.	The procedure in clause 6.3.2.1 executes with following difference to apply to EAS rediscovery:

In step 1, the UE may indicate its support for refreshing stale EAS information stored locally corresponding to the impact field per the EAS rediscovery indication from network to the V-SMF during the HR PDU Session Establishment procedure.

NOTE 1:	If the UE indicates such support, V-SMF may store this indication in the PDU session context.

1.	Due to the UE mobility the V-SMF triggers V-PSA insertion, change or removal for the PDU Session. The insertion, change or removal of V-PSA triggers EAS rediscovery.

NOTE 2:	This solution does not cover EAS rediscovery scenario triggered by AF.

2.	V-SMF sends PDU Session Modification Command (EAS rediscovery indication, [impact field]) to UE as described in step 2 of clause 6.2.3.3 of TS 23.548 [3], with the following differences:

V-SMF sends the impact field with the EAS rediscovery indication if UE supports this indication.

If V-SMF chooses new V-EASDF for the PDU Session, it does not provide the V-EASDF IP address to UE. V-SMF may configure V-UPF with the updated traffic routing rule (e.g., V-UPF routes DNS Queries for an FQDN (range) query to the new V-EASDF) and with corresponding EASDF IP replacement information (i.e., H-DNS server IP address and port number, new V-EASDF IP address and port number) as described in Step 1 of clause 6.3.2.1.

UE refreshes DNS record according to EAS rediscovery indication received from V-SMF.

3.	UE may trigger EAS discovery procedure to get new EAS information. This DNS query is sent to corresponding DNS server using IP address replacement in VPLMN.

### 6.3.3	Impacts on existing entities and interfaces

H-SMF:

-	sends H-DNS server IP address and an EC enabling indicator to V-SMF in the HR PDU session establishment procedure.

V-SMF:

-	identifies the EC enabling indicator and manages the EC service for UE in VPLMN;

-	selects and inserts the V-UL-CL and V-PSA based on local policy and does not need to interact with H-SMF;

-	stores the indication which indicates that UE supports to refresh stale EAS information stored locally corresponding to the impact field per the EAS rediscovery indication from network in the PDU session context;

-  sends the EAS rediscovery indication including the impact field to UE.

End of change