3GPP TSG-SA2 Meeting # 151E (e-meeting)	S2-2203841

Elbonia, May 16 - 20, 2022

Source: 	Ericsson

Title:	E2E QoS and ToS/TC over N33 for Traffic Identification within 5G Media Streaming

Document for:	Discussion/Agreement

Agenda Item:	4.1

Work Item / Release:	5GS_Ph1, TEI 17

Abstract of the contribution: This paper discusses the need to support ToS/TC over N33 to have e2e QoS control between the UE and the 5G Media Streaming server.

# Discussion

## SA4 Use Case: making use of ToS/TC for Traffic Identification within 5G Media Streaming in the Collaboration Scenarios

There is proposal in SA4 TR 26.804 (included in SA4 LS S2-2203628/S4-220305) to make use of ToS/TC for identification (for selected UEs) of traffic from/to the 5G Media Streaming server.

The Annex contains more information of the collaboration scenarios from TR 26.804. The model is also valid for deployments in which the 5GMS functions reside in the trusted DN.

## 5GS QoS model and E2E QoS architecture including backbone

Per TS 23.501, 5GS QoS model applies to packet forwarding between UE and PSA UPF. 5G QoS Flow is the finest granularity for packet forwarding treatment in the 5G System. All traffic mapped to the same 5G QoS Flow receive the same forwarding treatment (e.g., scheduling policy, queue management policy, rate shaping policy, RLC configuration, etc.).

[Observation-1] 5GS QoS model focus on traffic differentiation between the UE and PSA UPF. QoS support between the PSA UPF and the AF (in trusted or external domain) is not within 3GPP scope.

Per TS 23.207 (End-to-end Quality of Service (QoS) concept and architecture) includes the following:

Annex A (informative): QoS Conceptual Models

A.1 Introduction

There are many different end-to-end scenarios that may occur from a UE connected to a UTMS network. The following examples depict how end-to-end QoS will be delivered for a number of scenarios that are considered to be significant.

NOTE:	Further consideration of scenarios 2 and 3 is not needed for Stage 3 work in the Release 5 timeframe. The normative aspects of scenarios 2 and 3 are considered to be already covered by scenario 1.

In all the scenarios presented below, the network architecture is as shown in Figure A.1 below.

Figure A.1: Network Architecture for QoS Conceptual Models

NOTES:

-	Although the backbone IP network is shown as a single domain, it may consist of a number of separate domains.

-	The structure of the Local UE is not specified. It includes cases from a simple host, to a gateway to a network such as a LAN. If the UE is acting as a gateway, it is responsible for providing the IP BS Management towards the extended network.

-	The remote side is shown as a simple host. Other more complex cases on the remote side such as a private LAN with over-provisioning, or possibly LAN priority marking, and DiffServ and/or RSVP capable routing elements is not depicted. It is envisaged however that interworking between the QoS mechanisms in a more complex remote user side could also be performed with some similarities to the mechanisms shown at the local side.

The reference point shown at the UE is at the interface to the UE. Within the UE, the QoS control could be derived from any of the mechanisms that occur across that reference point, or it could use a different mechanism internally.

Although the scenarios currently identified are mainly using DiffServ in the backbone IP network (RSVP is indicated as an alternative in scenario 4), it is not mandated that DiffServ must be used in the backbone IP network. Other mechanisms, for example, over-provisioning and aggregated RSVP may be used.

[Observation-2] Although TS 23.207 is intended for GPRS, the end-to-end concept and architecture (as illustrated in Figure A.1 above) also apply to EPS and 5GS in our view.

[Observation-3] DiffServ (i.e., DSCP, which is part of the redefined ToS/TC field) may be used for providing QoS differentiation over the backbone between the packet core and application server. For using the full strength of the 5G QoS framework, the 3GPP specification should encourage to apply QoS to the e2e path up to the AF. When crossing domain boundaries, the QoS provisioning on the respective path is up for implementation.

## How to address the SA4 Use Case

5G Media Streaming server (as for other application servers) is residing in the Data Network over N6 (which may be inside or outside of the trust domain of the Network Operator). To have end-to-end QoS control (as described in [Observation-2]), in addition to 5G QoS model within 5GS, QoS differentiation for traffic over the backbone between 5GC and the application server may be desired.

Per [Observation-3], ToS/TC may be used in the backbone for traffic differentiation.

To have an aligned QoS differentiation along the traffic path (between UE and PSA UPF, and between PSA UPF and application server), ToS/TC (together with Flow Description (i.e., IP tuples)) is desired to be considered in 5GS QoS control. When the 5G Media Streaming server interact with 5GC via NEF, it means that ToS/TC needs to be transferred over N33.

To make use of ToS/TC in traffic identification and QoS differentiation for traffic between the UE and the application server, the precondition is that there is no remarking of ToS/TC field along the traffic path, which requires service level agreement between the operator and the application service provider, e.g., leveraging leased lines or direct peering between the MNO network and the application service provider network. If the precondition cannot be fulfilled, then ToS/TC in traffic identification (and 5G QoS) cannot be used.

[Proposal-1] It is proposed to allow ToS/TC over N33 in the related procedures and clarify the precondition that there is no remarking of ToS/TC. If there is remarking, then ToS/TC cannot be used in traffic identification.

# 2 Proposal

[Proposal-1] It is proposed to allow ToS/TC over N33 in the related procedures and clarify the precondition that there is no remarking of ToS/TC. If there is remarking, then ToS/TC should not be used in traffic identification.

See TS 23.502 CR3431 (S2-2203843)

# Annex: Excerpt from TR 26.804 v17.0.0

#### 5.3.2.1	General Collaboration Scenarios

The 5GMSd Application Provider negotiates with the MNO an SLA to provide differentiated treatment, including network QoS and charging for its 5GMSd-Aware Application. The Application Provider provides the necessary information to the MNO to detect the traffic, to ensure its correct and exclusive identification. The MNO detects the traffic correctly and applies the agreed traffic treatment.

Due to privacy concerns, the content hosting is provided by the Application Provider in an external Data Network. However, the 5GMSd Application Provider leverages the network features either via a 5GMSd AF in the external Data Network (Figure 5.9.2-1) or via a 5GMSd AF in the trusted Data Network (Figure 5.9.2-2).

Figure 5.9.2-1: Collaboration 1 (Collaboration 3 of TS 26.501)

Figure 5.9.2-2: Collaboration 2 (Collaboration 4 of TS 26.501)

In order to use flow-based network features (such as different QoS classes or different charging policies), the 5G System needs to detect the relevant traffic.

#### 5.3.2.2	Differentiated Services/ToS-enabled Collaboration Scenarios

Differentiated Services (DS) [76, 77] is a scalable scheme for managing application traffic by classifying the traffic into a set of coarse-grained traffic classes. A Differentiated Service (DS) domain is a continuous set of DS-capable routers, which are operated with a common set of configurations. Each IP packet in a DS domain is marked and conditioned according to its traffic class. A 6-bit DS Code Point (DSCP) of the 8-bit differentiated services field (DS field) is used for marking. The DS field replaces the ToS field in the IPv4 packet headers and the Traffic Class field in the IPv6 header.

End host systems may mark IP packets with a specific DSCP value prior to transmission. DS-enabled routers treat the packet according to the DSCP value when performing routing operations on it. Border gateway routers typically mark packets with a DSCP value based on some traffic policy, overriding any value set by hosts.

The RFCs defining Differentiated Services recommend a set of Per-Hop Behaviors (PHB), namely:

-	Default Forwarding (DF) PHB, defined in section 4.1 of RFC 2474 [76], is used for traffic without special treatment.

-	Class Selector PHB, defined in section 4.2.2.2 of RFC 2474 [76] is used for maintaining backwards compatibility with the IP precedence field of ToS.

-	Expedited Forwarding (EF) PHB, defined by RFC 3246 [78], is dedicated to low loss or low latency traffic.

-	Assured Forwarding (AF) PHB, defined by RFC 2597 [79], offers different levels of forwarding assurances.

The DS domain operator can also implement additional custom PHBs.

In the context of ToS-based traffic identification and separation, it is reasonable to assume the Data Network north of the UPF (N6) is DS-enabled. The 5G System is embedded in a larger DS domain, using same ToS values across multiple devices in order to provide Quality of Service Support like a DSCP-enabled link. However, it is not required to deploy DS capable routers for using in order to use the ToS field in the IP packet filter set for traffic identification.

According to clause 4.1 of TS 26.501 [15], the 5GMS functions may be deployed within the trusted Data Network or an external Data network. As noted above, DS Code Points are often reset at network domain borders, but not always. There may be deployments e.g., with localized Edge Computing or with direct peering realizations, where the DSCP values can be used up to the 5GMSd AF and/or 5GMS AS in an external Data Network. In this case, the logical DS domain is extended to include those externally-deployed 5GMS functions.

Figure 5.3.2.2-1 illustrates a deployment with a DS domain between the 5G System and the 5GMS functions deployed in the external DN. (The model is also valid for deployments in which the 5GMS functions both reside in the trusted DN.)

Figure 5.3.2.2-1: 5GMS deployment within a DS domain

Figure 5.3.2.2-2 illustrates a deployment with a DS domain between the 5G System and an externally deployed 5GMSd AS. The 5GMSd AF is deployed in the trusted DN.

Figure 5.3.2.2-1: 5GMS deployment within a DS domain

### 5.3.5	Potential open issues

The exact behaviour and information that needs to be provided to and by the 5GMSd AF as well as the Media Session Handler need to be specified.

The following open issues have been identified:

1.	The Npcf_PolicyAuthorization API as defined in TS 23.502 [24] only supports usage of a flow description or an application identifier. The flow description is not further defined in TS 23.501 or TS 23.502. In Stage 3 specifications, a flow description represents only a 5-tuple. Other information elements of the Service Data Flow Filter are not supported.

2.	The Nnef_ChargeableParty and Nnef_AFsessionWithQOS APIs support usage of a flow description or an Application Identifier for referencing one or more PFDs. The flow description is not further defined in TS 23.501 or TS 23.502. Other information elements of the Service Data Flow Filter are not supported.

3.	The Npcf_PolicyAuthorization API Stage 3 as defined in TS 29.514 [42], only supports a flow description and a ToS value. However, it is not possible to define whether the ToS value should be used in uplink traffic detection or downlink traffic detection.

4.	The Nnef_AFsessionWithQOS and Nnef_ChargeableParty stage 3 APIs, as defined in TS 29.522 [43], only supports a Packet Flow Description (through the FlowInfo Type) or an Application Identifier for referencing one or more PFDs. Other information elements of the Service Data Flow Filter (like a ToS value) are not supported. Note, the FlowInfo type from TS 29.122 [44] is different from the FlowInformation type in TS 29.512 [45].