SA WG2 Meeting #151E (e-meeting)	S2-2203852

16 - 20 May, 2022, Electronic meeting	(revision of)

Source:	Ericsson

Title:	New WID: Providing DNN for N6 routing from SMF to I-SMF

Document for:	Approval

Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: Providing DNN for N6 routing from SMF to I-SMF

Acronym: N6DNNI

Unique identifier:	TEI18xx

Potential target Release:	Rel-18

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Dependency on non-3GPP (draft) specification:

# 3	Justification

Network deployments in EPS shows that the APN used for SGi routing in PGW may be different from the APN received from MME to route the SGi traffic e.g., based on the enterprise that a specific user belongs to. This functionality is typically achieved in a proprietary way in the operator’s network, e.g., for an enterprise user, a Radius server may be deployed to provide the APN for the SGi routing.

The functionality in EPS above is expected to continue when 5GS is deployed in the operator’s network, that is, the DNN used for N6 routing may be different from the DNN received from AMF.

If the I-SMF is involved in selecting the local PSA, the I-SMF needs to know the DNN for N6 routing to select the appropriate local PSA. Currently I-SMF knows the DNN received from the AMF but not the final DNN for N6 routing if the AMF provided DNN is modified for N6 routing DNN. To address this issue, the SMF needs to provide the “DNN for N6 routing” to the I-SMF.

# 4	Objective

The objective is to allow the SMF to provide DNN used for N6 routing to the I-SMF.

Note: This work will require 0.25~0.5 TU.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.502 | Add description that I-SMF gets DNN used for N6 routing in clauses 4.23.5.1 (PDU Session establishment procedure) and 4.23.9.1 (Addition of PDU Session Anchor and Branching Point or UL CL controlled by I-SMF) | SA#98? | TBD according to SA2 agreed time plan. |

# 6	Work item Rapporteur(s)

xxxx, Ericsson (@ericsson.com)

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

-

# 9	Supporting Individual Members

Supporting IM name

Ericsson