SA WG2 Meeting #SA2-151E	S2-2203857

16 - 20 May, 2022, Electronic meeting

Source:	Peraton Labs, CISA ECD, Verizon UK Ltd, AT&T, T-Mobile USA

Title:	New WID on MPS for 5GC LBO

Document for:	Approval

Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	MPS for 5GC LBO

Acronym:	TEI18_MPS2

Unique identifier:

Potential target Release:	Rel-18

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| MPS2 | SA2 | 870002 | Multimedia Priority Service (MPS) Phase 2 |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Dependency on non-3GPP (draft) specification:

# 3	Justification

Under the Rel-17 SA2 MPS2 work item, a solution was created for home routed roaming of MPS for Data Transport Service.  However, a solution for operators using local breakout was not addressed. When a roaming UE invokes the MPS for Data Transport Service via the AF in the VPLMN, the PCF in the VPLMN cannot authorize the user, as it has no knowledge of the user’s MPS subscription.

# 4	Objective

Develop a solution for MPS for Data Transport Service for local breakout where the PCF in the VPLMN obtains the MPS subscription of the roaming MPS user via signalling so it can authorize MPS for Data Transport Service requests from the AF in the VPLMN.

The objective is to specify a solution reusing existing functionality (e.g., interfaces, service operations) and introduce minimum incremental functionality.

In the case of 5GC LBO where the AF is in the VPLMN, the V-PCF cannot obtain the Service User’s MPS subscription today. The MPS subscription is in the Access and Mobility Subscription data of the UDM in the home network.  The Access and Mobility Subscription data is transferred via N8 to the (V-)AMF during registration as well as via updates obtained with Nudm_SDM_Subscribe.

The solution involves allowing the AMF in the VPLMN to forward the MPS priority indication to the SMF in the VPLMN, which forwards/reports this indication to the PCF in the VPLMN. This allows the PCF to authorize MPS request invocations received via N5 from AFs in the case of LBO.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency Editor’s Note: This column should highlight if WT#x is self-contained, or is depended on completion of other WTs |
| --- | --- | --- | --- | --- |
| WT#1 | 0 | .50 | No | WT#1 is self-contained |

Total TU estimates for the study phase: 0

Total TU estimates for the normative phase: .5

Total TU estimates: .5

# 5	Expected Output and Time scale

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | Support of LBO for MPS Data Transport Service | TSG#99 |  |
| TS 23.502 | Support of LBO for MPS Data Transport Service | TSG#99 |  |
| TS 23.503 | Support of LBO for MPS Data Transport Service | TSG#99 |  |

# 6	Work item Rapporteur(s)

Streijl, Robert, Perspecta Labs, rstreijl@perspectalabs.com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

None.

# 9	Supporting Individual Members

Supporting IM name

Peraton Labs

CISA ECD

Verizon UK Ltd

AT&T

T-Mobile USA