SA WG2 Meeting #151E e-meeting	S2- 2203867

Elbonia, May 16th – 20th, 2022

Source:	Ericsson

Title:	ML model unique global identifier in Sol#1

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_ph3 / Rel-18

Abstract: This contribution proposes a solution to an Editor’s Note in Solution #1 for determining the uniqueness of an ML model ID.

# Discussion

Solution #1 in TR 23.700-81 v0.2.1 relies on an ML model that is identified by a unique identifier. However, it is not clear who and how this ML model unique global identifier is created. An Editor’s Note reflects this status:

Editor's note:	Who and how to create the unique identifier for ML model is FFS.

The exact details, format, and encoding of the ML model unique identifier are left to the stage 3 design. This paper proposes that the NWDAF containing MTLF that creates an ML model creates the unique identifier. Furthermore, the paper proposes that the ML model unique identifier is assumed to be created from the NF Instance ID of the NWDAF containing MTLF that created the ML model and a serial number. This guarantees that the ML model identifier is unique within the PLMN.

According to TS 29.510, the NF Instance ID has the property of being globally unique inside the PLMN of the NRF where the NF is being registered.

The variable {nfInstanceID} represents an identifier, provided by the NF Service Consumer that shall be globally unique inside the PLMN of the NRF where the NF is being registered.

# Proposed Solution

The following text is proposed to be added to TR 23.700-81 v0.2.1.

********** Start of Changes ***************

### 6.1.1	Description

TS 23.288 [5] assumes that the NWDAF containing AnLF has a single ML model for inferring analytics, at a given time, after applying any potential analytics filter. However, in many scenarios, it is possible that the NWDAF containing AnLF may have multiple ML models at its disposal, and therefore, multiple ML models can be used to generate the analytic reports provided by a given Analytics ID.

Having the NWDAF containing AnLF multiple ML models to choose from, enables use cases where an ML model can be selected depending on, e.g. the UE location, time of day, network load, or any other filter of information.

This solution is based on the existence of several models to draw a prediction for a single Analytics ID. In all these scenarios, the following functionalities needs to be taken into consideration:

-	Each ML model needs to have a unique identifier within the PLMN which is assigned by the ML model creator, i.e. an NWDAF containing MTLF. The ML Model unique identifier may be created, e.g. upon the NF Instance ID of the NWDAF containing an MTLF and a serial number. The exact format and encoding of the ML model unique identifier is left to stage 3.

-	An NWDAF containing AnLF should be able to get all available ML models for a specific Analytics ID, from an NWDAF containing MTLF.

-	For the case of ML model provisioning, an NWDAF containing AnLF should be able to request ML model information (i.e. the parameters described in clause 6.2A.2 of TS 23.288 [5]) for an Analytics ID, from an NWDAF containing MTLF, and be able to receive responses including more than one ML model for the same Analytics ID.

-	Upon reception of multiple ML model information for the same Analytics ID, the NWDAF containing AnLF should be able to choose and download the model(s) of its choice.

Figure 6.1.1-1 presents an example of an NWDAF containing AnLF that collects multiple data, feeds each data to a multiple and different ML models, and uses three ML models for generating predictions for the same Analytics ID. Once a given ML model has generated a prediction, an internal logic in the aggregation/voting unit selects the best prediction and delivers it to the consumer. Therefore, the consumer is provided with the best of the three predictions or an optimized aggregated prediction.

Figure 6.1.1-1: Aggregation/voting chooses the best prediction

NOTE:	The aggregation/voting function in Figure 6.1.1-1 is for illustration purposes and is left to implementation logic.

********** End of Changes ***************