SA WG2 Meeting #151E e-meeting	S2-2203872

Elbonia, May 16th – 20th, 2022

Source:	Ericsson

Title:	KI#7: New Solution: Supporting aggregated UE performance monitoring and exposure for a group of UEs

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys / Rel-18

Abstract: This contribution propose a new solution for Key Issue #7 to support monitoring and exposing performance of a group of UEs by reusing DN performance analytics.

# Discussion

KI#7 in TR 23.700-80 has the following problem to address to support Federated Learning operations:

On performance monitoring/exposure:

-	How to monitor and expose a UE or a group of UEs performance (e.g. aggregated QoS parameters) as described in TS 22.261 [2] related to FL operations.

-	Whether and what existing or new monitoring events (e.g. QoS, location, load, congestion) are required to capture specific System Performance and Predictions for traffic related to AI/ML operations for FL operation.

Regarding QoS parameters, the most important ones are packet latency, bit rate, packet drop rate. Those QoS parameters are already specified as part of the output parameters of DN Performance Analytics in clause 6.14 of TS 23.288. Table 6.14.3-1 of TS 23.288 shows the output of the analytics:

Table 6.14.3-1: DN service performance statistics

| Information | Description |
| --- | --- |
| Application ID | Identifies the application for which analytics information is provided. |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. See note 1. |
| DNN | Identifies the data network name (e.g. "internet") for which analytics information is provided. See NOTE 1. |
| DN performance (0-x) | List of DN performances for the application. |
| > Application Server Instance Address | Identifies the Application Server Instance (IP address/FQDN of the Application Server). |
| > Serving anchor UPF | The involved anchor UPF. See NOTE 2. |
| > DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |
| > Performance | Performance indicators. |
| >> Average Traffic rate | Average traffic rate observed for UEs communicating with the application. See NOTE 3. |
| >> Maximum Traffic rate | Maximum traffic rate observed for UEs communicating with the application. See NOTE 3. |
| >> Average Packet Delay | Average packet delay observed for UEs communicating with the application. See NOTE 3. |
| >> Maximum Packet Delay | Maximum packet delay for observed for UEs communicating with the application. See NOTE 3. |
| >> Average Packet Loss Rate | Average packet loss observed for UEs communicating with the application. See NOTE 3. |
| > Spatial Validity Condition | Area where the DN performance analytics applies. |
| > Temporal Validity Condition | Validity period for the DN performance analytics. |
| NOTE 1: The item "DNN" and "S-NSSAI" shall not be included if the consumer NF is an untrusted AF. NOTE 2: The item "Serving anchor UPF" shall not be included if the consumer is an AF. NOTE 3: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". |  |

To support exposing aggregated QoS parameters, we can add aggregated traffic rate, variance of the traffic rate, variance of packet delay and variance of packet loss rate.

Though the solution is proposed for KI#7 to support FL operations, it is suitable for any group-based AIML operations.

# Proposed Solution

The following text is proposed to be added to TR 23.700-80 v0.2.0.

********** Start of Changes ***************

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | X |  |  |  |  | X |  |
| 2 |  | X |  |  |  |  |  |
| 3 |  | X |  |  |  |  |  |
| 4 |  | X |  |  |  |  |  |
| 5 |  | X |  |  |  |  |  |
| 6 |  |  | X |  |  |  | X |
| 7 |  |  | X |  |  | X |  |
| 8 |  | X | X |  |  |  |  |
| 9 |  |  |  | X |  |  |  |
| 10 |  |  |  |  | X |  |  |
| 11 | X |  |  |  | X |  |  |
| 12 |  |  |  |  | X |  |  |
| 13 |  |  |  |  | X |  |  |
| 14 |  |  |  |  |  | X | X |
| 15 |  |  |  | X |  | X |  |
| 16 | X |  |  |  | X | X | X |
| 17 |  |  |  |  |  |  | X |
| 18 |  |  |  |  |  |  | X |
| 19 |  |  |  |  |  |  | X |
| 20 |  |  |  |  |  |  | X |
| 21 |  |  |  |  |  |  | X |
| 22 |  |  |  |  |  |  | X |
| 23 |  |  |  |  |  |  | X |
| 24 |  |  |  |  |  |  | X |
| 25 |  |  |  |  |  |  | X |
| 26 |  |  |  |  |  | X | X |
| 27 |  |  |  |  |  |  | X |
| x |  |  |  |  |  |  | x |

********** Next Changes (all new texts)***************

## 6.X	Solution #X: Solution for supporting aggregated UE performance monitoring and exposure for a group of UEs

## 6.X.1	Description

This solution aims to address the key issue #7 to support aggregated UE performance monitoring and exposure for a group of UEs. The proposed solution is suitable for any group based AIML operations, including Federated Learning operations.

Regarding UE performance, we focus on the QoS parameters, such as packet latency, bit rate, packet drop rate. Those QoS parameters are already specified as part of the output parameters of DN Performance Analytics in clause 6.14 of TS 23.288. However, current output parameters of DN Performance analytics need to be extended to include aggregated parameters. Therefore, the solution proposes to add the following parameters in the output parameters of DN Performance Analytics:

- aggregated traffic rate;

- variance of the traffic rate;

- variance of packet delay;

- variance of packet loss rate.

## 6.X.2	Procedures

The procedure is the same as clause 6.14.4 in TS 23.288[6], where the analytics consumer is the AF (via NEF) and the Target of Analytics Reporting is a group of UEs which are participating the AIML operations.

## 6.X.3	Impacts on services, entities and interfaces

-	The output of the DN Performance Analytics is extended with more parameters.

********** End of Changes***************