3GPP TSG-WG SA2 Meeting #151-e	S2-2203884

E-meeting, 2022‑05‑16 -- 2022‑05‑20	(revision of S2-220xxxx)

Source:	Ericsson

Title:	KI#4, New sol: Solution based on solution 16

Document for:	Approval

Agenda Item:	9.11

Work Item / Release:	FS_EDGE_Ph2 / Rel-18

Abstract: This solution proposes a solution based on solution 16

This solution uses EDI as means for synchronizing data between SMFs, and also some other minor functional differences compared to solution 16

## ************** 1st change **************

## 6.0	Solution-Key issue matrix

The solutions in clause 6 can apply to one or more key issues described in clause 5 of this report. Table 6.0-1 describes the relationship between solutions and key issues.

Editor's note:	The table below will be updated with actual content when generating the TR with approved contributions. Page number is automatically updated to ease reference (ctrl-left click to reach the solution).

Table 6.0-1: Solution-Key issue matrix

| Solution |  | Key issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title | (page) | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 | KI#7 |
| 01: EAS discovery in Home Routed roaming scenario | 18 | X |  |  |  |  |  |  |
| 02: Session Breakout in Visited PLMN | 21 | X |  |  |  |  |  |  |
| 03: EAS (re)discovery procedure in roaming scenario | 23 | X |  |  |  |  |  |  |
| 04: Support EAS discovery in VPLMN via HR PDU Session | 28 | X |  |  |  |  |  |  |
| 05: Accessing V-EHE via HR PDU session | 31 | X |  |  |  |  |  |  |
| 06: URSP solution to support roamers access to EHE in a VPLMN | 33 | X |  |  |  |  |  |  |
| 07: Using URSP Rules to Establish an LBO PDU Session | 35 | X |  |  |  |  |  |  |
| 08: V-ECS Discovery during Steering of Roaming | 36 | X |  |  |  |  |  |  |
| 09: PDU Session configuration from EASDF | 38 | X |  |  |  |  |  |  |
| 10: LBO PDU Session establishment using PLMN criteria in RSD | 40 | X |  |  |  |  |  |  |
| 11: Exposure of Network Congestion | 41 |  | X |  |  |  |  |  |
| 12: Efficient exposure of RAN information | 43 |  | X |  |  |  |  |  |
| 13: Fast and efficient network exposure improvements | 47 |  | X |  |  |  |  |  |
| 14: Group Management | 50 |  |  |  | X |  |  |  |
| 15: Selection of common DNAI | 54 |  |  |  | X |  |  |  |
| 16: Selecting the same EAS/DNAI for collection of UEs | 62 |  |  |  | X |  |  |  |
| 17: Application layer EAS selection for collections of UEs | 64 |  |  |  | X |  |  |  |
| 18: Discovery of the same EAS for collections of UEs | 66 |  |  |  | X |  |  |  |
| 19: Influencing UPF and EAS (re)location for collections of UEs | 70 |  |  |  | X |  |  |  |
| 20: Global EASDF | 71 |  |  |  |  | X |  |  |
| 21: EAS Deployment information differentiated by PLMN ID | 73 |  |  |  |  | X |  |  |
| 22: EAS discovery for federated OPs | 74 |  |  |  |  | X |  |  |
| 23: Improvements for EHE operated by separate party | 80 |  |  |  |  | X |  |  |
| Xx: Solution xx (KI#4): Selecting the same EAS/DNAI for collection of UEs (II) | nn |  |  |  | X |  |  |  |

## ************** 2nd change (all new) **************

## 6.xx	Solution xx: Selecting the same EAS/DNAI for collection of UEs

This solution corresponds to KI#4.

### 6.xx.1	Description

This solution is based on solution 16.

### 6.xx.2	Procedure

The following is the procedure for selecting the same EAS for collection of UEs accessing the same application. The procedure is based on the procedure in solution 16. The procedures defined in figure 4.3.6.2-1 in TS 23.502 [9] and figure 6.2.3.2.2-1 in TS 23.548 [3] are reused.

This solution uses EDI to make sure a collection defined by an AF uses the same DNAI and/or EAS. By this EDI is updated with EAS address field per DNAI. The per DNAI data in EDI is also updated with a Dynamic External Group Id that can per DNAI or per application ID. The AF can use the Dynamic External Group ID and external application id to influence routing for a collection of UEs.  The dynamic externa group id is an identifier that is generated by the AF. i.e. it shall not be confused with the externa group id configured in UDM. The AF associates a list of UEs with the dynamic external group id when requesting that the list of UEs should be using the same EAS and/or DNAI.

AF

AF

NEF

NEF

UDR

UDR

PCF

PCF

SMF

SMF

DNS server

DNS server

EASDF

EASDF

UE

UE

1. Provisioning of EDI

1. Provisioning of EDI

2. Steps 1-5 in AF request to influence routing

2. Steps 1-5 in AF request to influence routing

3. Steps 1-2 in 6.2.3.3 in TS 23.548

3. Steps 1-2 in 6.2.3.3 in TS 23.548

4. Steps 1-9 in 6.2.3.2.2 in TS 23.548

4. Steps 1-9 in 6.2.3.2.2 in TS 23.548

5. Determine if UE should be part of dynamic group

5. Determine if UE should be part of dynamic group

6. update EAS/DNAI data

6. update EAS/DNAI data

7. Steps 10-19 in 6.2.3.2.2 in TS 23.548

7. Steps 10-19 in 6.2.3.2.2 in TS 23.548

Figure 6.xx.2-1: Discovery procedure for selecting the same EAS/DNAI for collection of UEs

1.	AF provides EDI and SMF may get the EDI as per TS 23.248 [3]. AF includes a Dynamic External Group ID per DNAI or per Application ID, if on Externa application ID, then it is relevant for all DNAIs. There may be a list of Dynamic External Group IDs on both levels

2.	The AF request in step 1 of figure 4.3.6.2-1 in TS 23.502 [9] is used to request selecting the same EAS and/or same DNAI for a collection of UEs accessing the application as identified by external application ID in the AF Request. The AF provides a DNAI correlation indication and/or EAS correlation indication. The correlation indications are used as mean to request a common DNAI and/or a common EAS for the UEs.

NEF maps external application ID to application ID.

AF will also indicate if re-discovery is needed to NEF, this can only be used if a list if UEs is provided. This is to ensure that a common EAS will be used by the concerned UEs

In step 5 of figure 4.3.6.2-1 in TS 23.502 [9], PCF determines the UEs influenced by the AF Request, and based on AF request, PCF creates PCC rule with application ID and the Dynamic External Group ID and DNAI correlation indication and/or EAS correlation indication.

If not already subscribed to receive updates events of the EDI for the concerned DNN and S-NSSAI, SMF does that as per 23.548 [3] to retrieve the EDI.

3	If re-discovery requested SMF does a PDU session modification to trigger EAS re-discovery as per clause 6.2.3.3 in TS 23.548 [3].

4.	The same as steps 1-9 in figure 6.2.3.2.2-1 in TS 23.548 [3].

5.	If the notified FQDN from EASDF to SMF is related to the Application ID that is associated with a Dynamic External Group ID, and that DNAI and/or EAS correlation information was received in step 2, the SMF determines that the UE should be part of a dynamic group with same DNAI and/or same EAS (depending on what the AF originally requested). Using the data received in step 2, SMF determines if any of the DNAIs in the EDI with Dynamic Group ID is suitable for the session. If so, SMF uses this DNAI and if EAS correlation indication was received from PCF, and if there is an EAS address associated with the Dynamic External Group ID, DNAI SMF uses this in step 7.

6.	The following cases can occur:

-	If no relevant DNAI could be used in step 5, SMF selects a DNAI and updates the EDI with EAS/DNAI data for the Dynamic External group ID, Application ID via NEF (SMFs subscribing to update events to the EDI change will be notified).

-	If EAS correlation indication was set and SMF did not have any EAS address available in step 5, SMF provides the EAS address that was received from EASDF. If multiple EAS addresses ware received SMF selects one. SMF uses this in step 7.

7.	Based on steps 10-19 in figure 6.2.3.2.2-1 in TS 23.548 [3]. i.e. updates of the DNS handling rules with the EAS address, and breaking out the session at selected DNAI.

### 6.xx.3	Impacts on services, entities and interfaces

AF:

-	to be updated with Dynamic External Group ID ID, EAS correlation indicator, and DNAI correlation indicator.

NEF:

-	Nnef_TrafficInfluence service (or new) as per AF new information.

-	New data in EDI.

SMF:

-	using PCC rule to associate UEs to a dynamic group of UEs, based on the original AF provided information given by PCF. Using data in EDI.

UDR:

-	New data in EDI

EASDF:

-	may need to be updated for create and send DNS response to UE.

PCF:

-	services related to AF influence on routing to be updated with Dynamic External Group ID, EAS correlation ID, DNAI correlation ID.

**************  End of changes **************