3GPP TSG-WG SA2 Meeting #151-e	S2-2203891

# Electronic Meeting, 2022‑05‑16 -- 2022‑05‑20

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3476 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NRF discovery and selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI17 |  |  |  |  |  | Date: |  |  | 2022-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Discovery and selection of NRF no specified, alignment with stage 3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding discovery and selection and use of NRF boot strapping service |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missalignement between Stage 2 and Stage 3, can lead to wrong functionality from stage 2 perspective. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.17.x (new), 5.2.7.1, 5.2.7.x (new), 5.2.7.x.1 (new), 5.2.7.x.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS 23.501 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st changes ********************

### 4.17.x	NRF bootsrapping procedure

NF service Consumer

NF service Consumer

NRF

NRF

1. Nnrf_Bootstrapping_Get request

1. Nnrf_Bootstrapping_Get request

2. Nnrf_Bootstrapping_Get response

2. Nnrf_Bootstrapping_Get response

Figure 4.17.x-1: Bootstrapping procedure

1.	NF service consumer (e.g. v-NRF) sends a Nnrf_Bootsrapping_Get request to the configured address of the Bootstrapping Service instance.

2.	NRF responds with all the Service Instances of the NRF and their endpoint addresses. This also contains if the NRF is part of an NF Set.

******************** 2nd changes ********************

#### 5.2.7.1	General

The following table shows the NRF Services and Service Operations:

Table 5.2.7.1-1: NF services provided by the NRF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnrf_NFManagement | NFRegister | Request/Response | AMF, SMF, UDM, AUSF, NEF, PCF, SMSF, NSSF, UPF, BSF, CHF, NWDAF, P-CSCF, HSS, UDR, SCP, 5G DDNMF, DCCF, NSACF, MB-SMF, AF (NOTE 2), TSCTSF, EASDF, MFAF, ADRF |
|  | NFUpdate | Request/Response | AMF, SMF, UDM, AUSF, NEF, PCF, SMSF, NSSF, UPF, BSF, CHF, NWDAF, P-CSCF, HSS, UDR, SCP, 5G DDNMF, DCCF, NSACF, MB-SMF, TSCTSF, EASDF, MFAF, ADRF |
|  | NFDeregister | Request/Response | AMF, SMF, UDM, AUSF, NEF, PCF, SMSF, NSSF, UPF, BSF, CHF, NWDAF, P-CSCF, HSS, UDR, SCP, 5G DDNMF, DCCF, NSACF, MB-SMF, TSCTSF, EASDF, MFAF, ADRF |
|  | NFStatusSubscribe | Subscribe/Notify | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, CHF, NRF, NWDAF, I-CSCF, S-CSCF, IMS-AS, SCP, UDM, TSCTSF, DCCF |
|  | NFStatusNotify |  | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, CHF, NWDAF, I-CSCF, S-CSCF, IMS-AS, SCP, UDM, TSCTSF, DCCF |
|  | NFStatusUnSubscribe |  | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, CHF, NRF, NWDAF, I-CSCF, S-CSCF, IMS-AS, SCP, UDM, TSCTSF, DCCF |
| Nnrf_NFDiscovery | Request | Request/Response | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, CHF, NRF, NWDAF, I-CSCF, S-CSCF, IMS-AS, SCP, UDM, AF (NOTE 2), DCCF, MBSF, 5G DDNMF, TSCTSF |
| Nnrf_AccessToken | Get | Request/Response | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, UDM, NWDAF, I-CSCF, S-CSCF, IMS-AS, HSS, DCCF |
| Nnrf_Bootstrapping | Get | Request/Response | NRF, any NF or SCP |

NOTE 1:	HSS_IMS services are defined in TS 23.228 [55].

NOTE 2:	The AF is a trusted AF by an operator.

******************** 3rd changes ********************

#### 5.2.7.x	Nnrf_Bootstrapping service

##### 5.2.7.x.1	General

Service Description: The Nnrf_Bootstrapping service lets NF Service Consumers of the NRF know about the services endpoints it supports, the NRF Instance ID and NRF Set ID if the NRF is part of an NRF set, by using a version-independent URI endpoint that does not need to be discovered by using a Discovery service

##### 5.2.7.x.2	Nnrf_Bootstrapping_Get service operation

Service operation name: Nnrf_Bootstrapping_Get

Description: Get the NRF service instances, their addresses and associated data.

Inputs, Required: none

Inputs, Optional: none

Outputs, Required: NRF service instances and their endpoint addresses

Outputs, Optional: NF Set ID, NFR instance ID, Oath2 requirment for authorization, NFR status

******************** End changes ********************